Change Request

| Title: | CR to Close OpenCMAPI CONRR Comments D099, D100, D113 | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_0-20120220-D |  |
| Submission Date: | 21 Feb 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Magnus Westin, Birdstep Technology AB, magnus.westin@birdstep.com |  |
| Replaces: | N/A |  |

Reason for Change

This CR proposes resolutions for the following OpenCMAPI CONRR comments:

| D099 | 2011.12.20 | T/Q | 5.7.1 | Source: DTAG Form: doc#CONR-2011-0141 Comment: strIP, how to identify dynamical IP and static IP? Proposed Change: To clarify | Status: OPEN (conf call 08/02/12) - discussion postponed |
| --- | --- | --- | --- | --- | --- |
| D100 | 2011.12.20 | T/Q | 5.7.1 | Source: DTAG Form: doc#CONR-2011-0141 Comment: dwCellularProfileType, what’s the definition for this type? Proposed Change: To clarify | Status: OPEN see D102, D103, D104, D105, D106 & D107 (conf call 08/02/12) - discussion postponed – to do a CR addressing all remaining comments on 5.7.1 |
| D113 | 2011.12.20 | T | 5.7.3 | Source: DTAG Form: doc#CONR-2011-0141 Comment: There is no API for enumerating existing profiles names and modification of profiles An update of an existing profile would typically start with a GET operation, then modification of the profile and finally an UPDATE operation. However, updating a profile is not possible while connected I t is not possible to change the profile while connected, a return value for this should be added. Proposed Change: To add the API functions corresponding | Status: OPEN (conf call 08/02/12) – Thierry to work on CR / clarification |

For D099 and D100 See Change 1

For D113 see change 2

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

CD OpenCMAPI group should agree to this CR and close comments D099, D100, D113

Detailed Change Proposal

Modify TS section 5.7.1 CellularProfileType as follows:

| Field Name | Type | Description |
| --- | --- | --- |
| CellularProfileName | String* | The name of the Cellular Profile |
| DeviceName | String* | The device to use |
| UserName | String* | The user name assotiated to the connection |
| Password | String* | The password assotiated with the connection |
| PhoneNum | String* | The phone number assotiated with the connection |
| APN | String* | The APN used for this connection |
| IP | String* | The IP address |
| PrimaryDNS | String* | The primary DNS |
| SecondaryDNS | String* | The secondary DNS |
| AuthType | dword | The Authentication Protocol type: - 0x00: CHAP only - 0x01: PAP only - 0x02: Automatic |
| IPAddrType | dword | The type of the IP address: - 0x00: IPv4 - 0x01: IPv6 - 0x02: IPv4v6 |
| UseDhcpForIP | Boolean | Use DHCP for IP address. If this is true, then the IP field is unused. |
| UseDhcpForDNS | Boolean | Use DHCP for DNS address. If this is true, then the PrimaryDNS and SecondaryDNS fields are unused. |
| TimeoutSeconds | dword | The time out in seconds |

The following changes were made:

- CellularProfileType field was removed

- UseDhcpForIP field was added

- UseDhcpForDNS field was added

Add new TS section after 5.7.3

We need to add a new API call to list all configured cellular profiles. Suggests to implement as follows:

### CMAPI_NetConnectSrv_GetCellularProfileList()

The CMAPI_NetConnectSrv_GetCellularProfileList () function is used to get a list of all Cellular Profile names.

Prototype

dword CMAPI_NetConnectSrv_GetCellularProfileList (dword deviceID, ProfileNameType* pCellularProfileNameList, dword* dwCellularProfileNameListCount )

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned |
| pCellularProfileNameList | Output | ProfileNameType { string* pCellularProfileName; dword dwCellularProfileNameLength; } |
| dwCellularProfileNameListCount | Input/Output | Number of entires in the list. If pCellularProfileNameList is NULL then this will return the count needed to store the whole list. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |
| 0x00000002 | The structure is not sufficient to hold the data, the dwCellularProfileNameListCount will contain the minimize number of elements.. |
| 0x00000003 | The deviceID references a non-existent device or a device which is not open |