# Change Request

| Title: | CONR A059 resolution | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_1-20140408-D |  |
| Submission Date: | 5 May 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Dieter Gludovacz, Deutsche Telekom AG, dieter.gludovacz@t-mobile.at |  |
| Replaces: | n/a |  |

- Reason for Change

This CR proposes to add the “Allow/DisallowSimultaneousConnection” functions to the P2P Direct Management APIs in order to resolve CONR comment A059.

| A059 | 2014.02.14 | T | CMAPI-P2P-003 | Source: QUALCOMM Form: doc#CONR-2014-0005 Comment: Has this requirement been met? Proposed Change: If we have not met this requirement, remove it or enhance the TS to meet it | Status: CLOSED by CR OMA-CD-OpenCMAPI-2014-0030 |
| --- | --- | --- | --- | --- | --- |

| Label | Description | supported by function |
| --- | --- | --- |
| CMAPI-P2P-003 | The OpenCMAPI Enabler SHALL provide the capability to allow or not allow the device to have a P2P connection simultaneously to a normal data connection using the same radio technology (e.g. using Wi-Fi Direct to exchange pictures with other devices while being connected to internet through Wi-Fi). |  |

- Impact on Backward Compatibility

None

- Impact on Other Specifications

None

- Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

- Recommendation

The CD OpenCMAPI SWG should review and agree this CR.

- Detailed Change Proposal

- Add the following functions to P2P Direct Mgt section

### CMAPI_P2P_AllowSimultaneousConnection()

The CMAPI_P2P_AllowSimultaneousConnection() function is used to allow the device to have a P2P connection simultaneously to a normal data connection using the same radio technology.

Prototype

dword CMAPI_P2P_AllowSimultaneousConnection (dword deviceID, P2PinfoType P2PTechno)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| P2PTechno | Input | The P2P direct connection technology to enable Please see P2PInfoType definition (bitwise combination of one or several types) |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. |
| 0X00000101 | The deviceID references a non-existing device or a device which is not open |
| 0X00000104 | The device does not contain hardware which supports this operation. |
| 0X00000130 | The device is not in a power state which allows this operation. |
| 0X00008001 | The P2PTechnology is not supported |
| 0X00008002 | The P2P Technology is invalid |
| 0XF0000001 | The security request supplied when the API was opened does not grant privilege to access this functionality. You may close and reopen the API with updated credentials to perform this operation. |

### CMAPI_P2P_DisallowSimultaneousConnection()

The CMAPI_P2P_DisallowSimultaneousConnection() function is used to disallow the device to have a P2P connection simultaneously to a normal data connection using the same radio technology.

Prototype

dword CMAPI_P2P_DisallowSimultaneousConnection (dword deviceID, P2PinfoType P2PTechno)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| P2PTechno | Input | The P2P direct connection technology to disable Please see P2PInfoType definition (bitwise combination of one or several types) |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. |
| 0X00000101 | The deviceID references a non-existing device or a device which is not open |
| 0X00000104 | The device does not contain hardware which supports this operation. |
| 0X00000130 | The device is not in a power state which allows this operation. |
| 0X00008001 | The P2PTechnology is not supported |
| 0X00008002 | The P2P Technology is invalid |
| 0XF0000001 | The security request supplied when the API was opened does not grant privilege to access this functionality. You may close and reopen the API with updated credentials to perform this operation. |