# Change Request

| Title: | Resolution of P2P related comments to TS | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_1-20140422-D |  |
| Submission Date: | 22 May 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Dieter Gludovacz, Deutsche Telekom AG, dieter.gludovacz@t-mobile.at |  |
| Replaces: | n/a |  |

- Reason for Change

This CR proposes some change to the AD in order to resolve following CONR comment.

| C023 | 2014.02.13 | T | 7.20 | Source: DTAG Form: doc#CONR-2014-0004 Comment: Check and align P2P Direct Management APIs with 3GPP ProSe if necessary Proposed Change: | Status: CLOSED by CR OMA-OpenCMAPI-2014-0046 |
| --- | --- | --- | --- | --- | --- |
| C025 | 2014.02.14 | T | 7.20.8 | Source: QUALCOMM Form: doc#CONR-2014-0005 Comment: The capability referenced in the “Editor’s Note” is already addressed in the Announce API as Service Records are an input parameter Proposed Change: Remove “Editor’s Note” | Status: CLOSED by CR OMA-OpenCMAPI-2014-0046 18/02/2014: agreed – done online on the TS (20140218-D) But added informational note in the Service Record definition To Add references |

- Impact on Backward Compatibility

None

- Impact on Other Specifications

None

- Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

- Recommendation

The CD OpenCMAPI SWG should review and agree this CR.

- Detailed Change Proposal

- Add to section 2.1 Normative References

| [3GPP TS 23.334] | “TS 23.334 Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) User Equipment (UE) to Proximity-services (ProSe) Function Protocol aspecs; Stage 3; (Release 12)”, 3rd Generation Partnership Project (3GPP), URL: http://www.3gpp.org/ftp/Specs/archive/23_series/23.334/ |
| --- | --- |

- in Section 6.2.42	Service Record

### ServiceRecord

Definition ServiceRecord

This prototype defines a structure which describes a service record. Informational Note: In term of this specification, this is an unformatted field. For interpretation of this field please refer to the related specifications [3GPP 23.003], [3GPP 23.303] and [3GPP 24.334].

| Field Name | Type | Description |
| --- | --- | --- |
| serviceID | UTF8* | The identifier of a service. In case of ProSe Discovery it corresponds to the ProSe Application ID Name as specified [3GPP 23.003]. |
| appID | UTF8* | The identifier of an application. |
| rangeClass | UTF8* | Optional; The range class (rough indication of distance for use in ProSe discovery) requested. |
| metadata | UTF8* | Optional; Metadata of the service, used for DiscoveryMatch() and ResolveDiscovery() only. |

### ServiceRecords

Definition ServiceRecords

This prototype defines a structure which describes the number of service identifiers in the list.

| Field Name | Type | Description |
| --- | --- | --- |
| numberOfRecords | dword | The number of service identifiers in the array |
| serviceIDs | ServiceRecord* | The list of the service records |

### DeviceRecords

Definition DeviceRecords

This prototype defines a structure which describes the number of device identifiers in the list.

| Field Name | Type | Description |
| --- | --- | --- |
| numberOfRecords | dword | The number of device identifiers in the array |
| remoteDeviceIDs | dword* | The list of the remote device IDs |

- in section 7.20 P2P Direct Management APIs

### CMAPI_P2P_DiscoveryResolve ()

The CMAPI_P2P_DiscoveryResolve() function is used to resolve a ServiceRecord (LTE-D expression container) for metadata and/or connection info.

Prototype

dword CMAPI_P2P_DiscoveryResolve (dword deviceID, ServiceRecord* pService, P2PInfoType * pP2PInfo)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| pService | Input | The ServiceRecord to be resolved. |
| pP2PInfo | Output | Pointer to get the P2P direct connection technology type in bitmap. In the case of a device with multiple P2P direct connection technologies supported, there MAY be multiple settings returned. The bitmap definition follows the definition of P2PInfoType: - 0x00000000: None - 0x00000001: Wi-Fi Direct (could there be several versions) - 0x00000002: reserved for future use - 0x00000004: reserved for future use - 0x00000008: reserved for future use - 0x00000010: reserved for future use - 0x00010000: LTE Direct - 0x00020000: reserved for future use |

### CMAPI_P2P_DiscoveryMonitor()

The CMAPI_P2P_DiscoveryMonitor() function is used to request discovery of Remote Device(s) and the services offered.

Prototype

dword CMAPI_P2P_DiscoveryMonitor (dword deviceID, DeviceRecords* pRemoteDevices, ServiceRecords* pServices)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| pRemoteDevices | Input | Optional; List of Remote Devices to be discovered |
| pServices | Input | List of Service Identifiers to be discovered |

### CMAPI_P2P_DiscoveryAnnounce()

The CMAPI_P2P_DiscoveryAnnounce() function is used by applications in order to announce its P2P Direct services supported or by the Local Device to announce its presence to Remote Device(s).

Prototype

dword CMAPI_P2P_DiscoveryAnnounce (dword deviceID, DeviceRecords* pRemoteDevices, ServiceRecords* pServices)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| pRemoteDevices | Input | Optional; List of Remote Devices addressed by this announcement |
| pServices | Input | List of Service Identifiers announced by Local Device |

- in Section 8.3 Callback APIs

### CMAPI_Callback_P2P_DiscoveryMatch()

The CMAPI_Callback_P2P_DiscoveryMatch() function is used to alert the Local Device of a DeviceID/ServiceID/ServiceRecord (LTE-D expression container) discovery match as indicated in CMAPI_P2P_Monitor().

Prototype

dword CMAPI_Callback_P2P_DiscoveryMatch (CallbackStatus status, dword deviceID, dword remoteDeviceID, dqword serviceID, dword result)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| status | Input | The status of the callback. |
| deviceID | Input | The ID of the device concerned. |
| remoteDeviceID | Input | Optional; Remote Devices that match the criteria |
| serviceID | Input | Service Identifiers that match the criteria |
| result | Input | - 0x00000001: Device & Service matching criteria - 0x00000002: Device only matching criteria - 0x00000003: Service only matching criteria |