Change Request

| Title: | Remove Obex bearer. | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-TS-DM_StdObj-V1_3-20101207-C |  |
| Submission Date: | 12 July. 11 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Rama Gopala, rama.gopala@lge.com SeungKyu Park, seungk.park@lge.com |  |
| Replaces: | n/a |  |

Reason for Change

Removed OBEX from the bearer list under CBT node, as the OBEX is already considered as the transport binding in DM context.

Added Bluetooth and IrDA in the bearer list on which the OBEX transport works.

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG should review and agree this CR.

Detailed Change Proposal

Removing OBEX from bearer and adding Bluetooth and IRDA as bearers.

### 5.3.3    The DevDetail management object

Management object identifier: urn:oma:mo:oma-dm-devdetail:1.1

The following figure shows an overview of the DevDetail management object.

Figure 5: The DevDetail management object

The nodes making up DevDetail have the following meanings:

| ./DevDetail |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node specifies the unique object id of the DevDetail management object. Management Object Identifier for the DevDetail MO MUST be: “urn:oma:mo:oma-dm-devdetail:1.1”. |  |  |  |  |

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | An optional, interior node, designating the only branch of the DevDetail sub tree into which extensions can be added, permanently or dynamically. |  |  |  |  |

| Bearer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | An optional, interior node, designating a branch of the DevDetail sub tree into which items related to the bearer (CDMA, etc.) are stored. Use of this sub tree can be mandated by other standards. |  |  |  |  |

| Roaming |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence |  | Format |  | Min. Access Types |  |  |  |  |
|  | Required | One |  | int |  | Get |  |  |  |  |
|  | This node indicated the current roaming status for the current DM session. The following values are valid: Value; Description. 0; Current DM session is not over a roaming connection.. 1; Current DM session is over a roaming connection. 2; It is unknown if the current DM Session is over a roaming connection.. |
| CBT |  |  |  |  |  |  |  |  |  |  |
| Status | Occurrence | Format |  | Min. Access Types |  |  |
| Required | One | Int |  | Get |  |  |
| This node provides bearer type information over which the DM session is currently being carried. The content of this node is an integer with the value in range from 0 to 255, and currently the following values are allocated for different bearer types. For the bearer types not covered in this version of specification the value ‘0’ (Other Bearer Type) MUST be used. |  |  |  |  |  |  |  |  |
| Bearer |  |  | Technology |  |  | Value |
| Other Bearer Type |  |  |  |  |  | 0 |
| 3GPP Circuit Switched Bearer |  |  |  |  |  | 1 |
| 3GPP Packet Switched Bearer |  |  | GERAN |  |  | 2 |
|  |  |  | UTRAN / HSPA |  |  | 3 |
|  |  |  | LTE |  |  | 4 |
|  |  |  | LTE-Advanced |  |  | 5 |
|  |  |  | I-WLAN 3GPP IP Access |  |  | 6 |
|  |  |  | Other |  |  | 7 |
| 3GPP2 CDMA Packet Data Bearer |  |  | 1X / HRPD |  |  | 8 |
|  |  |  | UMB |  |  | 9 |
|  |  |  | Other |  |  | 10 |
| WLAN |  |  |  |  |  | 11 |
| DSL |  |  |  |  |  | 12 |
| WiMAX |  |  |  |  |  | 13 |
| BlueTooth |  |  |  |  |  | 14 |
| Ethernet |  |  |  |  |  | 15 |
| IrDA |  |  |  |  |  | 16 |

| URI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node holds URI related information. |  |  |  |  |

| URI/MaxDepth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Specifies the maximum depth of the management tree supported by the device. The maximum depth of the tree is defined as the maximum number of URI segments that the device supports. The value is a 16 bit, unsigned integer encoded as a numerical string. The value ‘0’ means that the device supports a tree of ‘unlimited’ depth. |  |  |  |  |

| URI/MaxTotLen |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Specifies the maximum total length of any URI used to address a node or node property. The maximum total length of a URI is defined as the largest total number of characters making up the URI which the device can support. Note that depending on the character set this might not be the same as the number of bytes. The value is a 16 bit, unsigned integer encoded as a numerical string. The value ‘0’ means that the device supports URI of ‘unlimited’ length. |  |  |  |  |

| URI/MaxSegLen |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Specifies the maximum total length of any URI segment in a URI used to address a node or node property. The maximum total length of a URI segment is defined as the largest number of characters which the device can support in a single URI segment. Note that depending on the used character set this might not be the same as the number of bytes. The value is a 16 bit, unsigned integer encoded as a numerical string. The value ‘0’ means that the device supports URI segments of ‘unlimited’ length. |  |  |  |  |

| DevType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Device type, for example. PDA, pager, or phone. |  |  |  |  |

| OEM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Original Equipment Manufacturer of the device. |  |  |  |  |

| FwV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Firmware version of the device. |  |  |  |  |

| SwV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Software version of the device. |  |  |  |  |

| HwV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Hardware version of the device. |  |  |  |  |

| LrgObj |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the OMA DM Large Object Handling specification, as defined in [DMPRO]. |  |  |  |  |

| UserInteraction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node is a placeholder for the user interaction leaf nodes. |  |  |  |  |

| UserInteraction/UserInput |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the User Input Alert, as defined in [DMPRO]. |  |  |  |  |

| UserInteraction/Display |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Display Alert, as defined in [DMPRO]. |  |  |  |  |

| UserInteraction/Confirm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Confirmation Alert, as defined in [DMPRO]. |  |  |  |  |

| UserInteraction/Choice |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Choice Alert, as defined in [DMPRO]. |  |  |  |  |

| UserInteraction/ProgressNotify |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Progress Notification Alert, as defined in [DMPRO]. |  |  |  |  |

It is RECOMMENDED that the combination of HwV, SwV, FwV, Man, Mod, and OEM provide a unique signature identifying the specific version of software, thus providing a means for other implementations to make special provisions based on that identification.

The complete DDF description of this management object can be found in [DevDetailDDF].

Doc# OMA-DM-DM13-2010-xxxx-CR_RD_Correction_Informative
Change Request

© 2010 Open Mobile Alliance Ltd.  All Rights Reserved.	Page 2 (of 2)
Used with the permission of the Open Mobile Alliance Ltd. under the terms as stated in this document.	[OMA-Template-ChangeRequest-20100101-I]

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

USE OF THIS DOCUMENT BY NON-OMA MEMBERS IS SUBJECT TO ALL OF THE TERMS AND CONDITIONS OF THE USE AGREEMENT (located at http://www.openmobilealliance.org/UseAgreement.html) AND IF YOU HAVE NOT AGREED TO THE TERMS OF THE USE AGREEMENT, YOU DO NOT HAVE THE RIGHT TO USE, COPY OR DISTRIBUTE THIS DOCUMENT.

THIS DOCUMENT IS PROVIDED ON AN "AS IS" "AS AVAILABLE" AND "WITH ALL FAULTS" BASIS.

© 2010 Open Mobile Alliance Ltd.  All Rights Reserved.	Page 17 (of 2)
Used with the permission of the Open Mobile Alliance Ltd. under the terms as stated in this document.	[OMA-Template-ChangeRequest-20100101-I]