Change Request

| Title: | Baseline for service QoS metrics function | Public OMA Confidential |
| --- | --- | --- |
| To: | DM-WG |  |
| Doc to Change: | OMA-TS-DiagMon_Functions-V1_2-20110711-D |  |
| Submission Date: | 4 Nov 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Friedhelm Rodermund, Vodafone, friedhelm.rodermund@vodafone.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR suggests a baseline for a service related QoS metrics function to fulfil the related RD requirement.

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to agree this CR.

Detailed Change Proposal

Change of title of chapter 6

# Metrics

New chapter  6.3  “QoS Metrics”

## QoS Metrics

### Introduction

If the device exposes QoS metrics functionality to the DiagMon Client, then the DiagMon Client MUST support this function. The DiagMon Server MUST support this function.

This function allows the Diagnostics and Monitoring system to retrieve ETSI QoS parameters as specified in ETSI TS 102 250-2. , measured by the device, that provide an indication of the QoS environment at the time of function invocation.

Examples of ETSI QoS parameters are:

Streaming Service Access Time

Streaming Audio Quality

Streaming Video Quality

Streaming Teardown Time

Telephony Speech Quality on Call Basis

Telephony Cut-off Call Ratio

IMS Multimedia Telephony

This function MUST be invoked explicitly.  The status of this DiagMon function can be reported asynchronously, using the Generic Alert mechanism [DMPRO] or it can be stored in the DM Tree for later retrieval.

.

### Non-applicable nodes from DiagMon MO definition

The following nodes SHOULD NOT be used for this function:

- Operations (the function is always available)

- DiagMonConfig (the function does not have any configurable parameters)

- ServerID (The Diagnostics and Monitoring data is either retrieved by using Get command or delivered to the alternate data server).

- Status (the function is always available)

### Function Description

# Figure 1 - QoS Metrics Function

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is a placeholder for the QoS Management Object. Identifier for the QoS MO MUST be: “urn:oma:mo:oma-diag:qos:1.0”. |  |  |  |  |

| <x>/Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonData/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for zero or more instances of QoS metrics data. |  |  |  |  |

| <x>/DiagMonData/<x>/QoSParameterName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the name of the QoS parameter to be measured. When using the QoS parameters from ETSI TS 102 250-2, the parameter name SHALL be as specified in this specification (e.g. Streaming Video Quality). |  |  |  |  |

| <x>/DiagMonData/<x>/QoSParameterValue |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the value of the parameter which is specified in QoSParameterName. |  |  |  |  |

| <x>/DiagMonData/<x>/CellID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | int | Get |  |
|  | This optional leaf node contains the value of the cell identifier. |  |  |  |  |

| <x>/DiagMonData/<x>/GeoLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bin | Get |  |
|  | This node contains the location where the measurements were taken, with the data encoded as per [3GPP-TS_23.032], section 7.3.1. |  |  |  |  |

| <x>/Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Start |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Get |  |
|  | See [DiagMonTS] for description of this node. This operation is to activate the QoS measurement. |  |  |  |  |

| <x>/Operations/Stop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Get |  |
|  | See [DiagMonTS] for description of this node. This operation is to deactivate the QoS measurement. |  |  |  |  |

| <x>/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |