Change Request

| Title: | Baseline for service QoS metrics function | Public OMA Confidential |
| --- | --- | --- |
| To: | DM-WG |  |
| Doc to Change: | OMA-TS-DiagMon_Functions-V1_2-20110711-D |  |
| Submission Date: | 21 Feb 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Friedhelm Rodermund, Vodafone, friedhelm.rodermund@vodafone.com Salvatore Scarpina, Telecom Italia S.p.A., salvatore.scarpina@telecomitalia.it |  |
| Replaces: | OMA-DM-Diag-2011-0033R02-CR_baseline_for_service_QoS_metrics_function.docx |  |

Reason for Change

This CR suggests a baseline for a service related QoS metrics function to fulfil the related RD requirement.

R03: Main changes to the version presented in Beijing:

- addition of DiagMonConfig with the threshold nodes

- addition of TimeStamp nodes

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to agree this CR.

Detailed Change Proposal

Change of title of chapter 6

# Metrics

New chapter  6.3  “QoS Metrics”

## QoS Metrics

### Introduction

If the device exposes QoS metrics functionality to the DiagMon Client, then the DiagMon Client MUST support this function. The DiagMon Server MUST support this function.

This function allows the DiagMon system to retrieve ETSI QoS parameters as specified in ETSI TS 102 250-2. , measured by the device, that provide an indication of the QoS environment at the time of function invocation.

Examples of ETSI QoS parameters are:

Streaming Service Access Time

Streaming Audio Quality

Streaming Video Quality

Streaming Teardown Time

Telephony Speech Quality on Call Basis

Telephony Cut-off Call Ratio

IMS Multimedia Telephony

This function MUST be invoked explicitly.  The status of this DiagMon Function can be reported asynchronously, using the Generic Alert mechanism [DMPRO] or it can be stored in the DM Tree for later retrieval.

.

### Non-applicable nodes from DiagMon MO definition

The following nodes SHOULD NOT be used for this function:

- Operations (the function is always available)

- ServerID (The Diagnostics and Monitoring data is either retrieved by using Get command or delivered to the alternate data server).

- Status (the function is always available)

### Function Description

# Figure 1 - QoS Metrics Function

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is a placeholder for the QoS Function. Identifier for the QoS MO MUST be: “urn:oma:mo:oma-diag:qos:1.0”. |  |  |  |  |

| <x>/Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |
| <x>/DiagMonConfig |  |  |  |  |  |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for instances of QoS configuration parameters. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This node specifies the QoS parameter name to be measured. When using the QoS parameters from ETSI TS 102 250-2, the parameter name SHALL be as specified in this specification (e.g. Streaming Video Quality). |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSLowerThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as lower threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSUpperThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as upper threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonData/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for zero or more instances of QoS metrics data. |  |  |  |  |

| <x>/DiagMonData/<x>/QoSName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the name of the QoS parameter to be measured. When using the QoS parameters from ETSI TS 102 250-2, the parameter name SHALL be as specified in this specification (e.g. Streaming Video Quality). |  |  |  |  |

| <x>/DiagMonData/<x>/QoSValue |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the value of the parameter which is specified in QoSParameterName. The value format MUST be interpreted according to parameter definition. |  |  |  |  |

| <x>/DiagMonData/<x>/CellID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | int | Get |  |
|  | This leaf node contains the value of the cell identifier – the CellGlobalIDEutra as defined in [3GPP-TS_36.331] |  |  |  |  |

| <x>/DiagMonData/<x>/GeoLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | bin | Get |  |
|  | This node contains the location where the measurements were taken, with the data encoded as per [3GPP-TS_23.032], section 7.3.1. |  |  |  |  |

| <x>/DiagMonData/<x>/Timestamp |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | The time and date the parameters sample measurement was taken, expressed as a UTC based [ISO8601] basic format. |  |  |  |  |

| <x>/Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Start |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Get |  |
|  | See [DiagMonTS] for description of this node. This operation is to activate the QoS measurement. |  |  |  |  |

| <x>/Operations/Stop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Get |  |
|  | See [DiagMonTS] for description of this node. This operation is to deactivate the QoS measurement. |  |  |  |  |

| <x>/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |