Change Request

| Title: | baseline for service sensor function | Public OMA Confidential |
| --- | --- | --- |
| To: | DM-WG |  |
| Doc to Change: | OMA-TS-DiagMon_Functions-V1_2-20110711-D |  |
| Submission Date: | 16 Feb 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Friedhelm Rodermund, Vodafone, friedhelm.rodermund@vodafone.com Olaf Seifert, Deutsche Telekom, olaf.seifert@telekom.de Salvatore Scarpina, Telecom Italia S.p.A., salvatore.scarpina@telecomitalia.it |  |
| Replaces: |  |  |

Reason for Change

This CR suggests a baseline for a sensor function to fulfil the related RD requirement.

Please note that the ReportCondition nodes are added to the DiagMon framework by 
CR OMA-DM-Diag-2012-0008-CR_Report_Channel

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to agree this CR.

Detailed Change Proposal

# Device

## Sensor

### Sensor Function

#### Introduction

If a device contains a sensor, then the DiagMon Client MUST support this function. The DiagMon Server MUST support this function.

This function will allow the Server to determine a variety of sensor information, such as the sensor data and unit, but also information about the sensor type and its location.

.

#### Non-applicable nodes from DiagMon MO definition

The following nodes SHOULD NOT be used for this function:

- ServerID (The Diagnostics and Monitoring data is either retrieved by using Get command or delivered to the alternate data server).

#### Function Description

# Figure 1 - Sensor Info Function

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is a placeholder for the Sensor Management Object. Identifier for the Sensor MO MUST be: “urn:oma:mo:oma-diag:sensorinfo:1.0”. |  |  |  |  |

| <x>/Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for zero or more instances of sensor ID. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/SensorID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the sensor ID. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/LowerThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | float | Get, Replace |  |
|  | This node specifies the lower reporting threshold for the sensor specified in SensorID. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/UpperThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | float | Get, Replace |  |
|  | This node specifies the upper reporting threshold for the sensor specified in SensorID. |  |  |  |  |

| <x>/DiagMonConfig/ReportCondition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ReportCondition/ReportChannel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | int | Get, Replace |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ReportCondition/ReportRecipient |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get, Replace |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonData/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for zero or more instances of sensor ID. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the sensor ID. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorStatus |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node is used to indicate the status of the sensor. The value of SensorStatus MUST be one of the following values: SensorStatus; Meaning; Description. 0; Off; The sensor is switched off.. 1; Active; The sensor is switched on and operating normally.. 2; Not Installed; The sensor is not installed.. 3; Malfunction - unspecified; The sensor has an unspecified error.. 4; Malfunction – out of range; The measured value is out of the sensor’s measurement range.. 5; Malfunction – calibration needed; The sensor needs calibration.. 6; Malfunction – SW error; A software error has occurred.. 7; Malfunction – transmission error; The sensor has detection transmission problems.. 8; Unknown; The sensor information is not available.. |

| <x>/DiagMonData/<x>/SensorData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the current sensor data in a unit as described in SensorUnit. In the case where the sensor data is not known, the value of the SensorData node SHALL be 0. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorUnit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the sensor data unit. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorSoftwareVersion |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | This node contains the SW version of the sensor |  |  |  |  |

| <x>/DiagMonData/<x>/SensorType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node contains the sensor type. Type examples are temperature, humidity, pressure, speed. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorMan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | This node contains the name of the sensor manufacturer. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorVer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | This node contains the version of the sensor. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorDate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | date | Get |  |
|  | This node contains the date of the sensor manufacture. If the date of manufacture is not available, this value MUST be null. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorGeoLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bin | Get |  |
|  | This node contains the geo-location of the sensor. with the data encoded as per [3GPP-TS_23.032], section 7.3.1. |  |  |  |  |

| <x>/DiagMonData/<x>/SensorSystemLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | This node contains the location of the sensor within a system e.g. an industry plant. |  |  |  |  |

| <x>/Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Start |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Get |  |
|  | See [DiagMonTS] for description of this node. This operation is to activate the sensor and start the sensor specific activation process (calibration process). |  |  |  |  |

| <x>/Operations/Stop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Get |  |
|  | See [DiagMonTS] for description of this node. This operation is to deactivate the sensor. |  |  |  |  |

| <x>/Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |