Change Request

| Title: | QoS threshold for RF metrics | Public OMA Confidential |
| --- | --- | --- |
| To: | DM-WG |  |
| Doc to Change: | OMA-TS-DiagMon_Functions-V1_2-20110711-D |  |
| Submission Date: | 21 Feb 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Friedhelm Rodermund, Vodafone, friedhelm.rodermund@vodafone.com Salvatore Scarpina, Telecom Italia S.p.A., salvatore.scarpina@telecomitalia.it |  |
| Replaces: |  |  |

Reason for Change

This CR suggests service related QoS thresholds as an additional trigger for the RF metrics function.

E.g. if the video quality gets poor (= QoS parameter “video quality” reaches lower threshold), then, the RF metrics function can be used to help identifying the reason for the poor video quality.

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to agree this CR.

Detailed Change Proposal

additions to RF metrics function

# Radio Protocol (3GPP, 3GPP2, WiFi, Bluetooth, WiMAX, etc.)

## RF Metrics

### 3GPP GSM Metrics

#### Introduction

If the device exposes GSM RF functionality to the DiagMon Client, then the DiagMon Client MUST support this function. The DiagMon Server MUST support this function.

This function allows the Diagnostics and Monitoring system to retrieve 3GPP GSM RF parameters, measured by the device, that provide an indication of the RF environment at the time of function invocation.  The measured parameters include Network Measurement Results as specified in [3GPP-TS_44018] as well as transmit and receive power and signal quality indicators as specified in [3GPP-TS_25.215] and [3GPP-TS_25.225].  This function is only applicable to 3GPP GSM devices or 3GPP UMTS devices operating on GSM.

This function MUST be invoked explicitly.  The status of this DiagMon function can be reported asynchronously, using the Generic Alert mechanism [DMPRO] or it can be stored in the DM Tree for later retrieval.

#### Non-applicable nodes from DiagMon MO definition

None

#### Function Description

# Figure 7 - 3GPP GSM RF Metrics Function

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is a placeholder for the 3GPP GSM RF Metrics data. Management Object Identifier for the 3GPP GSM RF Metrics MO MUST be: “urn:oma:mo:oma-diag:RFParms_3GPP_GSM:1.0”. |  |  |  |  |

| <x>/ServerID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/SaveLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the location information is requested to be saved. If the device is not able to determine the location, it is not required to save it. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This node acts as a placeholder for circular geographic locations. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get |  |
|  | This node acts as a placeholder for one or more circular geographic locations. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Lat |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get, Replace |  |
|  | This leaf node contains the latitude of the circular area as defined in [3GPP-TS_23.032], section 6.1. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Long |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get, Replace |  |
|  | This leaf node contains the longitude of the circular area as defined in [3GPP-TS_23.032], section 6.1. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the circular area as defined in [3GPP-TS_23.032], section 6.3. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/LAList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of Location Areas. Each LA is defined using LA Identity (LAI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. LAI is specified in 3GPP TS 23.003 section 4.1. The list SHALL be LA values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/RAList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of Routing Areas. Each RA is defined using RA Identity (RAI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. LAI is specified in 3GPP TS 23.003 section 4.2. The list SHALL be RA values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/CellList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of cells. Each cell is defined using Cell Identity (CI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. CI is specified in 3GPP TS 23.003 section 4.3. The list SHALL be CI values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/SampIntrvl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | int | Get |  |
|  | This leaf node contains an integer value indicating the sampling interval in seconds at which Diagnostics and Monitoring data are collected on the device. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for instances of QoS configuration parameters. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This node specifies the QoS parameter name to be measured. When using the QoS parameters from ETSI TS 102 250-2, the parameter name SHALL be as specified in this specification (e.g. Streaming Video Quality). |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSLowerThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as lower threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSUpperThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as upper threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This node identifies the collected Diagnostics and Monitoring data, which SHOULD be retrieved by the Diagnostics and Monitoring System. The collected DiagMon data results as identified by leaf nodes that are attached to this node are created dynamically and have appropriate format properties as per [DMTND]. Devices MUST support adding of leaf nodes under /<x>/DiagMonData. Devices SHOULD support these leaf nodes formatted in xml. |  |  |  |  |

| <x>/DiagMonData/<X> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This required node is a placeholder for the various collected RF data sets. The node name is constructed as the time the RF parameter measurements were taken, encoded per [ISO8601] |  |  |  |  |

| <x>/DiagMonData/<X>/NMR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | xml | Get |  |
|  | This node contains “Network Measurement Results”, as defined by [3GPP-TS_44.018] “Measurement Results”, encapsulated in XML format. |  |  |  |  |

| <x>/DiagMonData/<X>/TxPwr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains a measure of the total transmit power, as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225], “UE Transmitted Power”. |  |  |  |  |

| <x>/DiagMonData/<X>/GSMRSSI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the value of the GSM Received Signal Strength Indication, as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225], “GSM Carrier RSSI”. |  |  |  |  |

| <x>/DiagMonData/<X>/Location |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bin | Get |  |
|  | This optional leaf node contains the location where the measurements were taken, with the data encoded as per [3GPP-TS_23.032], section 7.3.1. Note that if the location is not desired by the server, or if the location data is unavailable, the node will not be present. |  |  |  |  |

| <x>/DiagMonData/<X>/CellID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node contains the value of the cell identifier – the CellGlobalIDEutra asdefined in [3GPP-TS_36.331]. |  |  |  |  |

| <x>/DiagMonData/<X>/ProbDesc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node contains the text describing any problem that may have occurred during the measurement. It is up to the device to determine if a problem description should be attached or not. |  |  |  |  |

| <x>/DiagMonData/<X>/Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node contains the configuration of the UE at the time of measurement – this node may be used to store data from an error condition. |  |  |  |  |

| <x>/Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Start |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Exec |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Stop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Exec |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

### 3GPP UMTS Metrics

#### Introduction

If the device exposes UMTS RF functionality to the DiagMon Client, then the DiagMon Client MUST support this function. The DiagMon Server MUST support this function.

This function allows the Diagnostics and Monitoring system to retrieve 3GPP UMTS RF parameters, measured by the device, that provide an indication of the RF environment at the time of function invocation.  The measured parameters include Network Measurement Results as specified in [3GPP-TS_44018] as well as transmit and receive power and signal quality indicators as specified in [3GPP-TS_25.215] and [3GPP-TS_25.225].  This function is only applicable to 3GPP UMTS devices.

This function MUST be invoked explicitly.  The status of this DiagMon function can be reported asynchronously, using the Generic Alert mechanism [DMPRO] or it can be stored in the DM Tree for later retrieval.

#### Non-applicable nodes from DiagMon MO definition

None

#### Function Description

# Figure 8 - 3GPP UMTS RF Metrics Function

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is a placeholder for the 3GPP UMTS RF Metrics data. Management Object Identifier for the 3GPP UMTS RF Metrics MO MUST be: “urn:oma:mo:oma-diag:RFParms_3GPP_UMTS:1.0”. |  |  |  |  |

| <x>/ServerID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/SaveLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the location information is requested to be saved. If the device is not able to determine the location, it is not required to save it. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This node acts as a placeholder for circular geographic locations. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get |  |
|  | This node acts as a placeholder for one or more circular geographic locations. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Lat |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get, Replace |  |
|  | This leaf node contains the latitude of the circular area as defined in [3GPP-TS_23.032], section 6.1. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Long |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get, Replace |  |
|  | This leaf node contains the longitude of the circular area as defined in [3GPP-TS_23.032], section 6.1. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the circular area as defined in [3GPP-TS_23.032], section 6.3. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/URAList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of UTRAN Registration Areas. Each URA is defined using URA Identity, broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. URA Identity is specified in [3GPP-TS_25.331] section 10.3.2.6. The list SHALL be URA values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/LAList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of Location Areas. Each LA is defined using LA Identity (LAI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. LAI is specified in 3GPP TS 23.003 section 4.1. The list SHALL be LA values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/RAList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of Routing Areas. Each RA is defined using RA Identity (RAI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. LAI is specified in 3GPP TS 23.003 section 4.2. The list SHALL be RA values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/CellList |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  |  | Required | ZeroOrOne | chr | Get |  |
|  | This node defines the list of cells. Each cell is defined using Cell Identity (CI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. CI is specified in 3GPP TS 23.003 section 4.3. The list SHALL be CI values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/SampIntrvl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | int | Get |  |
|  | This leaf node contains an integer value indicating the sampling interval in seconds at which Diagnostics and Monitoring data are collected on the device. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for instances of QoS configuration parameters. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This node specifies the QoS parameter name to be measured. When using the QoS parameters from ETSI TS 102 250-2, the parameter name SHALL be as specified in this specification (e.g. Streaming Video Quality). |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSLowerThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as lower threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSUpperThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as upper threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonConfig/ReportCondition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This interior node acts as a placeholder to contain different reporting conditions which indicates when Diagnostics and Monitoring data is reported back to the Diagnostics and Monitoring System via Generic Alert and subsequent Get commands mechanism |  |  |  |  |

| <x>/DiagMonConfig/ReportCondition/Frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | int | Get |  |
|  | This leaf node contains an integer value indicating the frequency at which alert about Diagnostics and Monitoring data is reported back to the Diagnostics and Monitoring System. The Diagnostics and Monitoring System MAY use Get command on the DiagMonData node to retrieve diagnostics data. Note: Each diagnostic function will define what the set of diagnostic data is to be collected. Values; Description. 0; The Device SHOULD NOT send generic alert to Diagnostics and Monitoring System when the set of diagnostics data has been completely collected.. 1; The Device MUST send generic alert as soon as practical to Diagnostics and Monitoring System when the set of diagnostics data has been completely collected.. any other value greater than 1; The Device MUST send generic alert as soon as practical to Diagnostics and Monitoring System after the nth set of diagnostics data has been completely collected.. |

| <x>/DiagMonConfig/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This interior node allows for the extensions on report configurations. |  |  |  |  |

| <x>/DiagMonData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This node identifies the collected Diagnostics and Monitoring data, which SHOULD be retrieved by the Diagnostics and Monitoring System. The collected DiagMon data results as identified by leaf nodes that are attached to this node are created dynamically and have appropriate format properties as per [DMTND]. Devices MUST support adding of leaf nodes under /<x>/DiagMonData. Devices SHOULD support these leaf nodes formatted in xml. |  |  |  |  |

| <x>/DiagMonData/<X> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This required node is a placeholder for the various collected RF data sets. The node name is constructed as the time the RF parameter measurements were taken, encoded per [ISO8601] |  |  |  |  |

| <x>/DiagMonData/<X>/PSC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node contains the “Primary Scrambling Code”, as defined by [3GPP-TS_25.213]. |  |  |  |  |

| <x>/DiagMonData/<X>/UARFCN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node contains the “Downlink UTRA Absolute Radio Frequency Channel Number”, as defined by [3GPP-TS_25.433]. |  |  |  |  |

| <x>/DiagMonData/<X>/CPICH_RSCP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the “Common Pilot Channel Received Signal Code Power”, as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225]. |  |  |  |  |

| <x>/DiagMonData/<X>/CPICH_Ec_No |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the ratio of the received energy per PN chip for the Common Pilot Channel to the total received noise power spectral density as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225]. |  |  |  |  |

| <x>/DiagMonData/<X>/TxPwr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains a measure of the total transmit power, as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225], “UE Transmitted Power”. |  |  |  |  |

| <x>/DiagMonData/<X>/TransportChannel_BLER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This node is a placeholder for the values of the transport channel block error rate measurements and channel IDs |  |  |  |  |

| <x>/DiagMonData/<X>/TransportChannel_BLER/<X> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This node is a placeholder for the BLER measurements and channel ID. |  |  |  |  |

| <x>/DiagMonData/<X>/TransportChannel_BLER/<X>/BLER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the value of the “Transport Channel Block Error Rate” as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225]. |  |  |  |  |

| <x>/DiagMonData/<X>/TransportChannel_BLER/<X>/TrChID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get |  |
|  | This node contains the value of the “Transport Channel Identity” as defined by [3GPP-TS_25.331]. |  |  |  |  |

| <x>/DiagMonData/<X>/UTRARSSI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the value of the UTRAN Received Signal Strength Indication, as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225], “UTRA Carrier RSSI”. |  |  |  |  |

| <x>/DiagMonData/<X>/RxWBPwr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the value of the total received wideband power, as defined by [3GPP-TS_25.215] and [3GPP-TS_25.225], “Received total wide band power”. |  |  |  |  |

| <x>/DiagMonData/<X>/Location |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bin | Get |  |
|  | This optional leaf node contains the location where the measurements were taken, with the data encoded as per [3GPP-TS_23.032], section 7.3.1. Note that if the location is not desired by the server, or if the location data is unavailable, the node will not be present. |  |  |  |  |

| <x>/DiagMonData/<X>/CellID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node contains the value of the cell identifier – the CellGlobalIDEutra asdefined in [3GPP-TS_36.331]. |  |  |  |  |

| <x>/DiagMonData/<X>/ProbDesc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node contains the text describing any problem that may have occurred during the measurement. It is up to the device to determine if a problem description should be attached or not. |  |  |  |  |

| <x>/DiagMonData/<X>/Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node contains the configuration of the UE at the time of measurement – this node may be used to store data from an error condition. |  |  |  |  |

| <x>/Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Start |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Exec |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Stop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | null | Exec |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

### 3GPP LTE Metrics

#### Introduction

If the device exposes LTE RF functionality to the DiagMon Client, then the DiagMon Client MUST support this function. The DiagMon Server MUST support this function.

This function allows the Diagnostics and Monitoring system to retrieve 3GPP LTE RF parameters, measured by the device, that provide an indication of the RF environment at the time of function invocation.  The measured parameters include Network Measurement Results as specified in [3GPP-TS_44018] as well as transmit and receive power and signal quality indicators as specified in [3GPP-TS_36.214]. This function is only applicable to 3GPP LTE devices.

This function MUST be invoked explicitly. The status of this DiagMon function can be reported asynchronously, using the Generic Alert mechanism [DMPRO] or it can be stored in the DM Tree for later retrieval.

#### Non-applicable nodes from DiagMon MO definition

None

#### Function Description

# Figure 9 - 3GPP LTE RF Metrics Function

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is a placeholder for the 3GPP LTE RF Metrics data. Management Object Identifier for the 3GPP LTE RF Metrics MO MUST be: “urn:oma:mo:oma-diag:RFParms_3GPP_LTE:1.0”. |  |  |  |  |

| <x>/ServerID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/SaveLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | bool | Get, Replace |  |
|  | This leaf node indicates whether the location information is requested to be saved. If the device is not able to determine the location, it is not required to save it. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This node acts as a placeholder for circular geographic locations. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get |  |
|  | This node acts as a placeholder for one or more circular geographic locations. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Lat |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get, Replace |  |
|  | This leaf node contains the latitude of the circular area as defined in [3GPP-TS_23.032], section 6.1. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Long |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | Get, Replace |  |
|  | This leaf node contains the longitude of the circular area as defined in [3GPP-TS_23.032], section 6.1. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/GeoLocation/<x>/Radius |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the radius of the circular area as defined in [3GPP-TS_23.032], section 6.3. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/TAList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node defines the list of Location Areas. Each TA is defined using TA Identity (TAI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. TAI is specified in [3GPP-TS_24.301] section 9.9.9.32. The list SHALL be LA values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/CellList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node defines the list of cells. Each cell is defined using Cell Identity (CI), broadcast by each UTRAN cell via RRC protocol over the Broadcast Channel. CI is specified in 3GPP TS 36.331. The list SHALL be CI values separated by commas. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/SampIntrvl |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | int | Get |  |
|  | This leaf node contains an integer value indicating the sampling interval in seconds at which Diagnostics and Monitoring data are collected on the device. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This node is a placeholder for instances of QoS configuration parameters. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | This node specifies the QoS parameter name to be measured. When using the QoS parameters from ETSI TS 102 250-2, the parameter name SHALL be as specified in this specification (e.g. Streaming Video Quality). |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSLowerThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as lower threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonConfig/ConfigParms/<x>/QoSUpperThreshold |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get, Replace |  |
|  | This node specifies the value of QoS parameters to be used as upper threshold. The value format MUST be interpreted according to parameter definition. If this node is not present or its value is null, then no threshold MUST be applied. |  |  |  |  |

| <x>/DiagMonData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This node identifies the collected Diagnostics and Monitoring data, which SHOULD be retrieved by the Diagnostics and Monitoring System. The collected DiagMon data results as identified by leaf nodes that are attached to this node are created dynamically and have appropriate format properties as per [DMTND]. Devices MUST support adding of leaf nodes under /<x>/DiagMonData. Devices SHOULD support these leaf nodes formatted in xml. |  |  |  |  |

| <x>/DiagMonData/<X> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This required node is a placeholder for the various collected RF data sets. The node name is constructed as the time the RF parameter measurements were taken, encoded per [ISO8601] |  |  |  |  |

| <x>/DiagMonData/<X>/PCID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node contains the “Physical Cell ID”, as defined by [3GPP-TS_36.211]. |  |  |  |  |

| <x>/DiagMonData/<X>/EARFCN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node contains the “Downlink E-UTRA Absolute Radio Frequency Channel Number”, as defined by [3GPP-TS_36.101]. |  |  |  |  |

| <x>/DiagMonData/<X>/RSRP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the “Received Signal Received Power”, as defined by [3GPP-TS_36.214], encoded in float values. |  |  |  |  |

| <x>/DiagMonData/<X>/RSRQ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the “Reference Signal Received Quality” (RSRQ) is defined as the ratio NxRSRP/(E-UTRA carrier RSSI), where N is the number of RB’s of the E-UTRA carrier RSSI measurement bandwidth, as defined by [3GPP-TS_36.214]. |  |  |  |  |

| <x>/DiagMonData/<X>/RSSI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains the “Received Signal Strength Indication” (RSSI), as defined by [3GPP-TS_36.214]. |  |  |  |  |

| <x>/DiagMonData/<X>/TxPwrHdrm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | float | Get |  |
|  | This node contains a measure of the power headroom (device maximum transmit power minus current transmit power), as defined by [3GPP-TS_36.213], “Power Headroom”. |  |  |  |  |

| <x>/DiagMonData/<X>/Location |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | xml | Get |  |
|  | This node contains the location where the measurements were taken, with the data encoded as per [3GPP-TS_23.032], section 7.3.1. Note that if the location is not desired by the server, or if the location data is unavailable, the node will not be present. |  |  |  |  |

| <x>/DiagMonData/<X>/CellID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This optional leaf node contains the value of the cell identifier – the CellGlobalIDEutra as defined in [3GPP-TS_36.331] |  |  |  |  |

| <x>/DiagMonData/<X>/ProbDesc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node contains the text describing any problem that may have occurred during the measurement. It is up to the device to determine if a problem description should be attached or not. |  |  |  |  |

| <x>/DiagMonData/<X>/Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node contains the configuration of the UE at the time of measurement – this node may be used to store data from an error condition. |  |  |  |  |

| <x>/Operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Start |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | null | Exec |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Operations/Stop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | null | Exec |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Status |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |

| <x>/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | See [DiagMonTS] for description of this node. |  |  |  |  |