Change Request

| Title: | OMA-IOP-2014-0031-CR_RCSint_HTTP_File_Transfer | Public OMA Confidential |
| --- | --- | --- |
| To: | IOP |  |
| Doc to Change: | New Baseline document (based on OMA-ETS-RCS_INT-V1_2_2-20130423-C) |  |
| Submission Date: | 13 April 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Emily Bateman, Comverse, <emily.candell@comverse.com> Elad Granot, Comverse, <Elad.Granot@comverse.com> |  |
| Replaces: | N/A |  |

Reason for Change

As discussed in Sorrento:

- IOP will create a new baseline and import all existing test cases.

- New test cases will be added.

- An annex table will show which tests are applicable for each RCS version.

This CR refers to step 2 above and adds new test cases for (1-to-1) file transfer over HTTP with active session established:

- For 1-to-1 session

- For Group session

Impact on Backward Compatibility

N/A.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The IOP group is recommended to accept the proposed addition to the INT ETS.

Detailed Change Proposal

Add interoperability test case for 1-to-1 HTTP File Transfer with an active chat established

| Test Case Id | RCS_5.1-int-003 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful HTTP File Transfer with delivered and display notifications with and active 1-to1 chat established |
| Specification Reference | 3.5.4.8.3.1 |
| Preconditions | - User A and B are IMS registered and the registration timer for each user is far from expiration - RCS services have been successfully configured on the phone/(U)SIM pair for both User A and B - HTTP File transfer is enabled for User A and User B - Display notifications are not switched off. - File Transfer Auto-Accept is enabled - User A and User B are in an active 1-to-1 chat |
| Test Procedure | - A selects one file from the local storage and then selects the file transfer option. - A selects user B as the recipient for the file transfer and chooses to send the file - User B receives and accepts the file download notification if prompted. - User B opens the file. |
| Pass-Criteria | - In step 2, A sees that file transfer option is available for User B - After step 2, B receives file transfer download notification and sees the size of the file and an indication of the file type (e.g. MIME icon or mini-preview). - After step 3, the file is transferred and received. The recipient can access the file (e.g. from storage on the handset). - After step 4, delivery and display notification are received and shown on client A. |

Add interoperability test case for HTTP File Transfer during an active group chat

| Test Case Id | RCS_5.1-int-006 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful HTTP File Transfer with delivery notifications during an active group chat |
| Specification Reference | 3.5.4.8.3 |
| Preconditions | - User A, B and C are IMS registered and the registration timer for each user is far from expiration - RCS services have been successfully configured on the phone/(U)SIM pair for both Users A, B and C - HTTP File transfer is enabled for User A, B and C - File Transfer Auto-Accept is enabled for User B and User C - Users A, B and C are in an active group chat |
| Test Procedure | - User A selects the share option within the group chat window and chooses a file from local storage - User A sends the file - User B receives and accepts the file download notification if prompted. - User C receives and accepts the file download notification if prompted. - User B opens the file. - User C opens the file. |
| Pass-Criteria | - After step 2, B receives file transfer download notification and sees the size of the file and an indication of the file type (e.g. MIME icon or mini-preview). - After step 2, C receives file transfer download notification and sees the size of the file and an indication of the file type (e.g. MIME icon or mini-preview). - After step 3, the file is transferred and received by User B. The recipient can access the file (e.g. from storage on the handset). - After step 4, the file is transferred and received by User C. The recipient can access the file (e.g. from storage on the handset). - After step 5, User A is notified that at least one recipient received the file - After step 6, User A is notified that all recipients have received the file |