Change Request

| Title: | OMA-IOP-2014-0031-CR_RCSint_HTTP_File_Transfer | Public OMA Confidential |
| --- | --- | --- |
| To: | IOP |  |
| Doc to Change: | New Baseline document (based on OMA-ETS-RCS_INT-V1_2_2-20130423-C) |  |
| Submission Date: | 13 April 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Emily Bateman, Comverse, <emily.candell@comverse.com> Elad Granot, Comverse, <Elad.Granot@comverse.com> |  |
| Replaces: | N/A |  |

Reason for Change

As discussed in Sorrento:

- IOP will create a new baseline and import all existing test cases.

- New test cases will be added.

- An annex table will show which tests are applicable for each RCS version.

This CR refers to step 2 above and adds new test cases for file transfer over HTTP with service interruption while file is being uploaded to server:

- For sender

- For recipient

Impact on Backward Compatibility

N/A.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The IOP group is recommended to accept the proposed addition to the INT ETS.

Detailed Change Proposal

Add interoperability test case for 1-to-1 HTTP File Transfer with temporary service interruption while file is being uploaded

| Test Case Id | RCS_5.1-int-004 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful HTTP File Transfer with delivered and display notifications with temporary service interruption while file is being uploaded |
| Specification Reference | 3.5.4.8.3.1 |
| Preconditions | - User A and B are IMS registered and the registration timer for each user is far from expiration - RCS services have been successfully configured on the phone/(U)SIM pair for both User A and B - HTTP File transfer is enabled for User A and User B - Display notifications are not switched off. - File Transfer Auto-Accept is enabled |
| Test Procedure | - A selects one file from the local storage and then selects the file transfer option. - A selects user B as the recipient for the file transfer and chooses to send the file - While file is being uploaded to server (based on status bar progress), disconnect the device from the network - Reconnect the device to the network - User B receives and accepts the file download notification if prompted. - User B opens the file. |
| Pass-Criteria | - In step 2, A sees that file transfer option is available for User B - After step 3, file upload status does not show active progress - After step 4, B receives file transfer download notification and sees the size of the file and an indication of the file type (e.g. MIME icon or mini-preview). - After step 3, the file is transferred and received. The recipient can access the file (e.g. from storage on the handset). - After step 4, delivery and display notification are received and shown on client A. |

Add interoperability test case for 1-to-1 HTTP File Transfer with temporary service interruption while file is being downloaded from server

| Test Case Id | RCS_5.1-int-005 |
| --- | --- |
| Test Object | Client/server |
| Test Case Description | Verifies successful HTTP File Transfer with delivered and display notifications with temporary service interruption while file is being downloaded from server |
| Specification Reference | 3.5.4.8.3.2 |
| Preconditions | - User A and B are IMS registered and the registration timer for each user is far from expiration - RCS services have been successfully configured on the phone/(U)SIM pair for both User A and B - HTTP File transfer is enabled for User A and User B - Display notifications are not switched off. - File Transfer Auto-Accept is enabled |
| Test Procedure | - A selects one file from the local storage and then selects the file transfer option. - A selects user B as the recipient for the file transfer and chooses to send the file - User B receives and accepts the file download notification if prompted. - While file is being downloaded (based on status bar progress), disconnect the device from the network - Reconnect the device to the network - User B opens the file. |
| Pass-Criteria | - In step 2, A sees that file transfer option is available for User B - After step 3, B receives file transfer download notification and sees the size of the file and an indication of the file type (e.g. MIME icon or mini-preview). - After step 4, file download status does not show active progress - After step 5, the file is transferred and received. The recipient can access the file (e.g. from storage on the handset). - After step 4, delivery and display notification are received and shown on client A. |