| # Enabler Test Specification for LPPe V1.0 |  |  |  |
| --- | --- | --- | --- |
| Candidate Version 1.0 – 29 Aug 2017 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-ETS-LPPe-V1_0-20170829-C |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2017 Open Mobile Alliance All Rights Reserved.
Used with the permission of the Open Mobile Alliance under the terms set forth above.

Contents

1.	Scope	4

2.	References	5

2.1	Normative References	5

2.2	Informative References	5

3.	Terminology and Conventions	6

3.1	Conventions	6

3.2	Definitions	6

3.3	Abbreviations	6

4.	Introduction	7

4.1	Running Test Cases	7

4.2	LPP and LPPe Scenarios	7

5.	LPPe Conformance Test Cases	8

5.1	Client Conformance: Normal Mode	8

5.1.1	Basic Functionality	8

5.1.2	LPP/LPPe Interworking	15

5.1.3	LPPe Cross Version Compatibility	15

6.	LPPe Interoperability Test Cases	16

6.1	LPPe-1.0-int-001 - Client reject based on the compatibility level	16

6.2	LPPe-1.0-int-002 - Server reject based on the compatibility level	16

6.3	LPPe-1.0-int-003 - Client version support	17

6.4	LPPe-1.0-int-004 - Server version support	18

6.5	LPPe-1.0-int-013 – Short Range Nodes (Includes Optional Features)	18

Appendix A.	Change History (Informative)	20

A.1	Approved Version History	20

A.2	Draft/Candidate Version 1.0 History	20

Appendix B.	Conformance Test Case applicability	21

B.1	Introduction	21

B.2	Test Cases testing only mandatory features	21

B.3	Applicability	21

B.3.1	Client ICS	21

B.3.2	Client IXIT	25

B.3.3	Server ICS	26

B.3.4	Server IXIT	26

B.4	ICS to test case mapping	26

# Scope

This document describes in detail available test cases for LPP Extensions Release V1.0, OMA-TS-LPPe-V1_0

# URL:http://www.openmobilealliance.org/

The test cases are split in two categories, conformance and interoperability test cases.

The conformance test cases are aimed at verifying the adherence to normative requirements described in the technical specifications. In this current version of this document, the conformance test cases cover the requirements of the initial deployments of LPPe only. Future deployments may require additional conformance test cases.

The interoperability test cases are aimed at verifying that implementations of the specifications work satisfactory.

# References

## Normative References

| [3GPP-RRC] | 3GPP TS 36.331 Radio Resource Control (RRC), Protocol specification. http://www.3gpp.org |
| --- | --- |
| [3GPP-LPP] | 3GPP TS 36.355 LTE Positioning Protocol, http://www.3gpp.org |
| [3GPP-37571-5] | 3GPP TS 37.571-5 UE conformance specification for UE positioning Part 5: Test scenarios and assistance data, http://www.3gpp.org |
| [LPPe-TS] | OMA LPP Extensions Technical Specification v1.0, Open Mobile Alliance™, OMA-TS-LPPe-V1_0, http://www.openmobilealliance.org |
| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| [SUPL-ETS] | Enabler Test Specification for SUPL V2.0.3, OMA-ETS-SUPL-V2_0_3, http://www.openmobilealliance.org |
| [ULP2-TS] | UserPlane for Location Protocol v2.0, Open Mobile Alliance™, OMA-TS-ULP-V2_0, URL:http://www.openmobilealliance.org/ |
| [ULP3-TS] | UserPlane for Location Protocol v3.0, Open Mobile Alliance™, OMA-TS-ULP-V3_0, URL:http://www.openmobilealliance.org/ |

## Informative References

| [OMADICT] | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |
| --- | --- |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope”, are normative, unless they are explicitly indicated to be informative.

The following numbering scheme is used:

xxx-y.z-con-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’con’	Indicating this test is a conformance test case
number	Leap number for the test case

Or

xxx-y.z-int-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’int’	Indicating this test is a interoperability test case
number	Leap number for the test case

## Definitions

## Abbreviations

| 3GPP | Third Generation Partnership Project |
| --- | --- |
| BLE | Bluetooth Low Energy |
| GNSS | Global Navigation Satellite System |
| LPP | 3GPP TS 36.355 LTE Positioning Protocol [3GPP-LPP] |
| LPPe | OMA LPP Extensions [LPPe-TS] |
| LTE | Long Term Evolution |
| OMA | Open Mobile Alliance |
| SET | SUPL Enabled Terminal |
| SLP | SUPL Location Platform |
| SUPL | Secure User Plane Location [ULP2-TS] [ULP3-TS] |
| UE | User Equipment |
| WLAN | Wireless Local Area Network |

# Introduction

The purpose of this document is to provide test cases for LPP Extensions Enabler Release 1.0 [LPPe-TS].

The implementation of some features is optional for the Clients and/or the Servers in the LPPe Enabler.  The tests associated with these optional features are marked as "(Includes Optional Features)" in the test specification.

The successful testing of LPPe requires an LPPe Client and an LPPe Server. LPPe messages need to be encapsulated in 3GPP LPP messages [3GPP-LPP]. To run an LPPe test scenario, either a Control Plane [3GPP – LPP] or a User Plane (OMA SUPL 2.0 - [ULP2-TS]) implementation is required.

## Running Test Cases

Some Test Cases are made up of multiple Tests (e.g. Test 1, Test 2 …). These Tests are independent of each other and are only grouped together for convenience. These Tests may or may not all be relevant to a particular UE or SET or SLP and applicabilities are given separately for each Test.

These Tests may be referred to by adding “-1”, or “-2” etc. after the Test number, e.g. LPPe-1.0-con-008-1, LPPe-1.0-con-008-2 etc.

Some Test Cases are made up of multiple Cases (e.g. Case 1, Case 2…). These Cases are all part of the one Test Case and should all be run as part of the Test Case. In particular, to pass the complete Test Case the UE or SET or SLP must pass every Case.

NOTE: Some SETs/UEs might require to establish an E911 call in order to perform the required measurements. These SETs/UEs should indicate this requirement with the ixit_emergency_call_required set to yes.

## LPP and LPPe Scenarios

Some Test Cases in this document require that a Location Session is run. For these sessions suitable scenarios and possibly also Assistance Data are required. Suitable scenarios and Assistance Data for A-GNSS, OTDOA and ECID are defined in the Enabler Test Specification for SUPL V2.0 [SUPL-ETS], Sections 4.4 to 4.6. Suitable scenarios and Assistance Data for WLAN and BLE are defined in [3GPP-37571-5] section 9.1.1 and 10.1.1.

# LPPe Conformance Test Cases

## Client Conformance: Normal Mode

### Basic Functionality

#### LPPe-1.0-con-001- Verification of the LPP EPDU-ID and LPPe message header

| Test Case Id | LPPe-1.0-con-001 |
| --- | --- |
| Test Object | Client |
| Test Case Description | To verify correct setting of the LPP EPDU-ID and the LPPe message header |
| Specification Reference | LPPe TS 5.1, 6.2.2 |
| SCR Reference | LPPe-MSG-C-001-M |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 |
| Test Procedure | - Start an LPP/LPPe Session - If the LPPe Client does not immediately send an LPP/LPPe Provide Capabilities message (according to ixit_immediate_provideCapabilities_message_sent) then send an LPP/LPPe Request Capabilities message with: - LPPe-CommonIEsRequestCapabilities: all IEs set to request all capabilities - The LPPe Client sends a LPP/LPPe Provide Capabilities message with: - the EPDU-ID in the LPP message set to 1. - in the LPPe message header: - LPPeCompatibilityLevel set to 0 - majorVersion set to 1 - minorVersion set to 0 - LPPeMode set to normal - End the LPP/LPPe session |
| Pass-Criteria | - At step 3 the LPPe client shall set: - the EPDU-ID in the LPP message to 1 and in the LPPe message header: - LPPeCompatibilityLevel set to 0 - majorVersion set to 1 - minorVersion set to 0 - LPPeMode set to normal |

#### LPPe-1.0-con-004- Basic LPPe support (Capabilities)

| Test Case Id | LPPe-1.0-con-004 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verify the LPPe Client correctly supports LPPe Request/Provide Capabilities |
| Specification Reference | LPPe TS 6.2.2, 6.3.1, 6.3.2 Test 1: LPPe TS 6.5.1.9, 6.5.1.10, 6.5.1.11 Test 2: LPPe TS 6.5.2.7, 6.5.2.8 Test 3: LPPe TS 6.5.3.7, 6.5.3.8 Test 4: LPPe TS 6.5.4.7, 6.5.4.8 Test 5: LPPe TS 6.5.5.7, 6.5.5.8 Test 6: LPPe TS 6.5.6.6, 6.5.6.7 Test 7: LPPe TS 6.5.7.7, 6.5.7.8 Test 8: LPPe TS 6.5.8.7, 6.5.8.8 Test 9: LPPe TS 6.5.9.4, 6.5.9.5 Test 10: LPPe TS 6.5.10.9, 6.5.10.10, 6.5.10.11 Test 11: LPPe TS 6.5.11.9, 6.5.11.10, 6.5.11.11 |
| SCR Reference | LPPe-MSG-C-003-M LPPe-MSG-C-004-M |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 - ics: Test 1: ics_Agnss Test 2 ics_Otdoa Test 3: ics_Eotd Test 4: ics_Otdoa_Utra Test 5: ics_Ecid_Lte Test 6: ics_Ecid_Gsm Test 7: ics_Ecid_Utra Test 8: ics_Wlan_Ap Test 9: ics_Ecid_Wimax Test 10: ics_Sensor Test 11a: ics_Srn_Bluetooth Test 11b: ics_Srn_Bluetooth_Low_Energy Test 11c: ics_Srn_Near_Field_Communications Test 11d: ics_Srn_Oma_Mobile_Codes - Test 11e: ics_Srn_Other |
| Test Procedure | Editors note: These subtest might be combined into one after further analysis Test 1: AGNSS (FFS) Test 2: OTDOA (FFS) Test 3: EOTD (FFS) Test 4: OTDOA UTRA (FFS) Test 5: ECID LTE (FFS) Test 6: ECID GSM (FFS) Test 7: ECID UTRA (FFS) Test 8: WLAN AP Test 9: ECID WIMAX (FFS) Test 10: Sensor Test 11a: SRN for SRN Technology: Bluetooth (FFS) Test 11b: SRN for SRN Technology: Bluetooth Low Energy Test 11c: SRN for SRN Technology: Near Field Communications (FFS) Test 11d: SRN for SRN Technology: OMA Mobile Codes (FFS) Test 11e: SRN for SRN Technology: Other (FFS) - Start an LPP/LPPe session - If the LPPe Client does not immediately send an LPP/LPPe Provide Capabilities message (according to ixit_immediate_provideCapabilities_message_sent) then send an LPP/LPPe Request Capabilities message with: - LPPe-CommonIEsRequestCapabilities: all IEs set to request all capabilities - Test 1: LPPe-AGNSS-RequestCapabilities: all IEs set to request all capabilities - Test 2: LPPe-OTDOA-RequestCapabilities: all IEs set to request all capabilities - Test 3: LPPe-EOTD-RequestCapabilities: all IEs set to request all capabilities - Test 4: LPPe-OTDOA-UTRA-RequestCapabilities: all IEs set to request all capabilities - Test 5: LPPe-ECID-LTE-RequestCapabilities: all IEs set to request all capabilities - Test 6: LPPe-ECID-GSM-RequestCapabilities: all IEs set to request all capabilities - Test 7: LPPe-ECID-UTRA-RequestCapabilities: all IEs set to request all capabilities - Test 8: LPPe-WLAN-AP-RequestCapabilities: all IEs set to request all capabilities - Test 9: LPPe-ECID-WiMax-RequestCapabilities: all IEs set to request all capabilities - Test 10: LPPe-Sensor-RequestCapabilities: all IEs set to request all capabilities - Test 11: LPPe-SRN-RequestCapabilities: all IEs set to request all capabilities - The LPPe Client sends an LPP/LPPe Provide Capabilities message with all the relevant IEs of the following fields of LPPe-ProvideCapabilities, depending on the features supported by the Client as shown by the relevant Client Provide Capabilities ics in Appendix B: Note that in the case that the LPPe Client immediately sends an LPP/LPPe Provide Capabilities message after step 1 (according to ixit_immediate_provideCapabilities_message_sent) then more than just the relevant fields of LPPe-ProvideCapabilities listed below may be sent. - LPPe-CommonIEsProvideCapabilities - Test 1: LPPe-AGNSS-ProvideCapabilities - Test 2: LPPe-OTDOA-ProvideCapabilities - Test 3: LPPe-EOTD-ProvideCapabilities - Test 4: LPPe-OTDOA-UTRA-ProvideCapabilities - Test 5: LPPe-ECID-LTE-ProvideCapabilities - Test 6: LPPe-ECID-GSM-ProvideCapabilities - Test 7: LPPe-ECID-UTRA-ProvideCapabilities - Test 8: LPPe-WLAN-AP-ProvideCapabilities - Test 9: LPPe-ECID-WiMax-ProvideCapabilities - Test 10: LPPe-Sensor-ProvideCapabilities - Test 11: LPPe-SRN-ProvideCapabilities - End the LPP/LPPe session - Repeat steps 1 to 4 for each subtest supported by the Client |
| Pass-Criteria | At step 3 the Client shall send an LPP/LPPe Provide Capabilities message with the correct LPPe Client capabilities for the features supported by the Client according to relevant Client Provide Capabilities ics in Appendix B. |

#### LPPe-1.0-con-005- Basic LPPe support (Assistance Data) (Includes Optional Features)

| Test Case Id | LPPe-1.0-con-005 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verification of the message extension for LPPe Request/Provide Assistance Data |
| Specification Reference | LPPe TS 6.2.2 |
| SCR Reference | LPPe-MSG-C-005-O LPPe-MSG-C-006-O |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 |
| Test Procedure | FFS |
| Pass-Criteria | FFS |

#### LPPe-1.0-con-006- Basic LPPe support (Location Information)

| Test Case Id | LPPe-1.0-con-006 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verification of the message extension for LPPe Request/Provide Location Information |
| Specification Reference | LPPe TS 6.2.2, 6.3.5, 6.3.6 Test 1: LPPe TS 6.5.1.5, 6.5.1.6, 6.5.1.7, 6.5.1.8 Test 2: LPPe TS 6.5.2.4, 6.5.2.5, 6.5.2.6 Test 3: LPPe TS 6.5.3.5, 6.5.3.6 Test 4: LPPe TS 6.5.4.4, 6.5.4.5, 6.5.4.6 Test 5: LPPe TS 6.5.5.4, 6.5.5.5, 6.5.5.6 Test 6: LPPe TS 6.5.6.4, 6.5.6.5 Test 7: LPPe TS 6.5.7.4, 6.5.7.5, 6.5.7.6 Test 8: LPPe TS 6.5.8.4, 6.5.8.5, 6.5.8.6 Test 9: LPPe TS 6.5.9.2, 6.5.9.3 Test 10: LPPe TS 6.5.10.5, 6.5.10.6, 6.5.10.7, 6.5.10.8 Test 11: LPPe TS 6.5.11.5, 6.5.11.6, 6.5.11.7, 6.5.11.8 |
| SCR Reference | LPPe-MSG-C-007-M LPPe-MSG-C-008-M |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 ics: Test 1: ics_Agnss Test 2 ics_Otdoa Test 3: ics_Eotd Test 4: ics_Otdoa_Utra Test 5: ics_Ecid_Lte Test 6: ics_Ecid_Gsm Test 7: ics_Ecid_Utra Test 8: ics_Wlan_Ap Test 9: ics_Ecid_Wimax Test 10: ics_Sensor Test 11a: ics_Srn_Bluetooth Test 11b: ics_Srn_Bluetooth_Low_Energy Test 11c: ics_Srn_Near_Field_Communications Test 11d: ics_Srn_Oma_Mobile_Codes - Test 11e: ics_Srn_Other |
| Test Procedure | Test 1: AGNSS (FFS) Test 2: OTDOA (FFS) Test 3: EOTD (FFS) Test 4: OTDOA UTRA (FFS) Test 5: ECID LTE (FFS) Test 6: ECID GSM (FFS) Test 7: ECID UTRA (FFS) Test 8: WLAN AP Test 9: ECID WIMAX (FFS) Test 10: Sensor Test 11a: SRN for SRN Technology: Bluetooth (FFS) Test 11b: SRN for SRN Technology: Bluetooth Low Energy Test 11c: SRN for SRN Technology: Near Field Communications (FFS) Test 11d: SRN for SRN Technology: OMA Mobile Codes (FFS) Test 11e: SRN for SRN Technology: Other (FFS) - Start an LPP/LPPe session - If the LPPe Client does not immediately send an LPP/LPPe Provide Capabilities message (according to ixit_immediate_provideCapabilities_message_sent) then send an LPP/LPPe Request Capabilities message with: - LPPe-CommonIEsRequestCapabilities: all IEs set to request all capabilities - The LPPe Client sends an LPP/LPPe Provide Capabilities message. - Send an LPP/LPPe Request Location Information message with: Editor’s note: we may need to provide more detailed information here - FFS - LPPe-CommonIEsRequestLocationInformation: IEs set to request the following Location Information: - Test 1: LPPe-AGNSS-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 2: LPPe-OTDOA-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 3: LPPe-EOTD-RequestLocationInformation: IEs set to request the followingLocation Information: - FFS - Test 4: LPPe-OTDOA-UTRA-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 5: LPPe-ECID-LTE-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 6: LPPe-ECID-GSM-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 7: LPPe-ECID-UTRA-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 8: LPPe-WLAN-AP-RequestLocationInformation: IEs set to request the following Location Information: - requestedMeasurements set to request all measurements supported by the client as indicated by ics_wlan-ecid-MeasSupported - additionalRequestedMeasurements set to request all measurements supported by the client as indicated by ics_additional-wlan-ecid-MeasSupported - Test 9: LPPe-ECID-WiMax-RequestLocationInformation: IEs set to request the following Location Information: - FFS - Test 10: LPPe-Sensor-RequestLocationInformation: IEs set to request the following Location Information: - motionStateReq set to request all measurements supported by the client as indicated by ics_motionStateSupport and ics_secondarySupport - atmosphericPressureReq set to request the measurement if supported by the client as indicated by ics_atmosphericPressureSupport - Test 11a: LPPe-SRN-RequestLocationInformation for SRN Technology: Bluetooth: IEs set to request the following Location Information: - FFS - Test 11b: LPPe-SRN-RequestLocationInformation for SRN Technology: Bluetooth Low Energy: IEs set to request the following Location Information: - multipleMeasurements set to requested (2) - allowedMeasurements set to request all measurements supported by the client as indicated by ics_supportedMeasurements - historicMeasurementsRequested set to present if supported by the client as indicated by ics_historicMeasurementsSupported - Test 11c: LPPe-SRN-RequestLocationInformation for SRN Technology: Near Field Communications: IEs set to request the following Location Information: - FFS - Test 11d: LPPe-SRN-RequestLocationInformation for SRN Technology: OMA Mobile Codes: IEs set to request the following Location Information - FFS - Test 11e: LPPe-SRN-RequestLocationInformation for SRN Technology: Other: IEs set to request the following Location Information: - FFS - The LPPe Client sends an LPP/LPPe Provide Location Information message with all the relevant IEs of the following fields of LPPe-ProvideLocationInformation, depending on the features requested in step 4: Editor’s note: we may want to provide more detailed information here - FFS - LPPe-CommonIEsProvideLocationInformation: - According to the features requested in step 4 - Test 1: LPPe-AGNSS-ProvideLocationInformation: - FFS - Test 2: LPPe-OTDOA-ProvideLocationInformation: - FFS - Test 3: LPPe-EOTD-ProvideLocationInformation: - FFS - Test 4: LPPe-OTDOA-UTRA-ProvideLocationInformation: - FFS - Test 5: LPPe-ECID-LTE-ProvideLocationInformation: - FFS - Test 6: LPPe-ECID-GSM-ProvideLocationInformation: - FFS - Test 7: LPPe-ECID-UTRA-ProvideLocationInformation: - FFS - Test 8: LPPe-WLAN-AP-ProvideLocationInformation: - According to the featuresrequested in step 4 - Test 9: LPPe-ECID-WiMax-ProvideLocationInformation: - FFS - Test 10: LPPe-Sensor-ProvideLocationInformation: - According to the features requested in step 4 - Test 11a: LPPe-SRN-ProvideLocationInformation for SRN Technology: Bluetooth: - FFS - Test 11b: LPPe-SRN-ProvideLocationInformation for SRN Technology: Bluetooth Low Energy: - According to the featuresrequested in step 4 - Test 11c: LPPe-SRN-ProvideLocationInformation for SRN Technology: Near Field Communications: - FFS - Test 11d: LPPe-SRN-ProvideLocationInformation for SRN Technology: OMA Mobile Codes: - FFS - Test 11e: LPPe-SRN-ProvideLocationInformation for SRN Technology: Other: - FFS - End the LPP/LPPe session - Repeat steps 1 to 6 for each subtest supported by the Client |
| Pass-Criteria | 1. At step 5, the Client shall return an LPP/LPPe Provide Location Information message with the correct LPPe Client location information for the features requested in step 4. |

#### LPPe-1.0-con-007- Basic LPPe support (Error) (Includes Optional Features)

| Test Case Id | LPPe-1.0-con-007 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verification of the message extension for LPPe Error |
| Specification Reference | LPPe TS 6.2.2, 6.3.8 Test 1: LPPe TS 6.5.1.12 Test 2: LPPe TS 6.5.2.9 Test 3: LPPe TS 6.5.3.9 Test 4: LPPe TS 6.5.4.9 Test 5: LPPe TS 6.5.5.9 Test 6: LPPe TS 6.5.6.8 Test 7: LPPe TS 6.5.7.9 Test 8: LPPe TS 6.5.8.9 Test 9: LPPe TS 6.5.9.6 Test 10: LPPe TS 6.5.10.12 Test 11: LPPe TS 6.5.11.12 |
| SCR Reference | LPPe-MSG-C-009-O |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - FFS |
| Test Procedure | FFS |
| Pass-Criteria | FFS |

#### LPPe-1.0-con-008- Basic LPPe support (Abort) (Includes Optional Features)

| Test Case Id | LPPe-1.0-con-008 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verification of the message extension for LPPe Abort |
| Specification Reference | LPPe TS 6.2.2 |
| SCR Reference | LPPe-MSG-C-010-O |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 |
| Test Procedure | FFS |
| Pass-Criteria | FFS |

### LPP/LPPe Interworking

TBD

### LPPe Cross Version Compatibility

#### LPPe-1.0-con-031-Compatible Versions: Higher Version

Test Case not applicable to LPPe Version 1.0.

#### LPPe-1.0-con-032-Compatible Versions: Lower Version

Test Case not applicable to LPPe Version 1.0.

#### LPPe-1.0-con-033-Correct support: LPPe V2.0 and V1.0

Test Case not applicable to LPPe Version 1.0.

#### LPPe-1.0-con-035-Unsupported Version: higher version

| Test Case Id | LPPe-1.0-con-035 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verifies LPPe client correctly handles unsupported higher version |
| Specification Reference | LPPe TS 4.2 |
| SCR Reference | LPPe-MSG-C-002-M |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 |
| Test Procedure | FFS |
| Pass-Criteria | FFS |

#### LPPe-1.0-con-037- Unsupported Compatibility Level

| Test Case Id | LPPe-1.0-con-037 |
| --- | --- |
| Test Object | LPPe Client |
| Test Case Description | Verifies LPPe client correctly handles unsupported Compatibility Level |
| Specification Reference | LPPe TS 4.2 |
| SCR Reference | LPPe-MSG-C-002-M |
| Tool | LPPe Client Conformance Test Tool |
| Test code | Validated test code for this test case |
| Preconditions | - LPPe Client supporting LPPe version 1.0 with compatibility level 0 |
| Test Procedure | FFS |
| Pass-Criteria | FFS |

# LPPe Interoperability Test Cases

## LPPe-1.0-int-001 - Client reject based on the compatibility level

| Test Case Id | LPPe-1.0-int-001 |
| --- | --- |
| Test Object | LPPe Server and LPPe Client |
| Test Case Description | To test the compatibility level adaptation in the beginning of the LPPe session |
| Specification Reference | LPPe TS chapter 4.2 |
| SCR Reference | LPPe-MSG-C-002-M LPPe-MSG-S-002-M |
| ETR Reference | LPPe mandatory test requirement with Feature Key ADAPT |
| Tool | Protocol analyser to monitor signalling between LPPe Client and LPPe Server. If a protocol analyser is not available log files in Client and Server can be used instead. |
| Test code | - |
| Preconditions | Equipment: LPPe Client supporting LPPe version 1.0 with compatibility level 0 LPPe Server supporting LPPe version 2.0 with compatibility level 1 Open connection between the client and the server, which connection can be used to carry LPPe messages |
| Test Procedure | LPPe Server sends the LPPe Client any LPPe message (not Error or Abort) allowed for the server to send in Normal mode with the version indication 2.0 and compatibility level 1. |
| Pass-Criteria | LPPe Client responds with LPPe Error message indicating version 1.0 and compatibility level 0. OR No response is received at all. |

## LPPe-1.0-int-002 - Server reject based on the compatibility level

| Test Case Id | LPPe-1.0-int-002 |
| --- | --- |
| Test Object | To test the compatibility level adaptation in the beginning of the LPPe session |
| Test Case Description | To test the version adaptation |
| Specification Reference | LPPe TS chapter 4.2 |
| SCR Reference | LPPe-MSG-C-002-M LPPe-MSG-S-002-M |
| ETR Reference | LPPe mandatory test requirement with Feature Key ADAPT |
| Tool | Protocol analyser to monitor signalling between LPPe Client and LPPe Server. If a protocol analyser is not available log files in Client and Server can be used instead. |
| Test code | - |
| Preconditions | Equipment: LPPe Server supporting LPPe version 1.0 with compatibility level 0 LPPe Client supporting LPPe version 2.0 with compatibility level 1 Open connection between the client and the server, which connection can be used to carry LPPe messages |
| Test Procedure | LPPe Client sends the LPPe Server any LPPe message (not Error or Abort) message allowed for the Client to send with the version indication 2.0 and compatibility level 1. |
| Pass-Criteria | LPPe Server responds with LPPe Error message indicating version 1.0 and compatibility level 0. OR No response is received at all. |

## LPPe-1.0-int-003 - Client version support

| Test Case Id | LPPe-1.0-int-003 |
| --- | --- |
| Test Object | LPPe Server and LPPe Client |
| Test Case Description | To test the version adaptation |
| Specification Reference | LPPe TS chapter 4.2 |
| SCR Reference | LPPe-MSG-C-002-M LPPe-MSG-S-002-M |
| ETR Reference | LPPe mandatory test requirement with Feature Key ADAPT |
| Tool | Protocol analyser to monitor signalling between LPPe Client and LPPe Server. If a protocol analyser is not available log files in Client and Server can be used instead. |
| Test code | - |
| Preconditions | Equipment: LPPe Server supporting LPPe version 1.0 with compatibility level 0 LPPe Client supporting LPPe versions 1.0 and 2.0 both with compatibility level 0 Open connection between the client and the server, which connection can be used to carry LPPe messages |
| Test Procedure | Test 1: LPPe Server sends the LPPe Client any LPPe message (not Error or Abort) allowed for the LPPe Server to send in the Normal mode with the version indication 1.0 and compatibility level 0. Test 2: LPPe Client sends the LPPe Client any LPPe message (not Error or Abort) allowed for the LPPe Client to send in the Normal mode with the version indication 2.0 and compatibility level 0. |
| Pass-Criteria | Test 1: LPPe Client responds with the corresponding LPPe message indicating version 1.0 and compatibility level 0. OR LPPe Client responds with the corresponding LPPe message indicating version 2.0 and compatibility level 0. Test 2: LPPe Server responds with the corresponding LPPe message indicating version 1.0 and compatibility level 0. |

## LPPe-1.0-int-004 - Server version support

| Test Case Id | LPPe-1.0-int-004 |
| --- | --- |
| Test Object | LPPe Server and LPPe Client |
| Test Case Description | To test the version adaptation |
| Specification Reference | LPPe TS chapter 4.2 |
| SCR Reference | LPPe-MSG-C-002-M LPPe-MSG-S-002-M |
| ETR Reference | LPPe mandatory test requirement with Feature Key ADAPT |
| Tool | Protocol analyser to monitor signalling between LPPe Client and LPPe Server. If a protocol analyser is not available log files in Client and Server can be used instead. |
| Test code | - |
| Preconditions | Equipment: LPPe Client supporting LPPe version 1.0 with compatibility level 0 LPPe Server supporting LPPe versions 1.0 and 2.0 both with compatibility level 0 Open connection between the client and the server, which connection can be used to carry LPPe messages |
| Test Procedure | Test 1: LPPe Client sends the LPPe Server any LPPe message (not Error or Abort) allowed for the LPPe Client to send in the Normal mode with the version indication 1.0 and compatibility level 0. Test 2: LPPe Server sends the LPPe Client any LPPe message (not Error or Abort) allowed for the LPPe Server to send in the Normal mode with the version indication 2.0 and compatibility level 0. |
| Pass-Criteria | Test 1: LPPe Server responds with the corresponding LPPe message indicating version 1.0 and compatibility level 0. OR LPPe Server responds with the corresponding LPPe message indicating version 2.0 and compatibility level 0. Test 2: LPPe Client responds with the corresponding LPPe message indicating version 1.0 and compatibility level 0. |

## LPPe-1.0-int-013 – Short Range Nodes (Includes Optional Features)

| Test Case Id | LPPe-1.0-int-013 |
| --- | --- |
| Test Object | LPPe Server and LPPe Client |
| Test Case Description | To test UE-assisted Short Range Node positioning with Bluetooth |
| Specification Reference | LPPe TS chapter 6.5.11 |
| SCR Reference | LPPe-LOC-C-030-O, LPPe-LOC-S-030-O |
| ETR Reference | LPPe optional test requirement with Feature Key SRN |
| Tool | Protocol analyser to monitor signalling between LPPe Client and LPPe Server. If a protocol analyser is not available log files in Client and Server can be used instead. |
| Test code | - |
| Preconditions | Equipment: LPPe Client supporting LPPe version 1.0 with compatibility level 0 and Bluetooth support LPPe Server supporting LPPe version 1.0 with compatibility level 0 and Bluetooth support Open connection between the client and the server, which connection can be used to carry LPPe messages Bluetooth transmitters |
| Test Procedure | - LPPe Server requests from LPPe Client Bluetooth Short Range Node measurements - LPPe Client observes its Bluetooth environment and provides the observations to the LPPe Server |
| Pass-Criteria | LPPe Client correctly provides its Bluetooth observations to the LPPe Server |

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version |

Draft/Candidate Version 1.0 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-ETS-LPPe-V1_0 | 15 Dec 2010 | all | Baseline ETS document |
|  | 10 Feb 2011 | 6 | OMA-IOP-MEC-2011-0003-CR_New_LPPe_Short_Range_Node_test_cases.doc |
|  | 10 Feb 2017 | all | Incorporated CR: OMA-IOP-2017-0018R03-CR_LPPe_addition_of_new_test_cases |
|  | 13 Feb 2017 | n/a | Fixed the print error. |
|  | 31 Mar 2017 | 1, 2.1, 3.3, 4, 5.1, B.2, B.3 | Incorporated CRs: OMA-IOP-2017-0024R01-CR_LPPe_compatibility_tests OMA-IOP-2017-0025R01-CR_LPPe_ETS_tidy_up OMA-IOP-2017-0027R02-CR_LPPe_ETS_revise_Introduction OMA-IOP-2017-0028-CR_LPPe_ETS_merge_con_001_003 OMA-IOP-2017-0029R01-CR_LPPe_ETS_add_missing_subtests OMA-IOP-2017-0030-CR_LPPe_ETS_Normal_Mode OMA-IOP-2017-0031R01-CR_LPPe_ETS_Update_Capabilities_test OMA-IOP-2017-0033R01-CR_LPPe_ETS_Adding_ics OMA-IOP-2017-0036-CR_LPPe_ETS_revise_Scope |
|  | 05 Apr 2017 | 1, 2.1, 3.3, 4, 5.1.1.1-5.1.1.6, 5.1.3.4, 5.1.3.6, 5.2, B.2, B.3.1.1, B.4 | Incorporated CR: OMA-IOP-2017-0039-CR_LPPe_ETS_tidy_up_and_way_forward |
|  | 04 May 2017 | 4, 5.1.1.1, 5.1.1.5, 5.1.1.7-5.1.1.11, 6.5-6.12 | Incorporated CRs: OMA-IOP-2017-0046-CR_LPPe_1.0_ETS_clean_up OMA-IOP-2017-0050-CR_LPPe_ETS_revise_con_001 |
|  | 24 May 2017 | 5.1.1.3, 5.1.1.4, 5.1.1.5, 5.1.1.6, B.3.1.2, B.3.1.3 | Incorporated CRs: OMA-IOP-2017-0061R01-CR_LPPe_ETS_optional_features OMA-IOP-2017-0064-CR_LPPe_ETS_update_srn_ics OMA-IOP-2017-0065-CR_LPPe_ETS_revise_con_006 |
|  | 22 Aug 2017 | 4.2, 4.3, 5.1.1.2, 5.1.1.4 | Incorporated CR: OMA-IOP-2017-0069-CR_LPPe_ETS_tidy_up |
| Candidate Version OMA-ETS-LPPe-V1_0 | 29 Aug 2017 | n/a | Status changed to Candidate by TP TP ref # OMA-TP-2017-0032-INP_LPPe-V1_0_ETS_for_1st_Candidate_Approval |

Conformance Test Case applicability

Introduction

This section shall help implementers of the LPPe Enabler to select appropriate Conformance test cases that are applicable to the features implemented.

This appendix lists all test cases testing only mandatory features, ICS (Implementation Conformance Specification), IXIT (Implementation eXtra Information) and a mapping from ICS/IXIT to applicable test cases as defined by the Open Mobile Alliance.

Test Cases testing only mandatory features

These Conformance test cases are independent from any applicability, are testing only mandatory SCRs and SHALL be run with every implementation.

Test Case

LPPe-1.0-con-001- Verification of the LPP EPDU-ID and LPPe message header

Applicability

Client ICS

Client general ICS

| ICS | Description | SCR Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |
| ics_Agnss | AGNSS supported in LPPe |  |  |
| ics_Otdoa | OTDOA supported in LPPe |  |  |
| ics_Eotd | EOTD supported in LPPe |  |  |
| ics_Otdoa_Utra | OTDOA UTRA supported in LPPe |  |  |
| ics_Ecid_Lte | ECID LTE supported in LPPe |  |  |
| ics_Ecid_Gsm | ECID GSM supported in LPPe |  |  |
| ics_Ecid_Utra | ECID UTRA supported in LPPe |  |  |
| ics_Wlan_Ap | WLAN AP supported in LPPe |  |  |
| ics_Ecid_Wimax | ECID WIMAX supported in LPPe |  |  |
| ics_Sensor | Sensor supported in LPPe |  |  |
| ics_Srn_Bluetooth | SRN Technology: Bluetooth supported in LPPe |  |  |
| ics_Srn_Bluetooth_Low_Energy | SRN Technology: Bluetooth Low Energy supported in LPPe |  |  |
| ics_Srn_Near_Field_Communications | SRN Technology: Near Field Communications supported in LPPe |  |  |
| ics_Srn_Oma_Mobile_Codes | SRN Technology: OMA Mobile Codes supported in LPPe |  |  |
| ics_Srn_Other | SRN Technology: Other supported in LPPe |  |  |

Client Provide Capabilities ICS

| LPPe Provide Capabilities IE | ICS | Description | SCR Reference(s) | Supported (yes/no/value(s)) |
| --- | --- | --- | --- | --- |
| LPPe-commonIEs | ics_iP-Address-Capabilities | IP address capabilities supported |  |  |
|  | ics_assistanceContainerSupport | Assistance data containers supported |  |  |
|  | ics_locationInformationContainerSupport | Location information containers supported |  |  |
|  | ics_relativeLocationChange-Capabilities | Relative location change capabilities supported |  |  |
|  | ics_highAccuracyFormatCapabilities | High accuracy formats supported |  |  |
|  | ics_segmentedAssistanceData-ProvideCapabs | Segmented assistance data supported |  |  |
|  | ics_referencePointCapabilities | Reference point supported |  |  |
|  | ics_scheduledLocation-Capabilities | Scheduled location supported |  |  |
|  | ics_accessCapabilities | Access capabilities supported |  |  |
|  | ics_segmentedLocationInformation-ProvideCapabs | Segmented location information supported |  |  |
| LPPe-agnss | FFS |  |  |  |
| LPPe-otdoa | FFS |  |  |  |
| LPPe-eotd | FFS |  |  |  |
| LPPe-otdoa-utra | FFS |  |  |  |
| LPPe-ecid-lte | FFS |  |  |  |
| LPPe-ecid-gsm | FFS |  |  |  |
| LPPe-ecid-utra | FFS |  |  |  |
| LPPe-wlan-ap | ics_wlan-ecid-MeasSupported | WLAN e-cid measurements supported |  |  |
|  | ics_wlan-types-Supported | WLAN types supported |  |  |
|  | ics_ap-Capability | AP capabilities supported |  |  |
|  | ics_wlan-ap-ADSupported | WLAN AP assistance data supported |  |  |
|  | ics_additional-wlan-ecid-MeasSupported | Additional WLAN e-cid measurements supported |  |  |
| LPPe-ecid-wimax | FFS |  |  |  |
| LPPe-sensor | ics_motionStateSupport | Motion state measurements supported |  |  |
|  | ics_secondarySupport | Secondary motion state measurements supported |  |  |
|  | ics_atmosphericPressureADSupport | Atmospheric pressure assistance data supported |  |  |
|  | ics_atmosphericPressureSupport | Atmospheric pressure measurements supported |  |  |
| LPPe-srn: SRN-Technologies: bt | FFS |  |  |  |
| LPPe-srn: SRN-Technologies: btle | ics_supportedMeasurements | Measurements supported |  |  |
|  | ics_supportedAssistanceData | Assistance data supported |  |  |
|  | ics_historicMeasurementsSupported | Historic measurements reporting supported |  |  |
| LPPe-srn: SRN-Technologies: nfc | FFS |  |  |  |
| LPPe-srn: SRN-Technologies: mobileCode | FFS |  |  |  |
| LPPe-srn: SRN-Technologies: other | FFS |  |  |  |

Client IXIT

| IXIT | Description | Unit <(Range of values)> | Value |
| --- | --- | --- | --- |
| ixit_immediate_provideCapabilities_message_sent | Client immediately sends an LPP/LPPe Provide Capabilities message when the LPP/LPPe session is started. Note this may depend on the type of session used. | - (Yes/No) |  |
| ixit_emergency_call_required | Emergency call required to be in progress for an Emergency Services Location Request | - (Yes/No) |  |

Server ICS

| ICS | Description | SCR Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |

Server IXIT

| IXIT | Description | Unit <(Range of values)> | Value |
| --- | --- | --- | --- |

ICS to test case mapping

# According to the ICS described above the applicable test cases can be derived from the following table.

| Applicability | Test Cases |
| --- | --- |
| ics_Agnss | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 1: AGNSS LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 1: AGNSS |
| ics_Otdoa | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 2: OTDOA LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 2: OTDOA |
| ics_Eotd | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 3: EOTD LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 3: EOTD |
| ics_Otdoa_Utra | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 4: OTDOA UTRA LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 4: OTDOA UTRA |
| ics_Ecid_Lte | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 5: ECID LTE LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 5: ECID LTE |
| ics_Ecid_Gsm | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 6: ECID GSM LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 6: ECID GSM |
| ics_Ecid_Utra | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 7: ECID UTRA LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 7: ECID UTRA |
| ics_Wlan_Ap | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 8: WLAN AP LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 8: WLAN AP |
| ics_Ecid_Wimax | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 9: ECID WIMAX LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 9: ECID WIMAX |
| ics_Sensor | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 10: Sensor LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 10: Sensor |
| ics_Srn_Bluetooth | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 11a: SRN for SRN Technology: Bluetooth LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 11a: SRN for SRN Technology: Bluetooth |
| ics_Srn_Bluetooth_Low_Energy | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 11b: SRN for SRN Technology: Bluetooth Low Energy LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 11b: SRN for SRN Technology: Bluetooth Low Energy |
| ics_Srn_Near_Field_Communications | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 11c: SRN for SRN Technology: Near Field Communications LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 11c: SRN for SRN Technology: Near Field Communications |
| ics_Srn_Oma_Mobile_Codes | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 11d: SRN for SRN Technology: OMA Mobile Codes LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 11d: SRN for SRN Technology: OMA Mobile Codes |
| ics_Srn_Other | LPPe-1.0-con-004- Basic LPPe support (Capabilities) Test 11e: SRN for SRN Technology: Other LPPe-1.0-con-006- Basic LPPe support (Location Information) Test 11e: SRN for SRN Technology: Other |