| Enabler Test Specification for RCS Conformance |  |  |  |
| --- | --- | --- | --- |
| Candidate Version 5.x – 22 Aug 2017 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-ETS-RCS-CON-V5_x-20170822-C |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2017 Open Mobile Alliance All Rights Reserved.
Used with the permission of the Open Mobile Alliance under the terms set forth above.

Contents

1.	Scope	9

2.	References	10

2.1	Normative References	10

2.2	Informative References	10

3.	Terminology and Conventions	11

3.1	Conventions	11

3.2	Definitions	11

3.3	Abbreviations	11

4.	Introduction	13

5.	RCS Client Conformance Test Cases	14

5.1	General	14

5.2	Configuration	15

5.2.1	RCS-con-001 First-time unsuccessful configuration - Subscriber unauthorized (Auto-Configuration Server)		15

5.2.2	RCS-con-004 Configuration - RCS re-configuration successful (Includes Optional Features)	15

5.3	Keep Alive	16

5.3.1	RCS-con-020 Keep-Alive for SIPoUDP (STUN) (Includes Optional Features)	16

5.4	Mobile Originated Capability Discovery	16

5.4.1	RCS-con-030 Mobile Originated Capability Discovery – Successful	16

5.4.2	RCS-con-031 Mobile Originated Capability Discovery – Unsuccessful – 480 Temporarily Unavailable	16

5.4.3	RCS-con-032 Mobile Originated Capability Discovery – Unsuccessful – 408 Request Timeout	17

5.4.4	RCS-con-033 Mobile Originated Capability Discovery – Unsuccessful – 404 Not Found	17

5.4.5	RCS-con-034 Mobile Originated Capability Discovery – Successful – Multiple Identities	18

5.4.6	RCS-con-035 Mobile Originated Capability Discovery – Successful (IMS registered, does not support 	RCS)	18

5.4.7	RCS-con-036 Mobile Originated Capability Discovery – Send and Receive Social Presence Information to 	non-VIP Contacts	19

5.4.8	RCS-con-037 Mobile Originated Capability Discovery – Social Presence Information including 	Geolocation	19

5.4.9	RCS-con-038 Mobile Originated Capability Discovery – Social Presence Information of a VIP contact 	(Pull)	20

5.5	Mobile Terminated Capability Discovery	21

5.5.1	RCS-con-040 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities	21

5.5.2	RCS-con-041 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS 	Capabilities	21

5.6	Capability Update	22

5.6.1	RCS-con-050 Capability update during a MO call for video /image sharing (Includes Optional Features)	22

5.6.2	RCS-con-051 Capability update during a MT call for video /image sharing	22

5.6.3	RCS-con-052 Capability update during initiation of an MO voice call — fails as other end has no 	capability (Includes Optional Features)	22

5.6.4	RCS-con-053 Capability Exchange Optimization during a Call	23

5.7	Mobile Originated 1-to-1 Chat	24

5.7.1	RCS-con-100 Mobile Originated 1-to-1 Chat — session establishment – Mobile Originated session 	termination	24

5.7.2	RCS-con-101 Mobile Originated 1-to-1 Chat — session establishment – Mobile Terminated session 	termination	24

5.7.3	RCS-con-102 Mobile Originated 1-to-1 Chat – display notification (Includes Optional Features)	25

5.7.4	RCS-con-103 Mobile Originated 1-to-1 Chat – duplicate session	26

5.7.5	RCS-con-104 Mobile Originated 1-to-1 Chat — race condition	27

5.7.6	RCS-con-105 Mobile Originated 1-to-1 Chat – inactivity timeout	28

5.7.7	RCS-con-106 Mobile Originated 1-to-1 Chat — store and forward – sender still in active IM session	29

5.7.8	RCS-con-107 Mobile Originated 1-to-1 Chat — store and forward – sender still online	30

5.7.9	RCS-con-108 Mobile Originated 1-to-1 Chat – Switch up to Group Chat	31

5.7.10	RCS-con-109 Mobile Originated 1-to-1 Chat – Maximum Concurrent Sessions (Includes Optional 	Features)	31

5.8	Mobile Terminated 1-to-1 Chat	33

5.8.1	RCS-con-120 Mobile Terminated 1-to-1 Chat — session establishment – Mobile Terminated session 	termination	33

5.8.2	RCS-con-121 Mobile Terminated 1-to-1 Chat — session establishment – Mobile Originated session 	termination	33

5.8.3	RCS-con-122 Mobile Terminated 1-to-1 Chat – spam/blacklist	34

5.8.4	RCS-con-123 Mobile Terminated 1-to-1 Chat – display notification	34

5.8.5	RCS-con-124 Mobile Terminated 1-to-1 Chat – duplicate session	35

5.8.6	RCS-con-125 Mobile Terminated 1-to-1 Chat — race condition	35

5.8.7	RCS-con-126 Mobile Terminated 1-to-1 Chat — store and forward	36

5.8.8	RCS-con-127 Mobile Terminated 1-to-1 Chat – Switch up to group Chat	36

5.9	Mobile Originated Group Chat	38

5.9.1	RCS-con-130 Mobile Originated Group Chat — establishment – Originating User leaves the session	38

5.9.2	RCS-con-131 Mobile Originated Group Chat — non-originating Users leave the session	38

5.9.3	RCS-con-132 Mobile Originated Group Chat — adding new User to the session	39

5.9.4	RCS-con-135 Mobile Originated Group Chat — closed Group Chat	40

5.10	Mobile Terminated Group Chat	41

5.10.1	RCS-con-140 Mobile Terminated Group Chat — establishment – Terminating User leaves the session	41

5.10.2	RCS-con-141 Mobile Terminated Group Chat – establishment – Terminating User tries to rejoin	41

5.11	Group Chat Store and Forward	43

5.11.1	RCS-con-145 Group Chat Store and Forward – notification for sent messages	43

5.11.2	RCS-con-146 Group Chat Store and Forward – delivery of stored messages	44

5.11.3	RCS-con-147 Group Chat Store and Forward – race condition	44

5.12	Group Chat Common	46

5.12.1	RCS-con-165 Group Chat automatic re-join	46

5.12.2	RCS-con-166 Group Chat re-start: 404 (Not Found)	46

5.12.3	RCS-con-167 Group Chat re-start: 403 (Forbidden)	47

5.12.4	RCS-con-168 Group Chat: Concurrent sessions	48

5.12.5	RCS-con-169 Group Chat: Maximum Concurrent Sessions (Includes Optional Features)	49

5.13	Mobile Originated File transfer	51

5.13.1	RCS-con-200 Mobile Originated File Transfer (Includes Optional Features)	51

5.13.2	RCS-con-201 Mobile Originated File Transfer – Rejected (Includes Optional Features)	51

5.13.3	RCS-con-202 Mobile Originated File Transfer – Store and Forward Sending 1	51

5.13.4	RCS-con-205 Mobile Originated File Transfer – File Transfer within a Group Chat	52

5.13.5	RCS-con-206 Mobile Originated File Transfer – Resume a Send File Transfer	53

5.13.6	RCS-con-209 Mobile Originated File Transfer – Send Audio Message	53

5.13.7	RCS-con-220 Mobile Originated File Transfer in a Group Chat – Blocked file transfer	54

5.14	Mobile Terminated File transfer	55

5.14.1	RCS-con-210 Mobile Terminated File Transfer (Includes Optional Features)	55

5.14.2	RCS-con-211 Mobile Terminated File Transfer – Rejected (Includes Optional Features)	55

5.14.3	RCS-con-212 Mobile Terminated File Transfer – File Size Limit (Includes Optional Features)	55

5.14.4	RCS-con-213 Mobile Terminated File Transfer – File Size Warn Limit (Includes Optional Features)	56

5.14.5	RCS-con-214 Mobile Terminated File Transfer – Store and Forward Receiving 1	57

5.14.6	RCS-con-218 Mobile Terminated File Transfer in a Group Chat – Receive a file	57

5.14.7	RCS-con-219 Mobile Terminated File Transfer in a Group Chat – Ignore to receive a file	58

5.14.8	RCS-con-223 Mobile Terminated File Transfer – Resume receiving a file	58

5.14.9	RCS-con-226 Mobile Terminated File Transfer – Receive Audio Message	59

5.15	Mobile Originated Image sharing	60

5.15.1	RCS-con-250 Mobile Originated Image Sharing – Session Establishment – Mobile Originated Session 	Termination (Includes Optional Features)	60

5.15.2	RCS-con-251 Mobile Originated Image Sharing – Session Establishment – rejected (Includes Optional 	Features)	60

5.16	Mobile Terminated Image sharing	61

5.16.1	RCS-con-270 Mobile Terminated Image Sharing – Session Establishment – Mobile Terminated Session 	Termination (Includes Optional Features)	61

5.16.2	RCS-con-271 Mobile Terminated Image Sharing – Session Establishment – rejected (Includes optional 	Features)	61

5.17	Mobile Originated Video sharing	62

5.17.1	RCS-con-300 Mobile Originated Video Sharing – Session Establishment – Mobile Originated Session 	Termination (Includes Optional Features)	62

5.17.2	RCS-con-301 Mobile Originated Video Sharing – Session Establishment – Mobile Terminated Session 	Termination (Includes Optional Features)	62

5.17.3	RCS-con-302 Mobile Originated Video Sharing – Session Establishment – Rejected (Includes Optional 	Features)	63

5.17.4	RCS-con-303 Mobile Originated Video Sharing – Send Video Sharing outside of a Voice Call	63

5.17.5	RCS-con-304 Mobile Originated Video Sharing – Send Video Sharing outside of a Voice Call - Rejected			64

5.17.6	RCS-con-305 Mobile Originated Video Sharing –Video Sharing outside of a Voice Call not possible	64

5.18	Mobile Terminated Video sharing	65

5.18.1	RCS-con-320 Mobile Terminated Video Sharing – Session Establishment – Mobile Terminated Session 	Termination (Includes Optional Features)	65

5.18.2	RCS-con-321 Mobile Terminated Video Sharing – Session Establishment – Mobile Originated Session 	Termination (Includes Optional features)	65

5.18.3	RCS-con-322 Mobile Terminated Video Sharing – Session Establishment – Rejected (Includes Optional 	Features)	66

5.18.4	RCS-con-323 Mobile Terminated Video Sharing – Receive Video Sharing outside of a Voice Call	66

5.18.5	RCS-con-324 Mobile Terminated Video Sharing – Ignore Video Sharing outside of a Voice Call	67

5.18.6	RCS-con-325 Mobile Terminated Video Sharing – Video Sharing outside of a Voice Call not possible	67

5.19	Mobile Originated Standalone Messaging	68

5.19.1	RCS-con-400 Mobile Originated Pager Mode – Send Text	68

5.19.2	RCS-con-401 Mobile Originated Large Message Mode – Send Multimedia	68

5.19.3	RCS-con-402 Mobile Originated Large Message Mode – Send Long Text (Includes Optional Features)	69

5.20	Mobile Terminated Standalone messaging	70

5.20.1	RCS-con-420 Mobile Terminated Pager Mode – Receive Text	70

5.20.2	RCS-con-421 Mobile Terminated Large Message Mode – Receive Multimedia	70

5.20.3	RCS-con-422 Mobile Terminated Large Message Mode – Receive Long Text (Includes Optional Features)		70

5.20.4	RCS-con-425 Mobile Terminated Pager Mode – No Displayed Notification Sent (Includes Optional 	Features)	71

5.21	Mobile Originated RCS IP Video Call	72

5.21.1	RCS-con-500 Mobile Originated RCS IP Video Call – Send an accepted Call (Duplex Mode)	72

5.21.2	RCS-con-501 Mobile Originated RCS IP Video Call – Send a declined Call	72

5.21.3	RCS-con-502 Mobile Originated RCS IP Video Call (Upgrade from CS voice call) – Send an uni-	directional accepted Call	72

5.21.4	RCS-con-503 Mobile Originated RCS IP Video Call (Upgrade from CS voice call) – Send a bi-directional 	accepted Call	73

5.21.5	RCS-con-504 Mobile Originated RCS IP Video Call (Upgrade from CS voice call) – Switch between a bi-	directional and uni-directional Call	73

5.21.6	RCS-con-505 Mobile Originated RCS IP Video Call – Receive a Decline of an upgrade from CS voice 	call	74

5.21.7	RCS-con-506 Mobile Originated RCS IP Video Call – Send an accepted Call (Simplex Mode)	74

5.22	Mobile Terminated RCS IP Video Call	75

5.22.1	RCS-con-530 Mobile Terminated RCS IP Video Call – Accept a Call (Duplex Mode)	75

5.22.2	RCS-con-531 Mobile Terminated RCS IP Video Call – Decline a Call	75

5.22.3	RCS-con-532 Mobile Terminated RCS IP Video Call (Upgrade from CS voice call) – Accept an uni-	directional Call	75

5.22.4	RCS-con-533 Mobile Terminated RCS IP Video Call (Upgrade from CS voice call) – Accept a bi-	directional Call	76

5.22.5	RCS-con-534 Mobile Terminated RCS IP Video Call – Decline an upgrade from CS voice call	76

5.22.6	RCS-con-535 Mobile Terminated RCS IP Video Call – Accept a Call (Simplex Mode)	77

5.23	Call Establishment and Termination	78

5.23.1	RCS-con-600 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination 	(Includes Optional Features)	78

5.24	Enriched Calling	79

5.24.1	RCS-con-620 Mobile originated Voice Call - Mobile terminated Geolocation Push	79

5.24.2	RCS-con-621 Mobile originated Voice Call – Send Shared Sketch	79

5.24.3	RCS-con-622 Mobile originated Voice Call – Receive Shared Sketch	80

5.24.4	RCS-con-623 Mobile originated Voice Call – MSRP Error during shared sketch	80

5.24.5	RCS-con-624 Mobile originated Voice Call – Send Shared Map	81

5.24.6	RCS-con-625 Mobile originated Voice Call – Receive Shared Map	81

5.24.7	RCS-con-627 Mobile Terminated Voice Call – MSRP error during shared sketch	81

Appendix A.	Change History (Informative)	83

A.1	Approved Version History	83

A.2	Draft/Candidate Version 5.x History	83

Appendix B.	Conformance Test Case applicability	88

B.1	Introduction	88

B.2	Client Test Cases testing only mandatory features	88

B.3	Applicability	89

B.3.1	Client ICS	89

B.3.2	Client IXIT	89

B.3.3	Server ICS	90

B.3.4	Server IXIT	90

B.4	Client ICS to test case mapping	90

Appendix C.	Default Procedures	92

C.1	Capability Exchange	92

C.1.1	Mobile Originated Capability Exchange	92

C.1.2	Mobile Terminated Capability Exchange	92

C.1.3	VOID	92

C.1.4	VOID	92

C.1.5	Mobile Originated Geolocation information	92

C.2	Session Establishment	92

C.2.1	Mobile Originated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat, RCS IP 	Voice Call	92

C.2.1a	 Mobile Originated Session Establishment: Video share (IMS mode)	93

C.2.2	Mobile Terminated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat	93

C.2.3	Mobile Originated Session Establishment: 1-to-1 Chat with display notification	93

C.2.4	Mobile Terminated Session Establishment: 1-to-1 Chat with display notification	93

C.2.5	Mobile Originated Session Establishment: 1-to-1 Chat with two messages	93

C.2.6	Mobile Terminated Session Establishment: 1-to-1 Chat with two messages	93

C.2.7	Mobile Terminated Session Establishment: Spam	94

C.2.8	Mobile Originated Session Establishment: simultaneous INVITEs	94

C.2.9	Mobile Terminated Session Establishment: new INVITE race	94

C.2.10	Mobile Originated Session: Store and Forward – Receiver offline	95

C.2.11	Mobile Originated Session: Store and Forward – deferred delivery	95

C.2.12	Mobile Terminated Session: Store and Forward – deferred delivery	95

C.2.13	Mobile Originated Session Establishment: Group Chat	95

C.2.14	Mobile Terminated Session Establishment: Group Chat	95

C.2.15	Group Chat automatic re-join	96

C.2.16	Group Chat re-start: 404 Not Found	96

C.2.17	Group Chat re-start: 403 Forbidden	96

C.2.18	Group Chat auto-start	96

C.2.19	Group Chat concurrent re-start: Mobile Terminated	96

C.2.20	Auto-accept Group Chat	96

C.2.21	Auto-invite Group Chat	96

C.2.22	Group Chat Session Tear Down	96

C.2.23	Group Chat re-start: Mobile Terminated	97

C.2.24	Mobile Originated Session Establishment: Video share, Image share, File share, Group Chat	97

C.2.25	Mobile Terminated File Transfer – Store and Forward Receiving	97

C.2.26	Mobile Originated Session Establishment: RCS IP Video Call (Duplex Mode)	97

C.2.27	Mobile Originated Session Establishment: RCS IP Video Call (Simplex Mode)	97

C.2.28	Mobile Terminated Session Establishment: RCS IP Video Call (Duplex Mode)	97

C.2.29	Mobile Terminated Session Establishment: RCS IP Video Call (Simplex Mode)	98

C.2.30	Mobile Originated Session File Transfer BlockedGroup Chat	98

C.3	Session Termination	98

C.3.1	Mobile Originated Session Termination: Video share, Image share, File share, 1-to-1 sessions, RCS IP 	Voice Call	98

C.3.2	Mobile Terminated Session Termination: Video share, Image share, File share, 1-to-1 sessions	98

C.3.3	Mobile Originated Session Termination: Group chat	98

C.3.4	Mobile Terminated Session Termination: Group chat	98

C.3.5	Mobile Originated Session Termination: Video share, Image share, File share, Group sessions	98

C.3.6	Mobile Originated Session Termination: File Transfer Store and Forward	99

C.4	Session Rejection	99

C.4.1	Mobile Originated Session — Reject	99

C.4.2	Mobile Terminated Session — Reject	99

C.4.3	Mobile Originated RCS IP Video Call Session – Rejected	99

C.4.4	Mobile Terminated RCS IP Video Call Session – Rejected	99

C.4.5	Mobile Originated IP Video Call Upgrade from CS voice call decline	99

C.4.6	Mobile Terminated IP Video Call Upgrade from CS voice call decline	99

C.5	Other Procedures	100

C.5.1	Participant List Update	100

C.5.2	Inviting new User to Group Chat	100

C.5.3	Get Participant List	100

C.5.4	Inviting new User to Closed Group Chat	100

C.5.5	Mobile Originated Pager Mode Message	100

C.5.6	Mobile Terminated Pager Mode Message	100

C.5.7	Mobile Originated Delivered Notification	100

C.5.8	Mobile Terminated Delivered Notification	100

C.5.9	Mobile Originated Displayed Notification	100

C.5.10	Mobile Terminated Displayed Notification	101

C.5.11	Mobile Originated Pager Mode Procedure	101

C.5.12	Mobile Terminated Pager Mode Procedure	101

C.5.13	Mobile Terminated Geolocation Push	101

C.5.14	Mobile Terminated Shared Sketch	101

C.5.15	Mobile Originated Shared Sketch/Map	101

C.5.16	Mobile Terminated Shared Map	101

C.6	Autoconfiguration Procedures	102

C.6.1	HTTP	102

C.7	MSRP session	102

C.7.1	MSRP Session Message	102

C.7.2	MSRP Session Message with Display Notification	102

C.7.3	MSRP Session Store and Forward	102

C.7.4	MSRP Session Store and Forward Message Delivered	103

C.7.5	MSRP Session Store and Forward Final Message Delivered	103

C.7.6	MSRP Group Session Message	103

C.7.7	MSRP Session Store and Forward Message Reception	103

C.7.8	MSRP Session Store and Forward File Transfer	103

C.7.9	MSRP Session Store and Forward File Transfer Completed	103

C.7.10	Store and Forward File Transfer Delivered Notification	103

C.8	PRESENCE based Capability Exchange	104

C.8.1	Publishing	104

C.8.2	Subscription to capabilities	104

Appendix D.	Default Message Content	105

D.1	SIP message content	105

D.1.1	INVITE	106

D.1.2	200 OK for INVITE	108

D.1.3	OPTIONS	109

D.1.4	200 OK for OPTIONS	109

D.1.5	404 Not found	111

D.1.6	408 Request Timeout	111

D.1.7	480 Temporarily Unavailable	112

D.1.8	403 Forbidden	112

D.1.9	486 BUSY HERE	113

D.1.10	SUBSCRIBE (conference)	113

D.1.11	NOTIFY (conference)	113

D.1.12	BYE	115

D.1.13	MESSAGE	115

D.1.14	200 OK (Generic)	116

D.1.15	REFER	116

D.1.16	202 ACCEPTED	116

D.1.17	PUBLISH	117

D.1.18	SUBSCRIBE (presence)	118

D.1.19	NOTIFY (presence)	118

D.2	HTTP autoconfiguration message content	119

D.2.1	Initial HTTP request	119

D.2.2	200 OK for the initial HTTP request	119

D.2.3	Initial HTTPS request	119

D.2.4	200 OK for the initial HTTPS request	119

D.2.5	Default provisioning XML [RCS 1.2.2]	120

D.3	MSRP message content	122

D.3.1	MSRP SEND	122

D.3.2	MSRP 200 OK for MSRP SEND message	124

Appendix E.	Mapping to RCS profiles	125

Figures

No table of figures entries found.

Tables

Table 1	89

Table 2	89

Table 3	90

Table 4	90

Table 5	90

# Scope

This document describes in detail the conformance test cases for the deployment suite of RCS 5.1, RCS 5.2 and RCS 5.3 as described in references [RCS 5.1], [RCS 5.2] and [RCS 5.3] together with the RCS Implementation Guidelines [RIG 5.1], [RIG 5.2] and [RIG 5.3].

The test cases are split into two categories, conformance and interoperability test cases. The interoperability test cases are defined in a separate interoperability ETS.

The conformance test cases are aimed to verify the adherence to the requirements described in the technical specifications.

Some of the test cases are also applicable for previous versions of RCS. Appendix E specifies the applicability of each conformance test for previous RCS profiles.

This document was updated to reflect the latest changes in the RCS Ecosystem in October, 2018.

# References

## Normative References

| [OMA_IM_TS] | “Instant Messaging Requirements”, Version 1.0, Open Mobile Alliance™, OMA-RD-IM-V1_0, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [OMA_IM_XDM] | “IM XDM Specification”, Version 1.0, Open Mobile Alliance™, OMA-TS-IM_XDM-V1_0, URL:http://www.openmobilealliance.org/ |
| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |

## Informative References

| [OMADICT] |  | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- |
| [RCS 1.2.2] |  | “RCS-e – Advanced Communications: Services and Client Specification”, v1.2.2, July 2012, GSM Association, RCS_e_Specification_Document_1_2_2, URL:http://www.gsma.com/rcs/wp-content/uploads/2012/03/rcs-e_advanced_comms_specification_v1_2_2_approved.pdf |  |
| [RCS 5.1] |  | “Rich Communication Suite 5.1 Advanced Communications Services and Client Specification". Version 4.0, 28 November 2013. URL:http://www.gsma.com/network2020/rcs/specs-and-product-docs/ |  |
| [RCS 5.2] |  | “Rich Communication Suite 5.2 Advanced Communications Services and Client Specification”. Version 5.0, 07 May 2014. URL:http://www.gsma.com/network2020/rcs/specs-and-product-docs/ |  |
| [RCS 5.3] |  | “Rich Communication Suite 5.3 Advanced Communications Services and Client Specification”. Version 6.0, 28 February 2015. URL:http://www.gsma.com/network2020/rcs/specs-and-product-docs/ |  |
| [RIG 1.2.2] |  | “GSMA RCS IOT RCS-e Implementation Guidelines”, Version 3.2, 10 December 2012, URL:http://www.gsma.com/rcs/wp-content/uploads/2012/12/RCS-e_Implementation_guidelines_v3_2_clean.pdf |  |
| [RIG 5.1] |  | "GSMA RCS IOT joyn Blackbird Implementation Guidelines". Version 1.5. 18 February 2014, URL:http://www.gsma.com/network2020/rcs/specs-and-product-docs/ |  |
| [RIG 5.2] |  | “GSMA RCS IOT joyn Blackbird Implementation Guidelines”, Version 1.3, 15 May 2014, URL:http://www.gsma.com/network2020/rcs/specs-and-product-docs/ |  |
| [RIG 5.3] |  | “GSMA RCS IOT joyn Crane Implementation Guidelines”, Version 1.5, 18 February 2015, URL:http://www.gsma.com/network2020/rcs/specs-and-product-docs/ |  |
| [VSI] |  | “GSMA Video Share Service Definition”, Version 1.4, 20 December 2010, URL:http://www.gsma.com/newsroom/ir-74-1-4-video-share-interoperability-specification |  |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope”, are normative, unless they are explicitly indicated to be informative.

The following numbering scheme is used:

xxx-y.z-con-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’con’	Indicating this test is a conformance test case
number	Leap number for the test case

Or

xxx-y.z-int-number where:
xxx	Name of enabler, e.g. MMS or Browsing
y.z	Version of enabler release, e.g. 1.2 or 1.2.1
’int’	Indicating this test is a interoperability test case
number	Leap number for the test case

## Definitions

| Auto-Configuration Service | A component of RCS functionality that provides an interoperable way to provision RCS configuration information from a Service Provider to an RCS Client. |
| --- | --- |
| Client | Uses definition from [OMADICT]. |
| Component | Uses definition from [OMADICT]. |
| IMS Mode | A mode of SIP signalling used for SIP session setup, as described in [VSI], section 3.4 |
| RCS Client | A Client which adheres and conforms to GSMA RCS specifications and is capable to receive RCS service. |
| RCS Group Chat ID (Group Chat ID) | A globally unique identifier that identifies a Group Chat and that is created when the group chat is first started and preserved over Group Chat restarts. The Group Chat ID is transported as the Contribution-ID header field in a SIMPLE-IM based communication and as the Conversation-ID header field when the communication is CPM based [RCS 5.1] |
| Service | Uses definition from [OMADICT]. |
| Service Provider | Uses definition from [OMADICT]. |

## Abbreviations

| AMR | Adaptive Multi-Rate |
| --- | --- |
| BPEF | Blacklist Enforcement Function |
| CPIM | Common Profile for Instant Messaging |
| CPR | Crane Priority Release |
| CS | Circuit Switch |
| FTF | File Transfer Function |
| HTTP | Hyper-Text Transfer Protocol |
| IMDN | Instant Message Disposition Notification |
| IMS | IP Media Service |
| MSRP | Message Session Relay Protocol |
| RCS | Rich Communications Suite |
| SDP | Session Description Protocol |
| SIM | Subscriber Identity Module |
| SIP | Session Initiation Protocol |
| SPI | Social Presence Information |
| STUN | Simple Traversal of UDP through NATs |
| USIM | Universal Subscriber Identity Module |
| UX | User eXperience |
| XDMS | XML Document Management Server |
| XML | EXtensible Markup Languange |

# Introduction

The purpose of this document is to provide conformance test cases for RCS versions 5.1, 5.2 and 5.3.

Some of the test cases are also applicable for previous versions of RCS.

Where tests, test steps, procedures etc. are only applicable to a certain version of RCS they are marked accordingly, for example with [RCS 5.1] etc.

The implementation of some features is optional for the Clients and/or the Servers in the RCS Enabler. The tests associated with these optional features are marked as "(Includes Optional Features)" in the test specification.

# RCS Client Conformance Test Cases

## General

In this section, the following conventions apply:

- Client A is the client on the device-under-test and User A is the notional user of the device.

- All other Users and Clients are simulated in the Test Tool.

- When User A triggers an action or sends or receives a message, file, image, video etc. this shall be performed or verified either on the UI of the device-under-test or via some test-automation interface to the device.

The common procedures used by some test cases are defined in Appendix C.

The message content for most messages is defined in Appendix D.

For tests requiring “video share”, video from any of the following, as defined in [RCS 1.2.2] section 3.3, may be used:

- The front camera (“me”)

- The rear camera (“what I see”)

- A file (“video streaming”)

For tests requiring “image share”, image from any of the following, as defined in [RCS 1.2.2] section 3.3, may be used:

- A picture taken using the front camera (“me”)

- A picture taken using the rear camera (“what I see”)

- A file (“send stored image”)

## Configuration

### RCS-con-001 First-time unsuccessful configuration - Subscriber unauthorized (Auto-Configuration Server)

| Test Case Id | RCS-con-001 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | First-time unsuccessful configuration: Subscriber unauthorized |
| Specification Reference | [RCS 1.2.2]: RCS 2.2.2.1.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A NOT IMS registered for RCS |
| Test Procedure | - Handset is powered on. - Execute "Successful autoconfiguration" (C.6.1.1). In step 4 set the version in the XML body to "0". - Reboot the handset. - Execute "Successful autoconfiguration" (C.6.1.1). - User A‘s RCS client executes the IMS registration successful. |
| Pass-Criteria | - After step 2 service is not enabled on the handset. - At step 4 of the test procedure in step 3 of C.6.1.1 "vers" parameter SHALL be 0. - After step 4 service is enabled on the handset. - After step 5 User’s RCS client is RCS registered. |

### RCS-con-004 Configuration - RCS re-configuration successful (Includes Optional Features)

| Test Case Id | RCS-con-004 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Successful re-configuration: RCS configuration enableRcseSwitch first set to ‘true’ and then set to ‘false’ |
| Specification Reference | [RCS 1.2.2]: RCS 2.10 (and A.2.6) |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | RCS services previously configured on the phone/(U)SIM pair |
| Test Procedure | - User A’s handset is powered on. - Execute "Successful autoconfiguration" (C.6.1.1). In step 4 set the enableRcseSwitch to true in the XML body. - User A’s RCS client performs IMS registration. - User A’s handset is power cycled. - Execute “Successful autoconfiguration” (C.6.1.1). In step 4 set the enableRcseSwitch to false in the XML body. - User A’s RCS client performs IMS registration. |
| Pass-Criteria | - After step 3 User A’s handset, RCS UX provides the following control elements: ‘RCS Service while roaming’ enabled/disabled switch; and ‘RCS Service’ enabled/disabled switch. - After step 6 User A’s handset, RCS UX does not provide the following control element: ‘RCS Service’ enabled/disabled switch. |

## Keep Alive

### RCS-con-020 Keep-Alive for SIPoUDP (STUN) (Includes Optional Features)

| Test Case Id | RCS-con-020 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Keep-alive for SIPoUDP (STUN) |
| Specification Reference | [RCS 1.2.2]: RCS 2.8 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | The client A is a registered RCS user. The client A’s handset coverage is Wi-Fi. - Applicability: ics_stun_udp |
| Test Procedure | - The client A exchanges initial STUN message. - The client A’s handset registers for the RCS service. - The client A’s constantly sends keep-alive message. |
| Pass-Criteria | - During step 3, the test tool receives STUN keep-alive messages from the client A. |

## Mobile Originated Capability Discovery

### RCS-con-030 Mobile Originated Capability Discovery – Successful

| Test Case Id | RCS-con-030 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Capability Discovery - Successful |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS |
| Test Procedure | - User A triggers a capability discovery procedure towards User B at User A’s device. This can be done by one of the following: a. Adding new User B entry (RCS user) to User A’s address book; or b. Refreshing User B’s status (RCS user) manually - Execute "Mobile Originated Capability Exchange" (C.1.1). - Check that User B is shown as available for RCS on User A’s RCS client. |
| Pass-Criteria | - At step 3 User A’s RCS client shows User B as available. |

### RCS-con-031 Mobile Originated Capability Discovery – Unsuccessful – 480 Temporarily Unavailable

| Test Case Id | RCS-con-031 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Capability Discovery – Unsuccessful – 480 Temporarily Unavailable |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS |
| Test Procedure | - User A triggers a capability discovery procedure towards User B on User A’s device. This can be done by one of the following: a. Adding new User B entry (RCS user) to User A’s address book; or b. Refreshing User B’s (RCS user) status manually - Execute step 1 of "Mobile Originated Capability Exchange" (C.1.1). - The test tool responds with a 480 TEMPORARILY UNAVAILABLE (D.1.7) towards User A’s RCS client. - Check that User B is shown as not available for RCS on User A’s RCS Client. |
| Pass-Criteria | - At step 4 User A’s RCS client shows User B as not available. |

### RCS-con-032 Mobile Originated Capability Discovery – Unsuccessful – 408 Request Timeout

| Test Case Id | RCS-con-032 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Capability Discovery – Unsuccessful – 408 Request Timeout |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS |
| Test Procedure | - User A triggers a capability discovery procedure towards User B on User A’s device. This can be done by one of the following: a. Adding new User B entry (RCS user) to User A’s address book; or b. Refreshing User B’s (RCS user) status manually - Execute step 1 of "Mobile Originated Capability Exchange" (C.1.1). - The test tool responds with a 408 REQUEST TIMEOUT (D.1.6) towards the RCS client. - Check that User B is shown as not available for RCS on User A’s RCS client. |
| Pass-Criteria | - At step 4 User A’s RCS client shows User B as not available. |

### RCS-con-033 Mobile Originated Capability Discovery – Unsuccessful – 404 Not Found

| Test Case Id | RCS-con-033 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Capability discovery – Unsuccessful – 404 Not Found |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS |
| Test Procedure | - User A triggers a capability discovery procedure towards User B on User A’s device. This can be done by one of the following: a. Adding new User B entry (RCS user) to User A’s address book; or b. Refreshing User B’s (RCS user) status manually - Execute step 1 of "Mobile Originated Capability Exchange" (C.1.1). - The test tool responds with a 404 NOT FOUND (D.1.5) towards User A’s RCS client. - Check that User B is shown as not available for RCS on User A’s RCS client. |
| Pass-Criteria | - At step 4 User A’s RCS client shows User B as not available. |

### RCS-con-034 Mobile Originated Capability Discovery – Successful – Multiple Identities

| Test Case Id | RCS-con-034 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Capability Discovery – Successful – Multiple Identities |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS |
| Test Procedure | - User A creates a new contact (User C) in address book with the following four unique entries - MSISDN number 1 (RCS capable) - MSISDN number 2 (without RCS capabilities) - MSISDN number 3 (RCS capable, different from 1) - SIP-URI (RCS capable, if supported by the UI, independent from 1, 2 or 3). - Execute step 1 of "Mobile Originated Capability Exchange" (C.1.1) for all four entries (three if SIP-URI was not supported). - The test tool responds with 200 OK towards User A’s RCS client for each OPTIONS message. - Check that User C’s identities 1, 3 and 4 (if supported) are shown as available for RCS on User A’s RCS client. |
| Pass-Criteria | - At step 4 User A’s RCS client shows User C’s identities 1, 3 and 4 (if supported) as available. |

### RCS-con-035 Mobile Originated Capability Discovery – Successful (IMS registered, does not support RCS)

| Test Case Id | RCS-con-035 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Capability Discovery - Successful |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS User B is IMS registered, but does not support RCS |
| Test Procedure | - User A triggers a capability discovery procedure towards User B at User A’s device. This can be done by one of the following: a. Adding new User B entry (RCS user) to User A’s address book; or b. Refreshing User B’s (RCS user) status manually - Execute "Mobile Originated Capability Exchange" (C.1.1) with exception to not sending feature param tag in the contact header of 200 OK. - Check that User B is shown as unavailable for RCS on User A’s RCS client |
| Pass-Criteria | - At step 3 User A’s RCS client shows User B as unavailable. |

### Void

### Void

### Void

## Mobile Terminated Capability Discovery

### RCS-con-040 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities

| Test Case Id | RCS-con-040 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities |
| Specification Reference | [RCS 1.2.2]: RCS 2.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS User B is stored in User A’s address book |
| Test Procedure | - Execute “Mobile Terminated Capability Exchange” (C.1.2). - Check that User B is shown as available for RCS on User A’s RCS client. |
| Pass-Criteria | - At step 2 User A’s RCS client shows User B as available for RCS. |

### RCS-con-041 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS Capabilities

| Test Case Id | RCS-con-041 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated Capability Discovery – Learning Unknown New Users RCS Capabilities |
| Specification Reference | [RCS 1.2.2]: RCS 2.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s and User X’s RCS devices are provisioned for RCS User A and User X are IMS registered for RCS User X is not currently stored in User A’s address book |
| Test Procedure | - Execute “Mobile Terminated Capability Exchange” (C.1.2) with User X as sender and condition B4 in D.1.3. - Execute “Mobile Terminated Session Establishment” (C.2.2) with the message “Hello A” from User X. - Execute “Mobile Terminated Session Termination” (C.3.2). - Store User X contact in the address book. |
| Pass-Criteria | - In Step 2 User A receives “Hello A” from User X. - In step 4 verify that User X capabilities are visible on User A’s terminal when the contact is stored in the address book. |

## Capability Update

### RCS-con-050 Capability update during a MO call for image sharing (Includes Optional Features)

| Test Case Id | RCS-con-050 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | When a voice call is established the participants have to update their capabilities |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS The client A’s handset coverage is 3G - Applicability: - ics_imageShare |
| Test Procedure | - User A initiates a CS voice call to User B at User A’s device. - Execute “Mobile Originated Capability Exchange” (C.1.1). - Check that User B is shown as available for image sharing on User A’s RCS client as supported by User A’s RCS client. |
| Pass-Criteria | - At step 3 User A’s RCS client shows User B as available for image sharing as matching User A’s RCS client’s capabilities. |

### RCS-con-051 Capability update during a MT call for image sharing

| Test Case Id | RCS-con-051 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | When a voice call is established the participants have to update their capabilities |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A is IMS registered for RCS User A’s RCS device is provisioned for RCS The client A’s handset coverage is 3G |
| Test Procedure | - User A accepts a CS voice call from User B at User A’s device. - Execute “Mobile Terminated Capability Exchange” (C.1.2). - Check that User B’s capabilities for image sharing are shown as available on User A’s RCS client matching User A’s RCS client’s capabilities according to ICS. |
| Pass-Criteria | - At step 3 User B’s capabilities for image sharing are shown as available on User A’s RCS client matching User A’s RCS client’s capabilities according to ICS. |

### RCS-con-052 Capability update during initiation of an MO voice call — fails as other end has no capability (Includes Optional Features)

| Test Case Id | RCS-con-052 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | When a voice call is initiated the call participants have to update their capabilities. The recipient should not be shown as available for image share if the corresponding tags are not included in the 200 OK response |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | The client A’s handset coverage is 3G User A is IMS registered for RCS User A’s RCS device is provisioned for RCS - Applicability: - ics_imageShare |
| Test Procedure | - User A initiates a CS voice call to User B at User A’s device. - Execute “Mobile Originated Capability Exchange” (C.1.1) and do not use condition B1 and B2 in step 2 (D.1.4). - Check that User B is shown as not available for image sharing on User A’s RCS client. |
| Pass-Criteria | - At step 3 User A’s RCS client shows User B as not available for image sharing. |

### RCS-con-053 Capability Exchange Optimization during a Call

| Test Case Id | RCS-con-053 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Capability exchange optimization during a call |
| Specification Reference | [RCS 1.2.2]: RCS 2.3.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | The client A’s handset coverage is 3G. The client A is RCS registered user. The client A’s handset is in an on-going CS voice call with a test tool (simulating an RCS user) and the test tool delivers its RCS capabilities to the client A during the execution of “Mobile Originated Capability Exchange” (C.1.1). The displayed capabilities on the client A’s handset for the current call include image share. |
| Test Procedure | - The client A’s handset’s coverage is changed to HSPA. - Wait for 15 seconds. - The client A’s handset’s coverage is changed to 3G. - Wait for 15 seconds. |
| Pass-Criteria | - Client A is not sending SIP OPTIONS message at step #2 and step #4. |

## Mobile Originated 1-to-1 Chat

### RCS-con-100 Mobile Originated 1-to-1 Chat — session establishment – Mobile Originated session termination

| Test Case Id | RCS-con-100 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated 1-to-1 Chat session establishment with Mobile Originated session termination |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. - Execute “Mobile Originated Session Establishment” (C.2.1) with the message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - If the session invitation in step 2 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Hello B”. - Execute “MSRP Session Message” (C.7.1) with the message “Hello A” from the Test Tool. - User A receives an “Is Composing” indication from User B. - User A receives “Hello A” from User B. - User A sends the message “Bye B” to User B. - Execute “MSRP Session Message” (C.7.1) with the message “Bye B” from client A. - User A receives an indication that the message “Bye B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. - User A terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 3 User A receives an indication that the message “Hello B” has been delivered. - At step 4 User A receives an indication that message “Hello B” has been displayed. - At step 6 User A receives an “Is Composing” indication from User B. - At step 7 User A receives “Hello A” from User B. - At step 10 User A receives an indication that the message “Bye B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |

### RCS-con-101 Mobile Originated 1-to-1 Chat — session establishment – Mobile Terminated session termination

| Test Case Id | RCS-con-101 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated 1-to-1 Chat session establishment with Mobile Terminated session termination |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. - Execute “Mobile Originated Session Establishment” (C.2.1) with the message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - If the session invitation in step 2 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Hello B”. - Execute “MSRP Session Message” (C.7.1) with the message “Hello A” from the Test Tool. - User A receives an “Is Composing” indication from User B. - User A receives “Hello A” from User B. - User A sends the message “Bye B” to User B. - Execute “MSRP Session Message” (C.7.1) with the message “Bye B” from client A. - User A receives an indication that the message “Bye B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 3 User A receives an indication that the message “Hello B” has been delivered. - At step 4 User A receives an indication that message “Hello B” has been displayed. - At step 6 User A receives an “Is Composing” indication from User B. - At step 7 User A receives “Hello A” from User B. - At step 10 User A receives an indication that the message “Bye B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |

### RCS-con-102 Mobile Originated 1-to-1 Chat – display notification (Includes Optional Features)

| Test Case Id | RCS-con-102 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies display notification in Mobile Originated 1-to-1 Chat session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.2.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS User A’s device is configured to request display notifications - Applicability: - ics_request_displayNotifications |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. - Execute “Mobile Originated Session: 1-to-1 Chat with display notification” (C.2.3) with the message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - User A receives an indication that the message “Hello B” has been displayed. - Execute “MSRP Session Message” (C.7.1) with the message “Hello A” from the Test Tool. - User A receives “Hello A” from User B. - User A sends the message “Bye B” to User B. - Execute “MSRP Session Message with Display Notification” (C.7.2) with the message “Bye B” from client A. - User A receives an indication that the message “Bye B” has been delivered. - User A receives an indication that the message “Bye B” has been displayed. - User A terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 3 User A receives an indication that the message “Hello B” has been delivered. - At step 4 User A receives an indication that the message “Hello B” has been displayed. - At step 9 User A receives an indication that the message “Bye B” has been delivered. - At step 10 User A receives an indication that the message “Bye B” has been displayed. |

### RCS-con-103 Mobile Originated 1-to-1 Chat – duplicate session

| Test Case Id | RCS-con-103 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies a duplicate session in a Mobile Originated 1-to-1 Chat session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the first message “Hello B”. - Execute “Mobile Originated Session Establishment — 1-to-1 Chat with two messages” (C.2.5) with the first message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - User A sends the second message “Wake up B” to User B. - Continue to execute “Mobile Originated Session Establishment — 1-to-1 Chat with two messages” (C.2.5) with the second message “Wake up B”. - User A receives an indication that the message “Wake up B” has been delivered. - If the session invitation in step 2 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Hello B”. - If the session invitation in step 5 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Wake up B”. - Execute “MSRP Session Message” (C.7.1) with the message “Hello A” from the Test Tool. - User A receives “Hello A” from User B. - User A terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 3 User A receives an indication that the message “Hello B” has been delivered. - At step 6 User A receives an indication that the message “Wake up B” has been delivered. - At step 7 User A receives an indication that message “Hello B” has been displayed. - At step 8 User A receives an indication that message “Wake up B” has been displayed. - At step 10 User A receives “Hello A” from User B. |

### RCS-con-104 Mobile Originated 1-to-1 Chat — race condition

| Test Case Id | RCS-con-104 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated 1-to-1 Chat session race condition with two simultaneous invites |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.18.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. - Execute “Mobile Originated Session Establishment: simultaneous INVITEs” (C.2.8) with the messages “Hello B” from User A and “Hello A” from User B. - User A receives “Hello A” from User B. - User A receives an indication that the message “Hello B” has been delivered. - If the session invitation in step 2 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Hello B”. - User A sends the message “Quick B” to User B. - Execute “MSRP Session Message” (C.7.1) with the message “Quick B” from client A. - Execute “MSRP Session Message” (C.7.1) with the message “Yes A” from the Test Tool. - User A receives “Yes A” from User B. - User A receives an indication that the message “Quick B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. - User A terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 3 User A receives “Hello A” from User B. - At step 4 User A receives an indication that the message “Hello B” has been delivered. - At step 5 User A receives an indication that message “Hello B” has been displayed. - At step 9 User A receives “Yes A” from User B. - At step 10 User A receives an indication that the message “Quick B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |

### RCS-con-105 Mobile Originated 1-to-1 Chat – inactivity timeout

| Test Case Id | RCS-con-105 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies 1-to-1 Chat inactivity timeout |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.8 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. - Execute “Mobile Originated Session Establishment” (C.2.1) with the message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - If the session invitation in step 2 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Hello B”. - Execute “MSRP Session Message” (C.7.1) with the message “Hello A” from the Test Tool. - User A receives “Hello A” from User B. - User A waits until the chat inactivity (ixit_chat_inactivity_timeout) timeout occurs and Client A then terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 8 Client A terminates the session. |

### RCS-con-106 Mobile Originated 1-to-1 Chat — store and forward – sender still in active IM session - FFS

| Test Case Id | RCS-con-106 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated 1-to-1 Chat session with store and forward when the sender is still in active IM session (with MSRP session still active) when the recipient comes back online. |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.11 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on. |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. (User B is offline) - Execute “Mobile Originated Session: Store and Forward — Receiver offline” (C.2.10) with the message “Hello B”. - User A sends the message “Again B” to User B. (User B is offline) - Execute “MSRP Session Store and Forward” (C.7.3) with the message “Again B” from Client A. - (User A keeps chat session open and User B comes back online) - Execute “MSRP Session Store and Forward Message Delivered” (C.7.4) for the message “Hello B” from Client A. - User A receives an indication that the message “Hello B” has been delivered. - If the MSRP SEND request in step 4 contains a request for display notification, then User A receives an indication that the message “Hello B” has been displayed. - Execute “MSRP Session Store and Forward Final Message Delivered” (C.7.5) for the message “Again B” from Client A. - User A receives an indication that the message “Again B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |
| Pass-Criteria | - At step 7 User A receives an indication that the message “Hello B” has been delivered. - At step 8 User A receives an indication that message “Hello B” has been displayed. - At step 10 User A receives an indication that the message “Again B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |

### RCS-con-107 Mobile Originated 1-to-1 Chat — store and forward – sender still online - FFS

| Test Case Id | RCS-con-107 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated 1-to-1 Chat session with store and forward when the sender is still online when the recipient comes back online. The same call-flow is also used for delivery of deferred notifications when the sender goes offline and then comes back on line later. |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.11 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. (User B is offline) - Execute “Mobile Originated Session Store and Forward — Receiver offline” (C.2.10) with the message “Hello B”. - User A sends the message “Again B” to User B. (User B is offline) - Execute “MSRP Session Store and Forward” (C.7.3) with the message “Again B” from Client A. - User A terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). - (User A stays online and User B comes back online) - Execute “Mobile Originated Session Store and Forward — deferred delivery” (C.2.11). - Execute “MSRP Session Store and Forward Message Delivered” (C.7.4) for the message “Hello B” from Client A. - Client A auto-accepts and User A receives an indication that the message “Hello B” has been delivered. - If the MSRP SEND request in step 4 contains a request for display notification, then User A receives an indication that the message “Hello B” has been displayed. - Execute “MSRP Session Store and Forward Final Message Delivered” (C.7.5) for the message “Again B” from Client A. - User A receives an indication that the message “Again B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |
| Pass-Criteria | - At step 10 User A receives an indication that the message “Hello B” has been delivered. - At step 11 User A receives an indication that message “Hello B” has been displayed. - At step 13 User A receives an indication that the message “Again B” has been delivered and if display notifications were requested, then an indication that the message has been displayed. |

### RCS-con-108 Mobile Originated 1-to-1 Chat – Switch up to Group Chat

| Test Case Id | RCS-con-108 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated 1-to-1 Chat – Switch up to group Chat |
| Specification Reference | [RCS 5.x]: RCS 3.4.6.1.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS configuration parameter CONFFCTY-URI is configured - SIP URI is available and not a dummy |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B (Test Tool) on his device with the message “Hello B”. - Execute “Mobile Originated Session Establishment” (C.2.1) with the message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - If the session invitation in step 2 contained a request for display notification, then execute MSRP SEND request towards A that contains display status for message “Hello B”. - Execute “MSRP Session Message” (C.7.1) with the message “Hello A” from the Test Tool. - User A receives “Hello A” from User B. - User A invites User C (Test Tool) to join the 1-to-1 Chat session. - After User B accepts the switch to the group chat. - User C accepts the invitation as well. |
| Pass-Criteria | - At Step 7 User A sends a SIP-Invite to the MSRP. - At Step 8 User B (Test Tool) accepts and sends a SIP-200-OK which the MSRP also sends to User A. - At Step 9 User A receives a SIP BYE of the original session and sends out a 200 OK to MSRP. Group Chat is established with a new RCS Group Chat ID if the group session hasn’t already existed before. |

### RCS-con-109 Mobile Originated 1-to-1 Chat – Maximum Concurrent Sessions (Includes Optional Features)

| Test Case Id | RCS-con-109 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated 1-to-1 Chat – Maximum Concurrent Sessions |
| Specification Reference | [RCS 5.x]: RCS 3.3.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A, B and C’s RCS device is provisioned for RCS User A, B and C are IMS registered for RCS RCS Provisioning Parameter MAX CONCURRENT SESSION is set to 1 |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. - Execute “Mobile Originated Session Establishment” (C.2.1) with the message “Hello B”. - User A receives an indication that the message “Hello B” has been delivered. - User A keep the chat to User B open and initiates a new 1-to-1 Chat session with User C on his device with the message “Hello C”. |
| Pass-Criteria | - At step 4 the chat with User B is torn down and the message to User C is sent. |

## Mobile Terminated 1-to-1 Chat

### RCS-con-120 Mobile Terminated 1-to-1 Chat — session establishment – Mobile Terminated session termination

| Test Case Id | RCS-con-120 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Terminated 1-to-1 Chat session establishment with Mobile Terminated session termination |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - Execute “Mobile Terminated Session Establishment” (C.2.2) with the message “Hello A” from User B. - User A receives “Hello A” from User B. - User A sends the message “Hello B” to User B before the session times out. - Execute “MSRP Session Message” (C.7.1) with the message “Hello B” from client A. - Execute “MSRP Session Message” (C.7.1) with the message “Bye A” from the Test Tool. - User A receives an “Is Composing” indication from User B. - User A receives “Bye A” from User B. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 2 User A receives “Hello A” from User B. - At step 6 User A receives an “Is Composing” indication from User B. - At step 7 User A receives “Bye A” from User B. |

### RCS-con-121 Mobile Terminated 1-to-1 Chat — session establishment – Mobile Originated session termination

| Test Case Id | RCS-con-121 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Terminated 1-to-1 Chat session establishment with Mobile Originated session termination |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - Execute “Mobile Terminated Session Establishment” (C.2.2) with the message “Hello A” from User B. - User A receives “Hello A” from User B. - User A sends the message “Hello B” to User B before the session times out. - Execute “MSRP Session Message” (C.7.1) with the message “Hello B” from client A. - Execute “MSRP Session Message” (C.7.1) with the message “Bye A” from the Test Tool. - User A receives an “Is Composing” indication from User B. - User A receives “Bye A” from User B. - User A terminates the session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 2 User A receives “Hello A” from User B. - At step 6 User A receives an “Is Composing” indication from User B. - At step 7 User A receives “Bye A” from User B. |

### RCS-con-122 Mobile Terminated 1-to-1 Chat – spam/blacklist

| Test Case Id | RCS-con-122 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Terminated 1-to-1 Chat session establishment when the sender is on the spam/blacklist |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.15 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS User A has User S listed in the spam/blacklist |
| Test Procedure | - Execute “Mobile Terminated Session Establishment: Spam” (C.2.7) with the message “Spam for A” from User S. - User A does not receive the message from User S. - The message “Spam for A” is available in the spam filter. |
| Pass-Criteria | - During step 1 Client A sends a delivery notification with status “delivered” to Client S. - At step 2 User A does not receive the message from User S. - At step 3 the message “Spam for A” is available in the spam filter. |

### RCS-con-123 Mobile Terminated 1-to-1 Chat – display notification

| Test Case Id | RCS-con-123 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies display notification in Mobile Terminated 1-to-1 Chat session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.2.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS User A’s device is configured to respond to display notification requests |
| Test Procedure | - Execute “Mobile Terminated Session Establishment: 1-to-1 Chat with display notification” (C.2.4) with the message “Hello A” from User B. - User A receives “Hello A” from User B. - User A sends the message “Hello B” to User B before the session times out. - Execute “MSRP Session Message” (C.7.1) with the message “HelloBye B” from client A. - Execute “MSRP Session with Display Notification” (C.7.2) with the message “Bye A” from the Test Tool. - User A receives “Bye A” from User B. - User A displays “Bye A” from User B. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - The pass-criteria are completely contained in the message flows in Appendix C. |

### RCS-con-124 Mobile Terminated 1-to-1 Chat – duplicate session

| Test Case Id | RCS-con-124 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies a duplicate session in a Mobile Terminated 1-to-1 Chat session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - Execute “Mobile Terminated Session Establishment: 1-to-1 Chat with two messages” (C.2.6) with the first message “Hello A” from User B and the second message “Wake up A” from User B. - User A receives “Hello A” from User B (but performs no action). - User A receives “Wake up A” from User B. - User A sends the message “Hello B” to User B before the session times out. - Execute “MSRP Session Message” (C.7.1) with the message “Hello B” from client A. - Execute “MSRP Session Message” (C.7.1) with the message “Bye A” from the Test Tool. - User A receives “Bye A” from User B. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 2 User A receives “Hello A” from User B. - At step 3 User A receives “Wake up A” from User B. - At step 7 User A receives “Bye A” from User B. |

### RCS-con-125 Mobile Terminated 1-to-1 Chat — race condition

| Test Case Id | RCS-con-125 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Terminated 1-to-1 Chat session race condition with new invite received after previous invite has been accepted |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.18.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - Execute “Mobile Terminated Session Establishment: new INVITE race” (C.2.9) with the first message “Hello A” from User B and the second message “Again A” from User B. - User A receives “Hello A” from User B. - User A receives “Again A” from User B. - User A sends the message “Two B” to User B before the session times out. - Execute “MSRP Session Message” (C.7.1) with the message “Two B” from client A. - Execute “MSRP Session Message” (C.7.1) with the message “Bye A” from the Test Tool. - User A receives “Bye A” from User B. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 2 User A receives “Hello A” from User B. - At step 3 User A receives “Again A” from User B. - At step 7 User A receives “Bye A” from User B. |

### RCS-con-126 Mobile Terminated 1-to-1 Chat — store and forward - FFS

| Test Case Id | RCS-con-126 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies delivery of deferred messages in a Mobile Terminated 1-to-1 Chat session. |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.4.11, RCS 3.2.4.19 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS User A is offline |
| Test Procedure | - User A comes online. - Execute “Mobile Terminated Session Store and Forward — deferred delivery” (C.2.12) with the first stored message “Hello A” from User B and the second stored message “Again A” from User B. - User A receives an indication of the stored message “Hello A” from User B and may optionally receive an indication of the stored message “Again A” from User B. User A opens the chat window. - Execute “MSRP Session Store and Forward Final Message Delivered” (C.7.5) with the stored message “Yet again A” from User B. - User A receives the stored message “Again A” from User B if not already received and then the message “Yet again A” from User B and the session is closed. |
| Pass-Criteria | - At step 3 User A receives an indication of the stored message “Hello A” from User B and optionally receives an indication of the stored message “Again A” from User B. - At step 5 User A receives the stored message “Again A” from User B if not already received and then receives the stored message “Yet again A” from User B and the session is closed. |

### RCS-con-127 Mobile Terminated 1-to-1 Chat – Switch up to group Chat

| Test Case Id | RCS-con-127 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated 1-to-1 Chat – Switch up to group Chat |
| Specification Reference | [RCS 5.x]: RCS 3.4.6.1.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | The client A’s handset coverage is 3G User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - User B (Test Tool) initiates a 1-to-1 Chat session with User A on his device with the message “Hello A”. - Execute “Mobile Originated Session Establishment” (?) with the message “Hello A”. - User B receives an indication that the message “Hello A” has been delivered. - User A receives “Hello A” from User B. - User B invites User C (Test Tool) to join the 1-to-1 Chat session. User A receives a SIP-Invite from the MSRP with Session-Replace and list of invitees (User A, User B & User C). - User A accepts the switch to the group chat. - User C accepts as well. |
| Pass-Criteria | - At Step 6 User A receives an Invite. - User A sends a SIP Bye of the original session and the Group Chat is established with a new RCS Group Chat ID if the group session hasn’t already existed before. |

## Mobile Originated Group Chat

### RCS-con-130 Mobile Originated Group Chat — establishment – Originating User leaves the session

| Test Case Id | RCS-con-130 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated Group Chat session establishment and Originating User leaves the session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5 [RCS 5.1]: RCS 3.4.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on [RCS 5.1] |
| Test Procedure | - User A initiates a Group Chat session with User B, User C and User D on his device with the subject “Test”. - Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with the subject “Test” and participants list with User B and User C as accepted (User D does not accept). - User A receives the participant list and the status for each one. - User A sends the message “Hello group” to the group. - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello group” from Client A. 5a. User A receives an indication that the message “Hello group” has been delivered [RCS 5.1]. 5b. If the MSRP SEND request in step 5 contains a request for display notification, then User A receives an indication that the message “Hello group” has been displayed [RCS 5.1]. - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello A” from the Test Tool from User B. 6a. User A receives an “Is Composing” indication from User B [RCS 5.1]. - User A receives “Hello A” from User B. - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 3 User A receives the participant list and the status for each one 1a. At step 5a User A receives an indication that the message “Hello group” has been delivered [RCS 5.1]. 1b. At step 5b. User A receives an indication that message “Hello group” has been displayed [RCS 5.1]. 1c. At step 6a User A receives an “Is Composing” indication from User B [RCS 5.1]. - At step 7 User A receives “Hello A” from User B. |

### RCS-con-131 Mobile Originated Group Chat — non-originating Users leave the session

| Test Case Id | RCS-con-131 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Originated Group Chat session when non-originating Users leave the session and the session is closed |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5 [RCS 5.1]: RCS 3.4.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on [RCS 5.1] |
| Test Procedure | - User A initiates a Group Chat session with User B, User C and User D on his device with the subject “Test”. - Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with the subject “Test” and participants list with User B and User C as accepted (User D does not accept). - Execute “MSRP Group Session Message” (C.7.6) with the message “B going” from the Test Tool from User B. 3a. User A receives an “Is Composing” indication from User B [RCS 5.1]. - User A receives the message “B going” from User B. - Execute “Participant List Update” (C.5.1) with User B as offline (and User C as accepted). - User A receives the participant list and the status for each one (User B as offline and User C as accepted). - Execute “MSRP Group Session Message” (C.7.6) with the message “C going” from the Test Tool from User C. 7a. User A receives an “Is Composing” indication from User C [RCS 5.1]. - User A receives the message “C going” from User C. - Execute “Participant List Update” (C.5.1) with User C as offline. - User A receives the participant list and the status for each one (User C as offline). - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 4 User A receives the message “B going” from User B. - At step 6 User A receives the participant list and the status for each one (User B as offline and User C as accepted). - At step 8 User A receives the message “C going” from User C. - At step 10 User A receives the participant list and the status for each one (User C as offline). |

### RCS-con-132 Mobile Originated Group Chat — adding new User to the session

| Test Case Id | RCS-con-132 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Group Chat when a new User is added to the session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5.5.4 [RCS 5.1]: RCS 3.4.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - User A initiates a Group Chat session with User B and User C on his device with the subject “Test”. - Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with the subject “Test” and participants list with User B and User C as accepted. - User A selects User D to add to the session. - Execute “Inviting new User to Group Chat” (C.5.2) with new User D added to the Group Chat. - User A receives the participant list and the status for each one (User D as added/accepted and User B and User C as accepted). - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 5 User A receives the participant list and the status for each one (User D as added/accepted and User B and User C as accepted). |

### RCS-con-135 Mobile Originated Group Chat — closed Group Chat

| Test Case Id | RCS-con-135 [RCS 5.1] |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies setting up a closed Group Chat and attempting to add a new User |
| Specification Reference | [RCS 5.1]: 3.4.4.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A initiates a Closed Group Chat session with User B and User C on his device with the subject “Closed group”. - Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with the subject “Closed group” with the additional attribute “a=chatroom:org.openmobilealliance.groupchat.closed” and participants list with User B and User C as accepted. - User A selects User D (who was not in the original invitation list) to add to the session. - User A may be prevented from selecting User D to add to the session, or if not prevented then execute “Inviting new User to Closed Group Chat” (C.5.4) with new User D invited to the Group Chat. In this last case User A receives an indication that User D cannot be added to the Closed Group Chat. - User A leaves the Closed Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 2, the pass-criteria are completely contained in the message flows in Appendix C. - At step 4 User A is either prevented from selecting User D to add to the session, or if not prevented then after selecting User D, User A receives an indication that User D cannot be added to the Closed Group Chat. |

## Mobile Terminated Group Chat

### RCS-con-140 Mobile Terminated Group Chat — establishment – Terminating User leaves the session

| Test Case Id | RCS-con-140 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Terminated Group Chat session establishment and Terminating User leaves the session |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5 [RCS 5.1]: RCS 3.4.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on [RCS 5.1] |
| Test Procedure | - Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User C also invited. - User A receives the invitation and the participant list. - User A accepts the invitation. - Continue to execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14). - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello group” from the Test Tool from Client B. 5a. User A receives an “Is Composing” indication from User B [RCS 5.1]. - User A receives the message “Hello group” from User B. - User A sends the message “Hello BC” to the Group. - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello BC” from User A to the Group. 8a. User A receives an indication that the message “Hello BC” has been delivered [RCS 5.1]. 8b. If the MSRP SEND request in step 8 contains a request for display notification, then User A receives an indication that the message “Hello BC” has been displayed [RCS 5.1]. - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 2 User A receives the invitation and the participant list (User B and User C). 1a. At step 5a User A receives an “Is Composing” indication from User B [RCS 5.1]. - At step 6 User A receives the message “Hello group” from User B. - At step 8a User A receives an indication that the message “Hello BC” has been delivered [RCS 5.1]. - At step 8b. User A receives an indication that message “Hello BC” has been displayed [RCS 5.1]. |

### RCS-con-141 Mobile Terminated Group Chat – establishment – Terminating User tries to rejoin

| Test Case Id | RCS-con-140 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Mobile Terminated Group Chat session establishment and Terminating User tries to rejoin |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5 [RCS 5.1]: RCS 3.4.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on [RCS 5.1] |
| Test Procedure | - Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User C also invited. - User A receives the invitation and the participant list. - User A accepts the invitation. - Continue to execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14). - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello group” from the Test Tool from Client B. 5a. User A receives an “Is Composing” indication from User B [RCS 5.1]. - User A receives the message “Hello group” from User B. - User A sends the message “Hello BC” to the Group. - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello BC” from User A to the Group 8a. User A receives an indication that the message “Hello BC” has been delivered [RCS 5.1]. 8b. If the MSRP SEND request in step 8 contains a request for display notification, then User A receives an indication that the message “Hello BC” has been displayed [RCS 5.1]. - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). - User A tries to rejoin the chat. |
| Pass-Criteria | - At step 11 User A is not able to rejoin the chat. |

## Group Chat Store and Forward

### RCS-con-145 Group Chat Store and Forward – notification for sent messages - FFS

| Test Case Id | RCS-con-145 [RCS 5.1] |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Group Chat Store and Forward – notification for sent messages delivered after the Group Chat has been closed |
| Specification Reference | [RCS 5.1]: 3.4.4.3.4, 3.4.4.3.5, B.1.14, B.1.15 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A, User B, User C and User D are in an established Group Chat session initiated by User B. ( Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A, User C and User D also invited). - User D leaves the session involuntarily. (Execute “Participant List Update” (C.5.1) with User D as offline). - User A sends the message “Hello group” to the group. - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello group” from Client A. - User A receives an indication that the message “Hello group” has been delivered to User B and User C (but not to User D). - If the MSRP SEND request in step 4 contains a request for display notification, then User A receives an indication that the message “Hello group” has been displayed by User B and User C (but not by User D). - The Group Chat session is torn down due to inactivity. Execute “Group Chat Session Tear Down” (C.2.22). - (User D gets back online and receives and then displays the message “Hello group”) - The IMS core (test tool) restarts the original Group Chat and Client A auto-accepts the invitation. Execute “Group Chat re-start: Mobile Terminated” (C.2.23), followed by “Auto-accept Group Chat” (C.2.20), followed by “Get Participant List” (C.5.3). - The IMS core (test tool) sends Client A an indication that the message “Hello group” has been delivered to User D. Execute “MSRP Session Store and Forward Message Delivered” (C.7.4). - User A receives an indication that the message “Hello group” has been delivered to User D. - If the MSRP SEND request in step 4 contains a request for display notification, then User A receives an indication that the message “Hello group” has been displayed by User D. - The IMS core (test tool) closes the Group Chat session. Execute “Group Chat Session Tear Down” (C.2.22). |
| Pass-Criteria | - At step 11 User A receives an indication that the message “Hello group” has been delivered to User D. - At step 12 User A receives an indication that the message “Hello group” has been displayed by User D. |

### RCS-con-146 Group Chat Store and Forward – delivery of stored messages - FFS

| Test Case Id | RCS-con-146 [RCS 5.1] |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Group Chat Store and Forward – delivery of stored messages after the Group Chat has been closed |
| Specification Reference | [RCS 5.1]: 3.4.4.3.4, 3.4.4.3.5, B.1.14, B.1.15 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A, User B, User C and User D are in an established Group Chat session initiated by User B. ( Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A, User C and User D also invited). - User A leaves the Group Chat involuntarily. Simulate by e.g. severely attenuating the cellular signal from the Test Tool to the device under test and waiting ixit_RF_loss_recognition_timer seconds. Note that Client A may succeed in sending a SIP BYE in this case. - (While User A is offline, User B sends the message “Hello group” to the group. The Group Chat is then torn down due to inactivity.) - Connectivity with the device under test is restored by re-establishing the cellular signal from the Test Tool. - Client A re-registers with the IMS core (Test Tool). - Client A automatically re-joins the original Group Chat session using the original Group Chat ID. Execute: “Group Chat automatic re-join” (C.2.15) with User A as the initiator. Client A receives and displays the participants list with User B, User C and User D as “FFS”. - The IMS core (Test Tool) sends the message “Hello group” to Client A. - Execute “MSRP Session Store and Forward Message Reception” (C.7.7) with the message “Hello group” (from Client B) to Client A. - User A receives the message “Hello group” from User B. - The IMS core (test tool) closes the Group Chat session. Execute “Group Chat Session Tear Down” (C.2.22). |
| Pass-Criteria | - At step 9 User A receives the message “Hello group” from User B. |

### RCS-con-147 Group Chat Store and Forward – race condition - FFS

| Test Case Id | RCS-con-147 [RCS 5.1] |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Group Chat – Store and Forward – race condition: user rejoins Group Chat which is being torn down due to inactivity |
| Specification Reference | [RCS 5.1]: 3.4.4.3.4, 3.4.4.3.5, B.1.16 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on |
| Test Procedure | - User A, User B, User C and User D are in an established Group Chat session initiated by User B. (Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A, User C and User D also invited). - User A leaves the Group Chat involuntarily. Simulate by e.g. severely attenuating the cellular signal from the Test Tool to the device under test and waiting ixit_RF_loss_recognition_timer seconds. Note that Client A may succeed in sending a SIP BYE in this case. - (While User A is offline, User B sends the message “Hello group” to the group.) - Connectivity with the device under test is restored by re-establishing the cellular signal from the Test Tool. - Client A re-registers with the IMS core (Test Tool). - Client A automatically re-joins the original Group Chat session using the original Group Chat ID. Execute: “Group Chat automatic re-join” (C.2.15) with User A as the initiator. Client A receives and displays the participants list with User B, User C and User D as present. - Before the stored message can be sent the Group Chat is torn down due to inactivity. Execute “Group Chat Session Tear Down” (C.2.22). - The IMS core (Test Tool) restarts the original Group Chat session using the original Group Chat ID. Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) using the original Group Chat ID and with User B, User C and User D as “FFS” in the participants list. - The IMS core (Test Tool) sends the message “Hello group” to Client A. - Execute “MSRP Session Store and Forward Message Reception” (C.7.7) with the message “Hello group” (from Client B) to Client A. - User A receives the message “Hello group” from User B. - The IMS core (test tool) closes the Group Chat session. Execute “Group Chat Session Tear Down” (C.2.22). |
| Pass-Criteria | - At step 11 User A receives the message “Hello group” from User B. |

## Group Chat Common

### RCS-con-165 Group Chat automatic re-join

| Test Case Id | RCS-con-165 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies Group Chat automatic re-join when a participant leaves the session due to loss of connectivity. |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5.5.6, [RIG 1.2.2]: RIG ID_4_21_2 [RCS 5.1]: RCS 3.4.4.3.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User B. ( Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A and User C also invited). - User A leaves the Group Chat involuntarily. Simulate by e.g. severely attenuating the cellular signal from the Test Tool to the device under test and waiting ixit_RF_loss_recognition_timer seconds. Note that Client A may succeed in sending a SIP BYE in this case [RCS 5.1]. - Connectivity with the device under test is restored by re-establishing the cellular signal from the Test Tool. - Client A re-registers with the IMS core (Test Tool). - Client A automatically re-joins the original Group Chat session using the original Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1]. Execute: “Group Chat automatic re-join” (C.2.15) with User A as the initiator. Client A receives and displays the participants list with User B and User C as “connected” [RCS 5.1]. - Execute “MSRP Group Session Message” (C.7.6) with the message “Hello again A” from User B to the Group. 6a. User A receives an “Is Composing” indication from User B [RCS 5.1]. - User A receives the message “Hello again A” from User B. - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 5 Client A automatically re-joins the original Group Chat. User A receives the participants list [RCS 5.1]. - At step 7 User A receives the message “Hello again A” from User B. |

### RCS-con-166 Group Chat re-start: 404 (Not Found)

| Test Case Id | RCS-con-166 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies correct behaviour after a Group Chat times out and a participant attempts to re-start the session and receives a 404 Not Found response from the IM server. |
| Specification Reference | [RCS 1.2.2]: RCS 3.2.5.5.6, [RIG 1.2.2]: RIG ID_4_21_3 [RCS 5.1]: RCS 3.4.4.1.7, RCS 3.4.4.3.3 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on [RCS 5.1] |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User B. (Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A and User C also invited). - The Group Chat session is torn down due to inactivity. Execute “Group Chat Session Tear Down” (C.2.22). - User A sends the message “Restart” to the original Group. - Client A attempts to automatically re-join the original Group Chat session using the original Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1]. This fails as the session has expired and the IM server (Test Tool) sends 404 Not Found. Execute: “Group Chat re-start: 404 Not Found” (C.2.16) with User A as the initiator. - Client A automatically starts a new Group Chat session using the original Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] and participant list. Execute: “Group Chat auto-start” (C.2.18) with User A as the initiator and User B and User C as the other members of the previous group session participants list. - Execute “MSRP Group Session Message” (C.7.6) with the message “Restart” from Client A. 6a. User A receives an indication that the message “Restart” has been delivered [RCS 5.1]. 6b. If the MSRP SEND request in step 6 contains a request for display notification, then User A receives an indication that the message “Restart” has been displayed [RCS 5.1]. - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 4 Client A attempts to automatically re-join the original Group Chat. - At step 5 Client A automatically starts a new Group Chat session using the original Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] and participant list. - At step 6a User A receives an indication that the message “Restart” has been delivered [RCS 5.1]. - At step 6b User A receives an indication that message “Restart” has been displayed [RCS 5.1]. |

### RCS-con-167 Group Chat re-start: 403 (Forbidden)

| Test Case Id | RCS-con-167 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies correct behaviour after a Group Chat times out and a participant attempts to re-start the session but receives a 403 Forbidden response from the IM server. |
| Specification Reference | [RIG 1.2.2]: RIG ID_4_21_3 [RCS 5.1]: RCS 3.4.4.1.7 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User B. (Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A and User C also invited). - The Group Chat session is torn down due to inactivity. Execute “Group Chat Session Tear Down” (C.2.22). - User A sends the message “Restart” to the original Group. - Client A attempts to automatically re-join the original Group Chat using the original Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1]. This fails as the session has expired and the IM server (Test Tool) sends 403 Forbidden. Execute: “Group Chat re-start: 403 Forbidden” (C.2.17) with User A as the initiator. - Client A either: - Abandons the attempt to start a Group Chat and sends no new messages [RCS 1.2.2 only]. Or b) Starts a new Group Chat in which case execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with User A as the initiator, with the subject “Restart” and participants list with User B and User C [RCS 1.2.2 and RCS 5.1]. Then User A leaves the Group Chat session. Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 4 Client A attempts to automatically re-join the original Group Chat. - At step 5 Client A either: - Sends no messages [RCS 1.2.2 only]. Or b) Starts a new Group Chat [RCS 1.2.2 and RCS 5.1]. |

### RCS-con-168 Group Chat: Concurrent sessions

| Test Case Id | RCS-con-168 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Verifies correct behaviour after a Group Chat is re-started more than once leading to two concurrent sessions. |
| Specification Reference | [RIG 1.2.2]: RIG ID_4_21_3 [RCS 5.1]: RCS 3.4.4.1.7 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS In case of User A’s device support UI for switching display notifications on/off then display notification are to be switched on [RCS 5.1] |
| Test Procedure | - User A, User B, User C and User D are in a re-started Group Chat session (session 1), re-started by User B. (Execute “Mobile Terminated Session Establishment: Group Chat” (C.2.14) with User B as the initiator and User A and User C and User D also invited). User C does not re-join (as is offline). - Wait 5 seconds. - User C (the Test Tool) re-starts the same Group Chat using the same Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] (session 2). (User C was offline at step 1 and also was not aware that User D was a member of the Group Chat). Execute: “Group Chat concurrent re-start: Mobile Terminated” (C.2.19) with User C as the initiator and User A and User B (only) as the other members of the participants list. - Client A auto-accepts the group session request from step 3. Execute: “Auto-accept Group Chat” (C.2.20). - Client A auto-invites Client D to the Group Chat (session 2). Execute: “Auto-invite Group Chat” (C.2.21). - User A sends the message “Hello again” to the group using session 2. - Execute “MSRP Group Session Message” (C.7.6) (session 2) with the message “Hello again” from Client A. 7a. User A receives an indication that the message “Hello again” has been delivered [RCS 5.1]. 7b. If the MSRP SEND request in step 7 contains a request for display notification, then User A receives an indication that the “Hello again” has been displayed [RCS 5.1]. - (User B (the Test Tool) did not receive the second re-start (session 2) at step 3 and so when he sends a message Client B uses session 1). Execute “MSRP Group Session Message” (C.7.6) with the message “Back again” from the Test Tool from Client B using session 1. 8a. User A receives an “Is Composing” indication from User B [RCS 5.1]. - User A receives the message “Back again” from User B. - Client A auto-invites Client B to the Group Chat (session 2). Execute: “Auto-invite Group Chat” (C.2.21). - User A leaves the Group Chat session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 4 Client A auto-accepts the group session request. - At step 5 Client A auto-invites Client D to the Group Chat session 2. - At step 7 the MSRP Group Session uses session 2. - At step 9 User A receives the message “Back again” from User B. - At step 10 Client A auto-invites Client B to the Group Chat session 2. |

### RCS-con-169 Group Chat: Maximum Concurrent Sessions (Includes Optional Features)

| Test Case Id | RCS-con-169 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Group Chat: Maximum Concurrent Sessions |
| Specification Reference | [RCS 5.x]: RCS 3.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS RCS Provisioning Parameter MAX CONCURRENT SESSION is set to 1 |
| Test Procedure | - User A initiates a Group Chat session with User B, User C and User D on his device with the subject “Test”. - Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with the subject “Test” and participants list with User B and User C as accepted (User D does not accept). - User A receives the participant list and the status for each one. - User A sends the message “Hello group” to the group. - User A initiates another Group Chat session with User E and User F on his device with the subject “Test2”. - Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with the subject “Test2” and participants list with User E and User F as accepted. - User A sends the message “Hello group2” to the group. |
| Pass-Criteria | - At step 6 the first group chat is torn down and User A receives the participant list of the second group chat with User E and User F and the status for each. - At step 7 User A receives an indication that the message “Hello group2” has been delivered [RCS 5.1]. |

## Mobile Originated File transfer

### RCS-con-200 Mobile Originated File Transfer (Includes Optional Features)

| Test Case Id | RCS-con-200 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated File Transfer |
| Specification Reference | [RCS 1.2.2]: RCS 3.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - User A initiates a File Transfer towards User B on his device. - Execute “Mobile Originated Session Establishment” (C.2.1). - User A’s RCS client sends a file in an MSRP session. - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 3 the MSRP session is setup correctly and a file is transferred successfully. |

### RCS-con-201 Mobile Originated File Transfer – Rejected (Includes Optional Features)

| Test Case Id | RCS-con-201 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile originated file transfer – Rejected |
| Specification Reference | [RCS 1.2.2]: RCS 3.4.3 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - User A initiates a File Transfer towards User B on his device. - Execute “Mobile Originated Session – Reject” (C.4.1). |
| Pass-Criteria | - At step 2 User A’s RCS client shows User B rejected the file sharing session. |

### RCS-con-202 Mobile Originated File Transfer – Store and Forward Sending 1 - FFS

| Test Case Id | RCS-con-202 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated File Transfer – Store and Forward Sending 1 |
| Specification Reference | [RCS 5.x]: RCS 3.5.4.7 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - User A initiates a 1-to-1 Chat session with User B on his device with the message “Hello B”. (User B is offline) - Execute “Mobile Originated Session: Store and Forward — Receiver offline” (C.2.10) with the message “Hello B”. - User A selects a file (e.g. image) to send it to User B. (User B is offline) - Execute “MSRP Session Store and Forward File Transfer” (C.7.8). - (User A keeps chat session open) - After transfer has been completed execute “MSRP Session Store and Forward File Transfer Completed” (C.7.9). - (User A keeps chat session open and User B comes back online) - Execute “MSRP Session Store and Forward Message Delivered” (C.7.4). - Execute “MSRP Session Store and Forward File Delivered Notification” (C.7.10). |
| Pass-Criteria | - At Step 4 User A’s RCS client shows the progress of the upload (transfer bar). - At Step 6 User A should not get a notification that the file has been delivered. - At Step 9 User A should get a notification that the file has been delivered. |

### RCS-con-205 Mobile Originated File Transfer – File Transfer within a Group Chat

| Test Case Id | RCS-con-205 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated File Transfer – Send a File Transfer within a Group Chat |
| Specification Reference | [RCS 5.x]: RCS 5.3.1 and RCS 3.5.4.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User A. (Execute “Mobile Originated Session Establishment: Group Chat” (C.2.1.3) with User A as the initiator and User B and User C also invited). - User A initiates a File Transfer towards User B and User C on his device. - Execute “Mobile Originated Session Establishment in Group Chat” (C.2.24). - User A’s RCS client sends a file in an MSRP session. - Execute “Mobile Originated Session Termination: Image share, File share, Group Sessions” (C.3.5). |
| Pass-Criteria | - At step 3 the MSRP session is setup correctly and a file is transferred successfully. |

### RCS-con-206 Mobile Originated File Transfer – Resume a Send File Transfer - FFS

| Test Case Id | RCS-con-206 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated File Transfer – Resume a Send File Transfer |
| Specification Reference | [RCS 5.x]: RCS 5.1.3 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - User A initiates a File Transfer towards User B on his device. - Execute “Mobile Originated Session Establishment” (C.2.1). - User A’s RCS client sends a file in an MSRP session. - User B (Test Tool) leaves the Group Chat involuntarily. Simulate by e.g. severely attenuating the cellular signal and waiting ixit_RF_loss_recognition_timer seconds. Note that Client B may succeed in sending a SIP BYE in this case [RCS 5.1]. - Connectivity with User B’s client is restored by re-establishing the cellular signal from the Test Tool. - Client B re-registers with the IMS core (Test Tool). - Client B automatically re-joins the original Group Chat session using the original Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1]. Execute: “Group Chat automatic re-join” (C.2.15). - File Transfer is resumed and completed. - Execute “Store and Forward File Transfer Delivered Notification” (C.7.10). - Execute “Mobile Originated Session Termination” (C.3.1). |
| Pass-Criteria | - At step 6 the MSRP session is again setup correctly and the file transfer is completed successfully. - At step 9 User A is notified that the file has been delivered. |

### RCS-con-209 Mobile Originated File Transfer – Send Audio Message

| Test Case Id | RCS-con-209 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated File Transfer – Send Audio Message |
| Specification Reference | [RCS 5.x]: RCS 3.11.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransferAudioMessage |
| Test Procedure | - User A selects User B (Test Tool) from the Adress Book and selects to send a Audio Message. - User A records the Audio Message using the AMR codec and the microphone of the DUT and sends it to User B. - Execute “Mobile Originated Session Establishment” (C.2.1) - User A’s RCS client sends the file in an HTTP or MSRP session based on the parameter set at FT DEFAULT MECH. - Execute “Mobile Originated Session Termination” (C.3.1) |
| Pass-Criteria | - At step 3 the HTTP or MSRP session is setup correctly and an audio file is transferred successfully. |

### RCS-con-220 Mobile Originated File Transfer in a Group Chat – Blocked file transfer

| Test Case Id | RCS-con-220 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated File Transfer in a Group Chat – Blocked File Transfer |
| Specification Reference | [RCS 5.x]: RCS3.5.4.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User A. (Execute “Mobile Originated Session Establishment: Group Chat” (C.2.1.3) with User A as the initiator and User B and User C also invited). Conference Focus indicated support for File Transfer in Chat. User A is blacklisted on the ‘rcs_pnb_ft_blockedusers’ list of the BPEF. - User A initiates a File Transfer towards User B and User C (both Test Tool) on his device. - BPEF (Test Tool) sends a 403 Forbidden with a warning header set to “122 Function not allowed”. |
| Pass-Criteria | - If the Common Message Store is supported it shall store the File Transfer History object data as defined in [RCS5-CPM-MSGSTOR-ENDORS] for the blocked File Transfer event in user´s dedicated Blocked Folder. - User A is informed that File Transfer is not allowed. |

## Mobile Terminated File transfer

### RCS-con-210 Mobile Terminated File Transfer (Includes Optional Features)

| Test Case Id | RCS-con-210 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated File Transfer |
| Specification Reference | [RCS 1.2.2]: RCS 3.4.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - Execute “Mobile Terminated Session Establishment” (C.2.2). - The test tool sends a file in an MSRP session to User A’s RCS client. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 2 User A’s RCS client receives the file. |

### RCS-con-211 Mobile Terminated File Transfer – Rejected (Includes Optional Features)

| Test Case Id | RCS-con-211 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile terminated file transfer – Rejected |
| Specification Reference | [RCS 1.2.2]: RCS 3.4.3 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS User A’s RCS device is configured with File Transfer Auto-accept set to “off” (ftAutAccept set to 0) - Applicability: - ics_fileTransfer |
| Test Procedure | - Execute “Mobile Terminated Session – Reject” (C.4.2). |
| Pass-Criteria | - At step 1 User A’s RCS client shows no ongoing file transfer. |

### Void

### Void

### RCS-con-214 Mobile Terminated File Transfer – Store and Forward Receiving 1 - FFS

| Test Case Id | RCS-con-214 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated File Transfer – Store and Forward Receiving 1 |
| Specification Reference | [RCS 5.x]: RCS 3.5.4.7 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS Have a testfile (e.g. image) ready on the FTF with a known SDP file-transfer-id and file-name - Applicability: - ics_fileTransfer |
| Test Procedure | - Execute “Mobile Terminated Session Establishment” (C.2.2). - Execute “Mobile Terminated File Transfer – Store and Forward Receiving” (C.2.25). - Execute “Mobile Terminated Session Termination” (C.3.6). |
| Pass-Criteria | - At step 2 User A’s RCS client receives the file. |

### RCS-con-218 Mobile Terminated File Transfer in a Group Chat – Receive a file

| Test Case Id | RCS-con-218 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated File Transfer in a Group Chat – Receive a file |
| Specification Reference | [RCS 5.x]: RCS 3.5.4.2 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS User A, B and C all include IARI tags for the RCS File Transfer services they support in the contact header (+g.3gpp.iari-ref="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.ft") - Applicability: - ics_fileTransfer |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User A. (Execute “Mobile Originated Session Establishment: Group Chat” (C.2.1.3) with User A as the initiator and User B and User C also invited). Conference Focus indicated support for File Transfer in Chat by including the IARI tags. - User B (Test Tool) sends a File in an MSRP session towards User A’s and User C’s (Test Tool) RCS client which is handled by the conference focus. - User A accepts the file transfer. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 3 a test file is received and successfully downloaded. |

### RCS-con-219 Mobile Terminated File Transfer in a Group Chat – Ignore to receive a file

| Test Case Id | RCS-con-219 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated File Transfer in a Group Chat – Ignore to receive a file |
| Specification Reference | [RCS 5.x]: RCS 3.5.4.7.1 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS User A, B and C all include IARI tags for the RCS File Transfer services they support in the contact header (+g.3gpp.iari-ref="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.ft") - Applicability: - ics_fileTransfer |
| Test Procedure | - User A, User B and User C are in an established Group Chat session initiated by User A. (Execute “Mobile Originated Session Establishment: Group Chat” (C.2.13) with User A as the initiator and User B and User C also invited). Conference Focus indicated support for File Transfer in Chat by including the IARI tags. - User B (Test Tool) sends a File in an MSRP session towards User A’s and User C’s (Test Tool) RCS client which is handled by the conference focus. - User A ignores the file transfer for at least the time defined in the corresponding configured timer. - The terminating FTF cancels the SIP INVITE request towards the receiver by sending a SIP CANCEL and includes a reason header based on [RFC3326]: Reason: SIP;cause=408;text="User not responding". |
| Pass-Criteria | - At step 4 User A receives a SIP CANCEL with the reason header “408 User not Responding”. |

### RCS-con-223 Mobile Terminated File Transfer – Resume receiving a file

| Test Case Id | RCS-con-223 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated File Transfer – Resume receiving a file |
| Specification Reference | [RCS 5.x]: RCS 3.5.4 and [RFC5547] |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransfer |
| Test Procedure | - Execute “Mobile Terminated Session Establishment” (C.2.2). - The test tool sends a large file in an MSRP session to User A’s RCS client. - While transferring the file User A’s handset is power cycled. - User A restarts the RCS client. - Execute “Mobile Originated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 4 the device reconnects and sends a SIP INVITE using the file-range attribute including the file-selector to User B (Test Tool) asking for the missing part of the file. After resuming the file transfer is completed. |

### RCS-con-226 Mobile Terminated File Transfer – Receive Audio Message

| Test Case Id | RCS-con-226 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated File Transfer – Receive Audio Message |
| Specification Reference | [RCS 5.x]: RCS 3.11.4 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_fileTransferAudioMessage |
| Test Procedure | - Execute “Mobile Terminated Session Establishment” (C.2.2). - The test tool sends a file in an HTTP or MSRP session based on the parameter set at FT DEFAULT MECH to User A’s RCS client. - Execute “Mobile Terminated Session Termination” (C.3.2). |
| Pass-Criteria | - At step 2 User A’s RCS client receives the file and is able to display it on the DUT. |

## Mobile Originated Image sharing

### RCS-con-250 Mobile Originated Image Sharing – Session Establishment – Mobile Originated Session Termination (Includes Optional Features)

| Test Case Id | RCS-con-250 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated Image sharing session – Mobile originated Session termination |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.8 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_imageShare |
| Test Procedure | - User A establishes a CS Voice call to User B. - Execute “Mobile Originated Capability Exchange” (C.1.1). - User A initiates an Image Share session towards User B on his device. - Execute “Mobile Originated Session Establishment” (C.2.1). - User A’s RCS client sends an image in an MSRP session. - Execute “Mobile Originated Session Termination” (C.3.1). - Execute “Mobile Terminated Capability Exchange” (C.1.2). |
| Pass-Criteria | - At step 5 the MSRP session is setup correctly and an image is transferred successfully. - At step 7 User A’s RCS client shows User B still as available for image sharing. |

### RCS-con-251 Mobile Originated Image Sharing – Session Establishment – rejected (Includes Optional Features)

| Test Case Id | RCS-con-251 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile originated image sharing session establishment – Rejected |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.11 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_imageShare |
| Test Procedure | - User A establishes a CS Voice call to User B. - Execute “Mobile Originated Capability Exchange” (C.1.1). - User A initiates an Image Share session towards User B on his device. - Execute “Mobile Originated Call – Reject” (C.4.1). |
| Pass-Criteria | - At step 4 User A’s RCS client shows User B rejected the image sharing session. |

## Mobile Terminated Image sharing

### RCS-con-270 Mobile Terminated Image Sharing – Session Establishment – Mobile Terminated Session Termination (Includes Optional Features)

| Test Case Id | RCS-con-270 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Terminated Image sharing session – Mobile Terminated session termination |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.8 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS Applicability: - ics_imageShare |
| Test Procedure | - User A establishes a CS Voice call to User B. - Execute “Mobile Originated Capability Exchange” (C.1.1). - Execute “Mobile Terminated Session Establishment” (C.2.2). - The test tool sends an image in an MSRP session to User A’s RCS client. - Execute “Mobile Terminated Session Termination” (C.3.2). - Execute “Mobile Originated Capability Exchange” (C.1.1). |
| Pass-Criteria | - At step 4 User A’s RCS client receives the image. - At step 6 User A’s RCS client shows User B still as available for image sharing. |

### RCS-con-271 Mobile Terminated Image Sharing – Session Establishment – rejected (Includes optional Features)

| Test Case Id | RCS-con-271 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile terminated Image sharing session establishment – Rejected |
| Specification Reference | [RCS 1.2.2]: RCS 3.3.11 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS - Applicability: - ics_imageShare |
| Test Procedure | - User A establishes a CS Voice call to User B. - Execute “Mobile Originated Capability Exchange” (C.1.1). - Execute “Mobile Terminated Session – Reject” (C.4.2). |
| Pass-Criteria | - At step 3 User A’s RCS client shows no ongoing image share session. |

## Void

## Void

## Void

## Void

## Void

## Void

## Call Establishment and Termination

### RCS-con-600 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination (Includes Optional Features)

| Test Case Id | RCS-con-600 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated IP Voice call set up – Mobile originated IP Voice call termination |
| Specification Reference | [RCS 5.2]: RCS 3.8 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS WiFi/WLAN connection only - Applicability: - ics_IPVoiceCall |
| Test Procedure | - User A establishes an IP Voice call with User B (Test Tool) - Execute “Mobile Originated IP Voice Call Establishment” (C.2.1) - User A ends the IP Voice call - Execute “Mobile Originated IP Voice Call termination” (C.3.1) |
| Pass-Criteria | - At step 2 the IP voice call is established. |

## Enriched Calling

### Void

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version |

Draft/Candidate Version 5.x History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-ETS-RCS-CON-V5_2 | 03 Nov 2014 | All | New conformance specification created from previous RCS 1.2.2 specification, in OMA-ETS-RCS-CON-V1_2_2-20140623-D |
|  | 11 Nov 2014 | 5.18, B.3.1, B.4, C.2, C.2.1, C.3.1, D.1.1, E | Incorporated CR: OMA-IOP-2014-0192R01-CR_RCS_IP_Voice_Call_test_case |
|  | 26 Jan 2015 | 2.2, 5.4.7, 5.4.8, 5.5.3, 5.5.4, 5.7.9-5.7.13, 5.8.8, 5.8.9, 5.9.4-5.9.10, 5.10.2-5.10.10, 5.11, 5.12.3-5.12.10, 5.13.5-5.13.17, 5.16.4, 5.17.4-5.17.8, 5.18, 5.19, 5.20, 5.21, E | Incorporated CRs: OMA-IOP-2015-0019-CR_RCS5.x_Workplan_Integration OMA-IOP-2015-0021-CR_RCS_5.2_ETS_update Editorial changes |
|  | 04 Feb 2015 | 5.5.3, 5.8.5, 5.8.8, 5.8.9, 5.9.4-5.9.9, 5.10.3-5.10.10, 5.14.1, 5.16.4, 5.17.4, 5.20, 5.21, E | Incorporated CR: OMA-IOP-2015-0026-CR_RCS5_Clean_Up |
|  | 17 Mar 2015 | 1, 2.2, 4, 5.7.4, 5.7.7, 5.9.1-5.9.7, 5.9.10, 5.10.1, 5.11.1, 5.11.2, 5.11.3, 5.11.4, C.2.15, C.7.1, C.7.5, C.7.6, D.1, D.1.1 | Incorporated CRs: OMA-IOP-2015-0044-CR_RCS_5.2_ETS_con_103_correction OMA-IOP-2015-0045-CR_RCS_5.2_ETS_displayed_notifications_correction OMA-IOP-2015-0046R01-CR_RCS_5.2_ETS_update_basic_group_chat_tests OMA-IOP-2015-0047-CR_RCS_5.2_ETS_update_complex_group_chat_tests |
|  | 01 Apr 2015 | 3.2, 5.11.1, 5.11.2, 5.11.3, 5.11.4, C.2.15-C.2.19, C.2.22 | Incorporated CRs: OMA-IOP-2015-0053-CR_RCS_5.2_ETS_update_con_168_group_chat_test OMA-IOP-2015-0054-CR_RCS_5.2_ETS_Group_Chat_ID_update OMA-IOP-2015-0055-CR_RCS_5.2_ETS_Group_Chat_session_tear_down OMA-IOP-2015-0056-CR_RCS_5.2_ETS_Group_Chat_re_start_403 Editorial changes |
|  | 14 Apr 2015 | 5.7.7, 5.7.8, 5.9.1, 5.9.4, 5.9.5, 5.9.8, 5.9.9, 5.10.1, 5.10.5, 5.10.6, 5.10.9, 5.10.10, 5.11.1, 5.11.2, 5.11.3, 5.12, 5.12.2, 5.12.4, B.3.1, B.3.3, C.2.19, C.2.23, C.5.3, C.7.4 | Incorporated CRs: OMA-IOP-2015-0066-CR_RCS_5.2_ETS_Group_Chat_display_notification_correction OMA-IOP-2015-0067-CR_RCS_5.2_ETS_Group_Chat_store_forward_tests Editorial changes |
| Draft Versions OMA-ETS-RCS-CON-V5_1 | 15 Apr 2015 | n/a | Incorporated CR: OMA-IOP-2015-0071-CR_OMA_ETS_RCS_CON_V5_1 |
|  | 28 Apr 2015 | 5.9.4, 5.9.5, 5.11.2, 5.11.3, 5.11.4, 5.11.5, C.5.4, C.7.7 | Incorporated CRs: OMA-IOP-2015-0077-CR_RCS_5.2_ETS_More_Group_Chat_store_and_forward_tests OMA-IOP-2015-0078-CR_RCS_5.1_ETS_Closed_Group_Chat_test Editorial changes |
|  | 18 May 2015 | E | Incorporated CR: OMA-IOP-2015-0086-CR_RCS_5.1_ETS_Update_Annex_E Editorial changes |
| Candidate Version OMA-ETS-RCS-CON-V5_1 | 26 May 2015 | n/a | Status changed to Candidate by TP TP Ref # OMA-TP-2015-0084-INP_RCS_V5_1_CON_ETS_for_Candidate_approval |
| Draft Versions OMA-ETS-RCS-CON-V5_1 | 12 Jun 2015 | 2.2, 3.3, 5.3.1, 5.6.4, 5.7.4, 5.7.9-5.7.13, 5.13.6, 5.13.7, 5.13.8, 5.13.11, 5.14.2, 5.14.9, 5.14.10, 5.14.11, 5.14.15, 5.15.1, 5.16.2, 5.17.1, 5.17.2, 5.23.1, C.2.5, C.2.24, C.3.5, D.2.5 | Incorporated CRs: OMA-IOP-2015-0090-CR_RCS_5.2_ETS_Editorial OMA-IOP-2015-0093-CR_RCS_5_File_Transfer_Part1 OMA-IOP-2015-0099-CR_RCS_5_File_Transfer_Part2 OMA-IOP-2015-0108-CR_RCS_Autoconfig_message_content_correction Editorial changes |
|  | 19 Jun 2015 | 5.13.1, 5.13.6, 5.13.7, 5.13.11, 5.14.9, 5.14.10 | Incorporated CR: OMA-IOP-2015-0123-CR_RCS_5_Precondition_Correction |
|  | 14 Jul 2015 | 3.3, 5.7.7, 5.13.3, 5.13.5, 5.14.5, 5.15, 5.16, 5.17, 5.18.1, 5.18.2, 5.18.3, 5.18.4, 5.19, 5.20, 5.21, 5.22, C.2.10, C.2.11, C.2.12, C.2.22, C.2.25, C.3.6, C.7.2, C.7.8, C.7.9, C.7.10 | Incorporated CRs: OMA-IOP-2015-0133R01-CR_RCS_FileTransfer3 OMA-IOP-2015-0134R01-CR_RCS_FileTransfer4 |
|  | 28 Jul 2015 | 3.3, 5.4.7, 5.4.8, 5.5, 5.6, 5.7.1, 5.7.2, 5.7.4, 5.7.5, 5.7.7, 5.7.8, 5.7.9, 5.8.1, 5.8.2, 5.8.6, 5.8.7, 5.8.8, 5.9, 5.10, 5.11, 5.12.1, 5.12.2, 5.12.3, 5.13.3, C.1.3, C.1.4, C.1.5, C.1.6, C.4, C.6.1.1, C.7.1-C.7.5, C.7.7, C.7.8, C.7.10, D.1, D.1.1, D.1.2, D.1.3, D.1.4, D.1.11, D.1.13, D.2.4, D.2.5, E | Incorporated CRs: OMA-IOP-2015-0130R01-CR_RCS_Switch_to_GroupChat OMA-IOP-2015-0140R01-CR_RCS_SocialPresence |
|  | 11 Aug 2015 | 5.21.1, 5.21.2, 5.21.7, 5.22.1, 5.22.2, 5.22.6, C.2.26, C.2.27, C.2.28, C.2.29, C.4.3, C.4.4, E | Incorporated CR: OMA-IOP-2015-0150-CR_RCS_IP_Video_Call_Tests_Cover |
|  | 02 Sep 2015 | 1, 2.2, 3.3, 4, 5.7.10, 5.7.11, 5.7.12, 5.7.13, 5.8.9, 5.12.5, 5.13.4, 5.13.5, 5.13.9, 5.13.10, 5.14.6, 5.14.7, 5.14.8, 5.14.13, 5.14.14, 5.14.17, 5.17.4, 5.17.5, 5.17.6, 5.18.4-5.18.8, 5.19, 5.19.2, 5.19.3, 5.19.4, 5.20, 5.20.2, 5.20.3, 5.20.5-5.20.10, 5.21.3, 5.21.4, 5.21.6, 5.22.3, 5.22.4, 5.22.5, 5.23.1, C.2.30, C.4.5, C.4.6, E | Incorporated CRs: OMA-IOP-2015-0141R03-CR_RCS_Large_Message_Mode OMA-IOP-2015-0159-CR_RCS_Cleanup OMA-IOP-2015-0160R01-CR_RCS_IP_Video_Call_Tests2 OMA-IOP-2015-0161R01-CR_RCS_Audio_Message_Test OMA-IOP-2015-0162-CR_RCS_Max_Concurrent_Sessions_Tests OMA-IOP-2015-0163-CR_RCS_Revoke_Message_Test OMA-IOP-2015-0167R01-CR_RCS_Resume_File_Reception OMA-IOP-2015-0169R01-CR_RCS_Video_Share_Outside_Call |
|  | 09 Sep 2015 | 5.7.11, 5.7.12, 5.8.9, 5.14.12, 5.14.16, 5.19, C.2.24 | Incorporated CR: OMA-IOP-2015-0181-CR_RCS_Cleanup_and_PagerMode Editorial changes |
|  | 29 Sep 2015 | 5.13.11, 5.14.10, 5.18.5, 5.19.2, 5.19.3, 5.20.2, 5.20.3, 5.21.5 | Incorporated CRs: OMA-IOP-2015-0182-CR_RCS5_Large_Message_Mode OMA-IOP-2015-0184-CR_RCS5_IP_Video_Call |
|  | 12 Nov 2015 | 2.2, 5.2.1, 5.10.2, 5.13.7, 5.13.11, 5.14.9, 5.14.10, 5.19.1, 5.20.1, 5.20.4-5.20.10, 5.21.3, 5.21.4, 5.21.5, 5.22.3, 5.22.4, 5.24, C.2.31, C.5.5-C.5.12, D.2.4, D.2.5, E | Incorporated CRs: OMA-IOP-2015-0192R02-CR_RCS5_Structural_Update OMA-IOP-2015-0194R02-CR_RCS5_Pager_Mode_Tests OMA-IOP-2015-0203R01-CR_RCS5_Annex_Fix1 OMA-IOP-2015-0204-CR_Add_IMS_registration_001 OMA-IOP-2015-0205-CR_Add_PagerMode_Default_procedure OMA-IOP-2015-0208-CR_RCS_autoconfig_clean_up OMA-IOP-2015-0209-CR_RCS_CS_to_IP_Video_call_clean_up Editorial changes |
|  | 15 Dec 2015 | 3.3, 5.7.11, 5.7.12, 5.7.13, 5.24, C.5.13, C.5.14, C.5.15, E | Incorporated CR: OMA-IOP-2015-0219-CR_RCS_Crane_Tests |
|  | 25 Jan 2016 | 5.8.9, 5.13.6, 5.13.10, 5.18.7, 5.18.8, 5.19.1, 5.19.2, 5.19.3, 5.20.1, 5.20.6-5.20.10, 5.21.5, 5.21.6, 5.23.1, 5.24.1-5.24.8, C.2.24, C.2.30, C.5.13, C.5.14, C.5.15, C.5.16, C.8, D.1.17, D.1.18, D.1.19, D.2.5, E | Incorporated CRs: OMA-IOP-2016-0002R01-CR_RCS_5_3_Features OMA-IOP-2016-0008-CR_Autoconfig_Corrections OMA-IOP-2016-0009-CR_RCS_Presence_procedures |
|  | 25 Feb 2016 | 5.2.2, 5.2.3, 5.4.9, 5.5.3, 5.5.4, 5.5.5, 5.9.5, 5.13.4, 5.13.5, 5.13.10, 5.13.11, 5.14.6, 5.14.7, 5.14.8, 5.14.11, 5.14.12, 5.14.13, 5.14.15, 5.14.16, 5.20.4, 5.20.5, 5.24.7, C.1.5, C.1.6, E | Incorporated CR: OMA-IOP-2016-0011R03-CR_RCS5_Geolocation_Procedures Editorial changes |
| Draft Versions OMA-ETS-RCS-CON-V5_x | 26 Aug 2016 | 3.3, 5.21.5, A.2 | Incorporated CRs: OMA-IOP-2016-0054-CR_RCS5.x_ETS_Titlechange OMA-IOP-2016-0056-CR_RCS5_ETS_Fix_con_504 |
|  | 08 Oct 2016 | 5.2.2, C.1.3, C.1.4 | Incorporated CR: OMA-IOP-2016-0072R01-CR_RCS5x_ETS_Update_Mandatory_Test_List |
| Candidate Version OMA-ETS-RCS-CON-V5_x | 18 Oct 2016 | n/a | Status changed to Candidate by TP TP Ref # OMA-TP-2016-0096R01-INP_RCS_V5_1_CON_ETS_for_Candidate_re_approval |
| Draft Version OMA-ETS-RCS-CON-V5_x | 03 Nov 2016 | D.1.5-D.1.9, D.1.11, D.1.12, D.1.14, D.1.15, D.1.16, D.1.17, D.3 | Incorporated CRs: OMA-IOP-2016-0086R01-CR_RCS_default_publish OMA-IOP-2016-0087-CR_RCS_default_responses OMA-IOP-2016-0093R01-CR_RCS_default_conference OMA-IOP-2016-0095-CR_RCS_default_MSRP |
|  | 04 May 2017 | C.2.1 | Incorporated CR: OMA-IOP-2017-0054-CR_RCS_Remove_183_for_IETF_mode |
| Candidate Version OMA-ETS-RCS-CON-V5_x | 22 Aug 2017 | n/a | Status changed to Candidate by TP TP Ref # OMA-TP-2017-0034-INP_RCS_5.x_Testing-V1_0_ETS_for_Notification |

Conformance Test Case applicability

Introduction

This appendix allows implementers of RCS clients or servers to select the appropriate Conformance test cases that are applicable to the features implemented.

This appendix lists:

- All test cases testing only mandatory features as described in [RCS 1.2.2] section 1.2.2 “Conformance”,

- ICS (Implementation Conformance Statement)

- IXIT (Implementation eXtra Information for testing)

- The mapping from ICS/IXIT to the applicable optional test cases.

Client Test Cases testing only mandatory features

# These Client Conformance test cases are independent from any applicability and are testing only mandatory features and SHALL be run with every implementation.

Client Test Cases

RCS-con-001 First-time unsuccessful configuration: Subscriber unauthorized (Auto-Configuration Server)

RCS-con-004 Configuration: RCS re-configuration successful

RCS-con-030 Mobile Originated Capability Discovery – Successful

RCS-con-031 Mobile Originated Capability Discovery – Unsuccessful – 480 Temporary Unavailable

RCS-con-032 Mobile Originated Capability Discovery – Unsuccessful – 408 Request Timeout

RCS-con-033 Mobile Originated Capability discovery – Unsuccessful – 404 Not Found

RCS-con-034 Mobile Originated Capability Discovery – Successful – Multiple Identities

RCS-con-035 Mobile Originated Capability Discovery – Successful (IMS registered, does not support RCS)

RCS-con-040 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities

RCS-con-041 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS Capabilities

RCS-con-051 Capability update during a MT call for video/image sharing

RCS-con-053 Capability Exchange Optimization During a Call

RCS-con-100 Mobile Originated 1-to-1 Chat – session establishment – Mobile Originated session termination

RCS-con-101 Mobile Originated 1-to-1 Chat – session establishment – Mobile Terminated session termination

RCS-con-103 Mobile Originated 1-to-1 Chat – duplicate session

RCS-con-104 Mobile Originated 1-to-1 Chat – session race condition

RCS-con-105 Mobile Originated 1-to-1 Chat – inactivity timeout

RCS-con-106 Mobile Originated 1-to-1 Chat session – store and forward – sender still in active IM session

RCS-con-107 Mobile Originated 1-to-1 Chat – session store and forward – sender still online

RCS-con-120 Mobile Terminated 1-to-1 Chat – session establishment – Mobile Terminated session termination

RCS-con-121 Mobile Terminated 1-to-1 Chat – session establishment – Mobile Originated session termination

RCS-con-122 Mobile Terminated 1-to-1 Chat – spam/blacklist

RCS-con-123 Mobile Terminated 1-to-1 Chat – display notification

RCS-con-124 Mobile Terminated 1-to-1 Chat – duplicate session

RCS-con-125 Mobile Terminated 1-to-1 Chat – race condition

RCS-con-126 Mobile Terminated 1-to-1 Chat – store and forward

RCS-con-130 Mobile Originated Group Chat – session establishment – Originating User leaves the session

RCS-con-131 Mobile Originated Group Chat – non-originating Users leave the session

RCS-con-132 Mobile Originated Group Chat – adding new User to the session

RCS-con-140 Mobile Terminated Group Chat – session establishment – Terminating User leaves the session

RCS-con-145 Group Chat automatic re-join

RCS-con-146 Group Chat re-start: 404 (Not Found)

RCS-con-147 Group Chat re-start: 403 (Forbidden)

RCS-con-148 Group Chat: Concurrent sessions

Table 1

Applicability

Client ICS

| ICS | Description | Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |
| ics_fileTransfer | Support of file transfer | [RCS 1.2.2] 1.2.2 |  |
| ics_imageShare | Support of in-call image share | [RCS 1.2.2] 1.2.2 |  |
| ics_videoShare | Support of in-call video share | [RCS 1.2.2] 1.2.2 |  |
| ics_request_displayNotifications | Support of requesting display notifications | [RCS 1.2.2] 3.2.2.2 |  |
| ics_stun_udp | Support of STUN when using SIP/UDP | [RCS 1.2.2] 2.8 |  |
| ics_IPVoiceCall | Support of RCS IP Voice Call | [RCS 5.2] 3.8 |  |

Table 2

Client IXIT

| IXIT | Description | Unit <(Range of values)> | Value |
| --- | --- | --- | --- |
| ixit_FTMAXSIZE | Value of FT MAX SIZE | <Any non-zero value>Kbytes |  |
| Ixit_FTWARNSIZE | Value of FT WARN SIZE | <Any non-zero value>Kbytes |  |
| ixit_RF_loss_recognition_ | Time until handset recognizes the RF connection is lost (default is 15) | seconds |  |

Table 3

Server ICS

| ICS | Description | Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |

Table 4

Server IXIT

| IXIT | Description | Unit <(Range of values)> | Value |
| --- | --- | --- | --- |

Table 5

Client ICS to test case mapping

# According to the Client ICS described above the applicable optional Client test cases can be derived from the following table.

| Applicability | Client Test Cases |
| --- | --- |
| ics_imageShare OR ics_videoShare | RCS-con-050 Capability update during a MO call for video /image sharing RCS-con-052 Capability update during a MO voice call fails as other end has no capability |
| ics_request_displayNotifications | RCS-con-102 Mobile Originated 1-to-1 Chat – display notification |
| ics_fileTransfer | RCS-con-200 Mobile Originated File Transfer RCS-con-201 Mobile Originated File Transfer – Rejected RCS-con-210 Mobile Terminated File Transfer RCS-con-211 Mobile Terminated File Transfer – Rejected RCS-con-212 Mobile Terminated File Transfer – File Size Limit (receiver) RCS-con-213 Mobile Terminated File Transfer – File Size Warn Limit (receiver) |
| ics_imageShare | RCS-con-250 Mobile Originated Image Sharing – Session Establishment – Mobile Originated Session Termination RCS-con-251 Mobile Originated Image Sharing – Session Establishment – rejected RCS-con-270 Mobile Terminated Image Sharing – Session Establishment – Mobile Terminated Session Termination RCS-con-271 Mobile Terminated Image sharing – Session Establishment –rejected |
| ics_videoShare | RCS-con-300 Mobile Originated Video Sharing Session Establishment – Mobile Originated Session Termination RCS-con-301 Mobile Originated Video Sharing Session Establishment – Mobile Terminated Session Termination RCS-con-302 Mobile Originated Video Sharing Session Establishment – Rejected RCS-con-320 Mobile Terminated Video Sharing Session Establishment – Mobile Terminated Session Termination RCS-con-321 Mobile Terminated Video Sharing Session Establishment – Mobile Originated Session Termination RCS-con-322 Mobile Terminated Video Sharing Session Establishment – Rejected |
| ics_stun_udp | RCS-con-020 Keep-Alive for SIPoUDP (STUN) |
| ics_IPVoiceCall | RCS-con-400 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination |

Default Procedures

This Appendix provides the details of some re-occurring RCS signalling procedures.

Capability Exchange

Mobile Originated Capability Exchange

- User A’s RCS client sends a SIP OPTIONS (see D.1.3) message towards User B.

- The test tool responds with a 200 OK (D.1.4) towards User A’s RCS client.

Mobile Terminated Capability Exchange

- The test tool sends a SIP OPTIONS (see D.1.3) message towards User A’s RCS client.

- User A’s RCS client responds with a 200 OK (D.1.4) towards User B.

VOID

VOID

Mobile Originated Geolocation information

- User A’s RCS client sends a SIP INVITE (File transfer <<Fetch>>) to User B (Test Tool)

- The test tool responds with a SIP 200 OK towards User A

- The test tool sends the geolocation coordinates via MSRP

- User A’s RCS client responds with a MSRP OK

- User A’s RCS client sends a SIP BYE

- The test tool responds with a SIP 200 OK

Session Establishment

This section SHALL serve with default procedures to establish one of the following session types:

Video Sharing

Image Sharing

File Sharing

Group Chat

IM 1-1 Chat

RCS IP Voice Call

Mobile Originated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat, RCS IP Voice Call

NOTE: For Video share IMS mode or RCS IP Voice Call (according to pc_precondition) execute C.2.1a otherwise for IETF mode execute the following steps.

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards User B to start the session.

- void

- The test tool responds with a 180 RINGING towards User A’s RCS client.

- (1-to-1 chat only) The test tool responds with a SIP MESSAGE (D.1.13) (message delivered) towards User A’s RCS client.

- (1-to-1 chat only) User A’s RCS client responds with a 200 OK (D.1.14) towards User B.

- The test tool responds with a 200 OK (D.1.2) towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards User B.

- C.2.1a		Mobile Originated Session Establishment: Video share (IMS mode)

Execute the message flow of 3GPP 34.229-1 C.21 with the following exceptions:

- Use the message content from D.1.1 and D.1.2.

Mobile Terminated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat

- The test tool sends a SIP INVITE message (D.1.1) towards User A’s RCS client to start the session.

- (Optional) User A’s RCS client responds with a 100 TRYING.

- (Optional for Image and Video share) User A’s RCS client responds with a 180 RINGING.

- (1-to-1 chat only) User A’s RCS client sends a SIP MESSAGE (D.1.13) (message delivered) towards the test tool.

- (1-to-1 chat only) The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- User A accepts the incoming session request.

- User A’s RCS client responds with a 200 OK (D.1.2).

- The test tool sends a SIP ACK message.

Mobile Originated Session Establishment: 1-to-1 Chat with display notification

- Repeat steps 1 to 7 of C.2.1 Mobile Originated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat.

Mobile Terminated Session Establishment: 1-to-1 Chat with display notification

- Repeat steps 1 to 8 of C.2.1 Mobile Originated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat.

Mobile Originated Session Establishment: 1-to-1 Chat with two messages

- Repeat steps 1 to 5 of C.2.1 Mobile Originated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat with the first message.

- User A’s RCS client sends a SIP INVITE message (D.1.1) with the second message towards User B.

- The test tool responds with a SIP 183 SESSION PROGRESS towards User A’s RCS client.

- The test tool responds with a 180 RINGING towards User A’s RCS client.

- The test tool responds with a SIP MESSAGE (D.1.13) (second message delivered) towards User A’s RCS client.

- User A’s RCS client responds with a 200 OK (D.1.14) towards User B.

- The test tool responds with a 486 BUSY HERE (D.1.9) for the first INVITE towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards User B.

- The test tool responds with a 200 OK (D.1.2) for the second INVITE towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards User B.

Mobile Terminated Session Establishment: 1-to-1 Chat with two messages

- Repeat steps 1 to 5 of C.2.2 Mobile Terminated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat with the first message.

- (User A ignores the incoming session request.)

- The test tool sends a SIP INVITE message (D.1.1) with the second message towards User A’s RCS client.

- (Optional) User A’s RCS client responds with a 100 TRYING.

- User A’s RCS client responds with a 180 RINGING.

- User A’s RCS client sends a SIP MESSAGE (D.1.13) (second message delivered) towards the test tool.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- User A’s RCS client sends a 486 BUSY HERE (D.1.9) for the first INVITE towards the test tool.

- The test tool sends a SIP ACK message.

- User A accepts the incoming session request.

- User A’s RCS client responds with a 200 OK (D.1.2) for the second INVITE towards the test tool.

- The test tool sends a SIP ACK message.

Mobile Terminated Session Establishment: Spam

- Repeat steps 1 to 5 of C.2.2 Mobile Terminated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat.

- User A’s RCS client responds with a 486 BUSY HERE (D.1.9).

- The test tool sends a SIP ACK message.

Mobile Originated Session Establishment: simultaneous INVITEs

- User A’s RCS client sends a SIP INVITE message (D.1.1) with message one towards User B to start the session.

- The test tool responds with a SIP 183 SESSION PROGRESS towards User A’s RCS client.

- The test tool responds with a 180 RINGING towards User A’s RCS client.

- The test tool sends a SIP INVITE message (D.1.1) with message two towards User A’s RCS client.

- (Optional) User A’s RCS client responds with a 100 TRYING.

- User A’s RCS client responds with a 180 RINGING.

- User A’s RCS client responds with a 486 BUSY HERE (D.1.9).

- The test tool sends a 486 BUSY HERE (D.1.9) towards User A’s RCS client.

- The test tool sends a SIP ACK message towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards User B.

- User A’s RCS client sends a SIP MESSAGE (D.1.13) (message two delivered) towards the test tool.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- The test tool responds with a SIP MESSAGE (D.1.13) (message one delivered) towards User A’s RCS client.

- User A’s RCS client responds with a 200 OK (D.1.14) towards User B.

Mobile Terminated Session Establishment: new INVITE race

- Repeat steps 1 to 7 of C.2.2 Mobile Terminated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat with the first message.

- The test tool sends a second SIP INVITE message (D.1.1) with the second message towards User A’s RCS client.

- (Optional) User A’s RCS client responds with a 100 TRYING.

- User A’s RCS client responds with a 180 RINGING.

- The test tool sends a SIP ACK message for the first session established in step 1.

- MSRP session is established

- User A’s RCS client sends a SIP BYE (D.1.12) for the session established in step 1.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- User A’s RCS client sends a SIP MESSAGE (D.1.13) (second message delivered) towards the test tool.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- User A’s RCS client responds with a 200 OK (D.1.2).

- The test tool sends a SIP ACK message for the second session established.

Mobile Originated Session: Store and Forward – Receiver offline

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards User B to start the session.

- The test tool responds with a SIP 183 SESSION PROGRESS towards User A’s RCS client.

- The test tool responds with a 200 OK (D.1.2) towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards User B.

Mobile Originated Session: Store and Forward – deferred delivery

- The test tool sends a SIP INVITE message (D.1.1) towards User A’s RCS client.

- Client A auto-accepts the incoming session request.

- User A’s RCS client responds with a 200 OK (D.1.2).

- The test tool sends a SIP ACK message.

Mobile Terminated Session: Store and Forward – deferred delivery

- Repeat steps 1 to 12 of C.2.6 Mobile Terminated Session Establishment: 1-to-1 Chat with two messages.

Mobile Originated Session Establishment: Group Chat

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards the IMS core (test tool) to start the group session.

- The test tool responds with a 200 OK (D.1.2) towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards the IMS core (test tool).

- void

- User A’s RCS client sends a SIP SUBSCRIBE message (0) towards the IMS core (test tool) to get the participants.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- The test tool sends a SIP NOTIFY (D.1.11) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK message towards the IMS core (test tool).

Mobile Terminated Session Establishment: Group Chat

- The IMS core (test tool) sends a SIP INVITE message (D.1.1) towards User A’s RCS client to invite User A to the group session.

- User A accepts the incoming group session request.

- User A’s RCS client responds with a 200 OK (D.1.2).

- The IMS core (test tool) sends a SIP ACK message.

- User A’s RCS client sends a SIP SUBSCRIBE message (0) towards the IMS core (test tool) to get the participants.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- The test tool sends a SIP NOTIFY (D.1.11) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK message towards the IMS core (test tool).

Group Chat automatic re-join

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards the IMS core (test tool) to re-join the group session using the same Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] as in the previous group session.

- The test tool responds with a 200 OK (D.1.2) towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards the IMS core (test tool).

For [RCS 5.1] the following extra steps are required:

- User A’s RCS client sends a SIP SUBSCRIBE message (0) towards the IMS core (test tool) to get the participants.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- The test tool sends a SIP NOTIFY (D.1.11) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK message towards the IMS core (test tool).

Group Chat re-start: 404 Not Found

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards the IMS core (test tool) to re-join the group session using the same Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] as in the previous group session.

- The test tool responds with a 404 Not Found (D.1.5) towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards the IMS core (test tool).

Group Chat re-start: 403 Forbidden

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards the IMS core (test tool) to re-join the group session using the same Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] as in the previous group session.

- The test tool responds with a 403 Forbidden with the warning text set to “127 Service not authorised” (D.1.8) towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards the IMS core (test tool).

Group Chat auto-start

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards the IMS core (test tool) to auto-start the group session using the same Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] and participant list as in the previous group session.

- Execute steps 2 to 8 of “Mobile Originated Session Establishment: Group Chat” (C.2.13).

Group Chat concurrent re-start: Mobile Terminated

- The IMS core (test tool) sends a SIP INVITE message (D.1.1) towards User A’s RCS client to invite User A to the group session using the same Contribution ID [RCS 1.2.2] or Group Chat ID [RCS 5.1] as in the previous group session but a new session ID (session 2).

Auto-accept Group Chat

- Client A auto-accepts the incoming group session request.

- User A’s RCS client responds with a 200 OK (D.1.2).

- The IMS core (test tool) sends a SIP ACK message.

Auto-invite Group Chat

- Client A auto-invites the new User to the Group Chat

- Execute “Inviting new User to Group Chat” (C.5.2)

Group Chat Session Tear Down

- The IMS core (test tool) sends a SIP BYE message with Reason: SIP; cause=480; text=“Temporarily Unavailable”.

- User A’s RCS client responds with a 200 OK (D.1.2).

- The IMS core (test tool) sends a SIP NOTIFY towards User A’s RCS client with state=“terminated”.

- User A’s RCS client responds with a 200 OK (D.1.2).

Group Chat re-start: Mobile Terminated

- The IMS core (test tool) sends a SIP INVITE message (D.1.1) towards User A’s RCS client to invite User A to the group session using the same Group Chat ID as in the previous group session.

Mobile Originated Session Establishment: Video share, Image share, File share, Group Chat

- User A’s RCS client sends a SIP INVITE

- The IMS core (test tool) responds with a SIP 200 OK

- User A’s RCS client sends MSRP SEND

- IMS core (test tool) sends a MSRP 200 OK

Mobile Terminated File Transfer – Store and Forward Receiving

- The test tool sends a cancellation (protocol-cause 408 in the Reason header field indicating that store and forward took place) to User A.

- User A’s RCS client sends a SIP INVITE (Fetch, FT ID, filename)

- The test tool responds with 200 OK (D.1.14)

- User A’s RCS client sends a SIP ACK message towards the FTF and the MSRP file transfer is started

Mobile Originated Session Establishment: RCS IP Video Call (Duplex Mode)

- User A’s RCS client sends a SIP INVITE (SDP audio/video session) to User B (Test Tool)

- User B (Test tool) accepts with a SIP 200 OK (SDP audio/video session)

- User A’s RCS client sends a ACK

- User A’s RCS client sends a RTP (video data) stream

- User B (Test Tool) RCS client sends a RTP (video data) stream

- User A’s RCS client sends a RTP (audio data) stream

- User B (Test Tool) RCS client sends a RTP (audio data) stream

This results in two bidirectional RTP/RTCP streams, one for the audio and one for the video.

Mobile Originated Session Establishment: RCS IP Video Call (Simplex Mode)

- User A’s RCS client sends a SIP INVITE (SDP audio/video session) to User B (Test Tool)

- User B (Test tool) accepts with a SIP 200 OK (SDP audio/”recvonly” video session)

- User A’s RCS client sends a ACK

- User A’s RCS client sends a RTP (video data) stream

- User B (Test Tool) RCS client sends a RTP (audio data) stream

- User A’s RCS client sends a RTP (audio data) stream

- User A’s RCS client sends a RTP (video data) stream

- User B (Test Tool) RCS client sends a RTP (audio data) stream

This results in two RTP/RTCP streams, one bidirectional for the audio and one unidirectional (from User A to User B) for the video.

Mobile Terminated Session Establishment: RCS IP Video Call (Duplex Mode)

- User B (Test Tool) sends a SIP INVITE (SDP audio/video session) towards User A

- User A accepts sending video back: SIP 200 OK (SDP audio/video session)

- User B (Test Tool) sends a ACK

- User B (Test Tool) sends RTP (video data)

- User A’s RCS client sends a RTP (video data) stream

- User B (Test Tool) RCS client sends a RTP (audio data) stream

- User A’s RCS client sends a RTP (audio data) stream

This results in two bidirectional RTP/RTCP streams, one for the audio and one for the video.

Mobile Terminated Session Establishment: RCS IP Video Call (Simplex Mode)

- User B (Test Tool) sends a SIP INVITE (SDP audio/video session) towards User A

- User A accepts without sending video back: SIP 200 OK (SDP audio/”recvonly” video session)

- User B (Test Tool) sends a ACK

- User B (Test Tool) sends RTP (video data)

- User A’s RCS client sends a RTP (audio data)

- User B (Test Tool) sends RTP (audio data)

- User B (Test Tool) sends RTP (video data)

This results in two RTP/RTCP streams, one bidirectional for the audio and one unidirectional (from User B to User A) for the video.

Mobile Originated Session File Transfer BlockedGroup Chat

BPEF (Test Tool) sends a 403 Forbidden with a warning header set to “122 Function not allowed”.

Session Termination

Mobile Originated Session Termination: Video share, Image share, File share, 1-to-1 sessions, RCS IP Voice Call

- User A’s RCS client sends a SIP BYE message towards User B to terminate the session.

- The test tool responds with 200 OK (D.1.14) towards User A’s RCS client.

Mobile Terminated Session Termination: Video share, Image share, File share, 1-to-1 sessions

- The test tool sends a SIP BYE message towards User A’s RCS client to terminate the session.

- User A’s RCS client responds with a 200 OK (D.1.14).

Mobile Originated Session Termination: Group chat

- User A’s RCS client sends a SIP BYE message towards the IMS core (test tool) to leave the group session.

- The IMS core (test tool) responds with 200 OK (D.1.14) towards User A’s RCS client.

Mobile Terminated Session Termination: Group chat

- The IMS core (test tool) sends a SIP BYE message towards User A’s RCS client to terminate the group session.

- User A’s RCS client responds with a 200 OK (D.1.14).

Mobile Originated Session Termination: Video share, Image share, File share, Group sessions

- User A´s client sends a SIP BYE message towards User B and User C to terminate the session

- The test tool responds with a 200 OK (D.1.14)

Mobile Originated Session Termination: File Transfer Store and Forward

- User A´s client sends a SIP BYE message towards the FTF to terminate the session

- The test tool responds with a 200 OK (D.1.14)

- User A´s client sends a SIP MESSAGE (CPIM/IMDN (MESSAGE-ID))

- The test tool responds with a SIP 200 OK (D.1.14)

Session Rejection

Mobile Originated Session — Reject

- User A’s RCS client sends a SIP INVITE message (D.1.1) towards User B to start the session.

- The test tool responds with a SIP 183 SESSION PROGRESS towards User A’s RCS client.

- The test tool responds with a 180 RINGING towards User A’s RCS client.

- The test tool responds with a 603 DECLINE towards User A’s RCS client.

- User A’s RCS client sends a SIP ACK message towards User B.

Mobile Terminated Session — Reject

- The test tool sends a SIP INVITE message (D.1.1) towards User A’s RCS client to start the session.

- (Optional) User A’s RCS client responds with a 100 TRYING.

- (Optional for Image and Video share) User A’s RCS client responds with a 180 RINGING.

- User A rejects the incoming session request.

- User A’s RCS client responds with a 603 DECLINE.

- The test tool sends a SIP ACK message.

Mobile Originated RCS IP Video Call Session – Rejected

- User A’s RCS client sends a SIP INVITE (SDP video session) towards User B (test tool) to start the session.

- The test tool resoinds with a SIP 603 DECLINE towards User A’s RCS client.

- User A’s RCS client sends a ACK message towards User B.

Mobile Terminated RCS IP Video Call Session – Rejected

- The test tool sends a SIP INVITE (SDP video session) towards User A’s RCS client to start the session.

- User A rejects the incoming session request.

- User A’s RCS client responds with a 603 DECLINE.

- The test tool sends a SIP ACK message towards User B.

Mobile Originated IP Video Call Upgrade from CS voice call decline

- The active CS Voice call is terminated by User A

- User A sends a SIP INVITE (SDP audio/video session) to User B (Test Tool)

- User B rejects with SIP 603 Decline

- User A sends a ACK message.

Mobile Terminated IP Video Call Upgrade from CS voice call decline

- The active CS Voice call is terminated by the test tool.

- User B (Test Tool) sends a SIP INVITE (SDP audio/video session) to User A.

- User A rejects with SIP 603 Decline.

- User B sends a ACK message.

Other Procedures

Participant List Update

- The IMS core (test tool) sends a SIP NOTIFY message (D.1.11) towards User A’s RCS client with new participants’ status.

- User A’s RCS client responds with a 200 OK (D.1.14).

Inviting new User to Group Chat

- User A’s RCS client sends a SIP REFER message (0) towards the IMS core (test tool) to invite a new User to the group session.

- The IMS core (test tool) responds with a 202 ACCEPTED (D.1.16) towards User A’s RCS client.

- Wait 1 second

- The IMS core (test tool) sends a SIP NOTIFY (D.1.11) towards User A’s RCS client with the list of participants.

- User A’s RCS client responds with a SIP 200 OK (D.1.14) message towards the IMS core (test tool).

Get Participant List

- User A’s RCS client sends a SIP SUBSCRIBE message (0) towards the IMS core (test tool) to get the participants.

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- The test tool sends a SIP NOTIFY (D.1.11) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK message towards the IMS core (test tool).

Inviting new User to Closed Group Chat

- User A’s RCS client sends a SIP REFER message (0) towards the IMS core (test tool) to invite a new User to the group session.

- The IMS core (test tool) responds with a SIP 486 ERROR towards User A’s RCS client.

Mobile Originated Pager Mode Message

- User A’s RCS client sends a SIP MESSAGE message (D.1.13) towards the IMS core (test tool).

- The IMS core (test tool) responds with a SIP 200 OK towards User A’s RCS client.

Mobile Terminated Pager Mode Message

- The IMS core (test tool) sends a SIP MESSAGE message (D.1.13) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK towards User A’s RCS client.

Mobile Originated Delivered Notification

- User A’s RCS client sends a SIP MESSAGE message (D.1.13) towards the IMS core (test tool).

- The IMS core (test tool) responds with a SIP 200 OK towards User A’s RCS client.

Mobile Terminated Delivered Notification

- The IMS core (test tool) sends a SIP MESSAGE message (D.1.13) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK towards User A’s RCS client.

Mobile Originated Displayed Notification

- User A’s RCS client sends a SIP MESSAGE message (D.1.13) towards the IMS core (test tool).

- The IMS core (test tool) responds with a SIP 200 OK towards User A’s RCS client.

Mobile Terminated Displayed Notification

- The IMS core (test tool) sends a SIP MESSAGE message (D.1.13) towards User A’s RCS client.

- User A’s RCS client responds with a SIP 200 OK towards User A’s RCS client.

Mobile Originated Pager Mode Procedure

- Execute C.5.5

- Execute C.5.8

- Execute C.5.10

Mobile Terminated Pager Mode Procedure

- Execute C.5.6

- Execute C.5.7

- Execute C.5.9

Mobile Terminated Geolocation Push

- User B’s RCS client (Test Tool) sends a SIP INVITE

- User A’s RCS client responds with a SIP 200 OK

- User B’s RCS client (Test Tool) sends MSRP SEND

- User A’s RCS client sends a MSRP 200 OK

- User B’s RCS client (Test Tool) sends a shared map with +g.3gpp.icsi-ref=”urn%3Aurn-7%3A3gpp-service.ims.icsi.gsma.sharedmap” as value in Accept-Contact or Contact Header. The value carried in a P-Preferred-Service or P-Asserted-Service header is urn:urn-7:3gpp-service.ims.icsi.gsma.sharedmap and the Service Tuple is org.3gpp.urn:urn-7:3gpp-service.ims.icsi.gsma.sharedmap Version: 1.0

Mobile Terminated Shared Sketch

- User B’s RCS client (Test Tool) sends a SIP INVITE

- User A’s RCS client responds with a SIP 200 OK

- User B’s RCS client (Test Tool) sends MSRP SEND

- User A’s RCS client sends a MSRP 200 OK

- User B’s RCS client (Test Tool) sends a shared sketch with file type jpg or png and +g.3gpp.icsi-ref=”urn%3Aurn-7%3A3gpp-service.ims.icsi.gsma.sharedsketch” as value in Accept-Contact or Contact Header. The value carried in a P-Preferred-Service or P-Asserted-Service header is urn:urn-7:3gpp-service.ims.icsi.gsma.sharedsketch and the Service Tuple is org.3gpp.urn:urn-7:3gpp-service.ims.icsi.gsma.sharedsketch Version: 1.0

Mobile Originated Shared Sketch/Map

- User A’s RCS client sends a SIP INVITE

- User B’s RCS client (test tool) responds with a SIP 200 OK

- User A’s RCS client sends MSRP SEND

- User B’s RCS client (test tool) sends a MSRP 200 OK

Mobile Terminated Shared Map

- User B’s RCS client (Test Tool) sends a SIP INVITE

- User A’s RCS client responds with a SIP 200 OK

- User B’s RCS client (Test Tool) sends MSRP SEND

- User A’s RCS client (test tool) sends a MSRP 200 OK

- User B’s RCS client (Test Tool) sends a shared map with file type jpg or png and +g.3gpp.icsi-ref=”urn%3Aurn-7%3A3gpp-service.ims.icsi.gsma.sharedmap” as value in Accept-Contact or Contact Header. The value carried in a P-Preferred-Service or P-Asserted-Service header is urn:urn-7:3gpp-service.ims.icsi.gsma.sharedmap and the Service Tuple is org.3gpp.urn:urn-7:3gpp-service.ims.icsi.gsma.sharedmap Version: 1.0

Autoconfiguration Procedures

HTTP

Successful autoconfiguration

- User A’s RCS client sends a HTTP request (D.2.1) to the autoconfiguration server (test tool).

- The test tool responds with 200 OK (D.2.2).

- User A’s RCS client sends a HTTPS request to the autoconfiguration server (D.2.3).

- The test tool responds with 200 OK (D.2.4).

MSRP session

MSRP Session Message

- The Originating end sends a MSRP SEND message (D.3.1) to the Terminating end with an “iscomposing” notification.

- The Terminating end responds with MSRP 200 OK (0).

- In the case of the test tool being the Originating end, wait 2 seconds.

- The Originating end sends a MSRP SEND message (D.3.1) to the Terminating end with the required message.

- The Terminating end responds with MSRP 200 OK (0).

- The Terminating end sends a MSRP SEND message (D.3.1) to the Originating end with a “delivered” notification.

- The Originating end responds with MSRP 200 OK (0).

In the case that the Originating end is User A and the MSRP SEND message in step 4 requests a “displayed” notification then the following extra steps are required:

- Wait 2 seconds

- The test tool sends a MSRP SEND message (D.3.1) to User A’s RCS client with a “displayed” notification.

- User A’s RCS client responds with MSRP 200 OK (0).

MSRP Session Message with Display Notification

- The Originating end sends a MSRP SEND message (D.3.1) to the Terminating end with an “iscomposing” notification.

- The Terminating end responds with MSRP 200 OK (0).

- In the case of the test tool being the Originating end, wait 2 seconds

- The Originating end sends a MSRP SEND message (D.3.1) to the Terminating end with the required message.

- The Terminating end responds with MSRP 200 OK (0).

- The Terminating end sends a MSRP SEND message (D.3.1) to the Originating end with a “delivered” notification.

- The Originating end responds with MSRP 200 OK (0).

- In the case of the test tool being the Terminating end, wait 2 seconds

- The Terminating end sends a MSRP SEND message (D.3.1) to the Originating end with a “displayed” notification.

- The Originating end responds with MSRP 200 OK (0).

MSRP Session Store and Forward

- User A’s RCS client sends a MSRP SEND message (D.3.1) to the test tool with an “iscomposing” notification.

- The test tool responds with MSRP 200 OK (0).

- User A’s RCS client sends a MSRP SEND message (D.3.1) to the test tool with the required message.

- The test tool responds with MSRP 200 OK (0).

MSRP Session Store and Forward Message Delivered

- The test tool sends a MSRP SEND message (D.3.1) to User A’s RCS client with a “delivered” notification.

- User A’s RCS client responds with MSRP 200 OK (0).

In the case that the original MSRP SEND with the originating message requested a “displayed” notification then the following extra steps are required:

- Wait 2 seconds

- The test tool sends a MSRP SEND message (D.3.1) to User A’s RCS client with a “displayed” notification.

- User A’s RCS client responds with MSRP 200 OK (0).

MSRP Session Store and Forward Final Message Delivered

- The test tool sends a MSRP SEND message (D.3.1) to User A’s RCS client with a “delivered” notification.

- User A’s RCS client responds with MSRP 200 OK (0).

In the case that the MSRP SEND message previously sent in step 3 of C.7.3 requests a “displayed” notification then the following extra steps 2a, 2b and 2c are required:

2a.	Wait 2 seconds

2b.	The test tool sends a MSRP SEND message (D.3.1) to User A’s RCS client with a “displayed” notification.

2c.	User A’s RCS client responds with MSRP 200 OK (0).

- Execute procedure “Mobile Terminated Session Termination: Video share, Image share, File share, 1-to-1 sessions” (C.3.2).

MSRP Group Session Message

For [RCS 5.1] use C.7.1 MSRP Session Message

- The Originating end sends a MSRP SEND message (D.3.1) to the Terminating end with the required message.

- The Terminating end responds with MSRP 200 OK (0).

MSRP Session Store and Forward Message Reception

- The test tool sends a MSRP SEND message (D.3.1) to User A’s RCS client with the required message.

- User A’s RCS client responds with MSRP 200 OK (0).

- User A’s RCS client sends a MSRP SEND message (D.3.1) with a “delivered” notification.

- The test tool responds with MSRP 200 OK (0).

MSRP Session Store and Forward File Transfer

- User A’s RCS client sends a SIP INVITE (SDP, IMDN) to the test tool

- The test tool responds with SIP 200 OK

- User A´s RCS clients responds with SIP ACK

MSRP Session Store and Forward File Transfer Completed

- User A’s RCS clients sends a SIP BYE to the test tool after the transfer is completed

- The test tool responds with SIP 200 OK

Store and Forward File Transfer Delivered Notification

- The test tool sends a SIP MESSAGE (CPIM/IMDN (Message-ID)) to User A’s RCS client with a “delivered” notification.

- User A’s RCS client responds with SIP 200 OK (0).

PRESENCE based Capability Exchange

Publishing

- User A’s RCS client sends a SIP PUBLISH message (D.1.17) towards the IMS core (test tool).

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

Note: This procedure can be executed as soon as the RCS client is SIP registered (also in parallel to SUBSCRIBE/NOTIFY for reg event)

Subscription to capabilities

- User A’s RCS client sends a SIP SUBSCRIBE message (0) towards the IMS core (test tool).

- The test tool responds with a 200 OK (D.1.14) towards User A’s RCS client.

- The test tool sends a NOTIFY message (D.1.19) towards User A’s RCS client.

- User A’s RCS client responds with a 200 OK (D.1.14) towards the IMS core (test tool).

Note: This procedure can be executed as soon as the RCS client is SIP registered (also in parallel to SUBSCRIBE/NOTIFY for reg event)

Default Message Content

SIP message content

The default message content from 3GPP TS 34.229-1 Appendix A is used per default. This section just modifies or clarifies the default messages specified there.

Editor’s note: Additions for CPM are required in all sections.

INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | B1 | +g.3gpp.cs-voice |  |  |
|  | B2 | +g.3gpp.iari-ref=”urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-is” |  |  |
|  | B3 | +g.oma.sip-im |  |  |
|  | B4, B5 | +g.oma.sip-im |  |  |
|  | B6 | +g.gsma.rcs.ipcall |  |  |
| Contact |  |  |  |  |
| feature-param | B1 | +g.3gpp.cs-voice+ |  |  |
|  | B2 | +g.3gpp.iari-ref=”urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-is” |  |  |
|  | B3 | +g.oma.sip-im |  |  |
|  | B4, B5 | +g.oma.sip-im |  |  |
|  | B6 | +g.gsma.rcs.ipcall |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type | B1, B2, B3, B6 | application/sdp |  |  |
|  | B4, B5 | multipart/mixed;boundary=”boundary1” |  |  |
| Supported |  |  |  | RFC 3261 [15] |
|  |  | Precondition is not included |  |  |
| Require |  |  |  | RFC 3261 [15] |
|  |  | Precondition is not included |  |  |
| Message-body | B1 | m=video port RTP/AVP (rtp payload types) a=sendonly a=rtpmap: payload type H264/90000 a=fmtp: payload type profile-level-id=42C00D; packetization-mode=0 a=rtpmap: payload type H263-2000/90000 a=framesize: payload type 176-144 a=framerate:8 a=fmtp: payload type profile=0; level=45 |  | 4119 [99RFC] |
|  | B2 | m=message port TCP/MSRP * a=sendonly a=accept-types:message/cpim a=accept-wrapped-types:* a=path:msrp://( IP address):port/jshA7we;tcp a=file-selector: type:image/jpeg sizesize of file) a=file-transfer-id:Q6LMoGymJdh0IKIgD6wD0jkcfgva4xvE |  |  |
|  | B3 | m=message port TCP/MSRP * a=sendonly a=accept-types:message/cpim a=accept-wrapped-types:* a=path:msrp://( IP address):port/jshA7we;tcp a=file-selector: type:image/jpeg size:(size of file) a=file-transfer-id:Q6LMoGymJdh0IKIgD6wD0jkcfgva4xvE |  |  |
|  | B4 | --boundary1 Content-Type: application/sdp m=message port TCP/MSRP * a=accept-types: message/cpim text/plain [RCS 1.2.2] a=accept-types: message/cpim [RCS 5.1] a=accept-wrapped-types:* [RCS 5.1] if multimedia in Chat supported a=accept-wrapped-types: text/plain message/imdn+xml application/im-iscomposing+xml [RCS 5.1] if multimedia in Chat not supported.Add application/vnd.gsma.rcs-ft-http+xml if File Transfer using HTTP is supported. Add application/vnd.gsma.rcspushlocation+xml if Geolocation PUSH is supported a=max-size: 7665 a=path:msrp://( IP address):port/jshA7we;tcp --boundary1 Content-Type: message/cpim From: <sip:anonymous@anonymous.invalid> To: <sip:anonymous@anonymous.invalid> DateTime: 2014-01-15T10:17:49.727+01:00 NS: imdn <urn:ietf:params:imdn> imdn.Message-ID: 7QsgkAl8QZvr9XPFtiiCtLj3sls0oNok imdn.Disposition-Notification: positive-delivery, display Content-type: text/plain;charset=UTF-8 Initial message --boundary1-- |  |  |
|  | B5 | --boundary1 Content-Type: application/sdp <B4 content> --boundary1 Content-Type: application/resource-lists+xml Content-Disposition: recipient-list <?xml versio”="1”0" encodin”="UTF”8"?> <resource-lists xmln”="urn:ietf:params:xml:ns:resource-lis”s" xmlns:c”="urn:ietf:params:xml:ns:copycontr”l"> <list> <entry ur”="<SIP or TEL URI of User ”>" cp:copyContro”="”o"/> … (NOTE: Add further users as necessary) </list> </resource-lists> --boundary1-- |  |  |
|  | B6 | m=audio (transport port) RTP/AVP (fmt) b=AS: (bandwidth-value) b=RS:0 b=RR:0 a=rtpmap:(payload type) AMR/8000/1 a=fmtp:(format) mode-change-capability=2; max-red=220 a=rtpmap:(payload type) telephone-event a=ptime:20 a=maxptime:240 |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | Video Sharing (ics_videoShare) |
| B2 | Image Sharing (ics_imageShare) |
| B3 | File Sharing (ics_fileTransfer) |
| B4 | IM Chat (1 to 1 session) |
| B5 | IM Group chat |
| B6 | RCS IP Voice Call |

200 OK for INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Contact |  |  |  |  |
| feature-param | B1 | +g.3gpp.cs-voice |  |  |
|  | B2 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-”s" |  |  |
|  | B3 | +g.oma.sip-im |  |  |
|  | B4 | +g.oma.sip-im |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Message-body | B1 | m=video port RTP/AVP (rtp payload types) a= recvonly a=rtpmap: payload type H264/90000 a=fmtp: payload type profile-level-id=42C00D; packetization-mode=0 a=rtpmap: payload type H263-2000/90000 a=framesize: payload type 176-144 a=framerate:8 a=fmtp: payload type profile=0; level=45 |  | RFC 4119 [99] |
|  | B2 | m=message port TCP/MSRP * a=recvonly a=accept-types:message/cpim a=accept-wrapped-types:* a=path:msrp://( IP address):port/jshA7we;tcp a=file-selector: (copied from INVITE) a=file-transfer-id: (copied from INVITE) |  |  |
|  | B3 | m=message port TCP/MSRP * a=recvonly a=accept-types:message/cpim a=accept-wrapped-types:* a=path:msrp://( IP address):port/jshA7we;tcp a=file-selector: (copied from INVITE) a=file-transfer-id: (copied from INVITE) |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | Video Sharing (ics_videoShare) |
| B2 | Image Sharing (ics_imageShare) |
| B3 | File Sharing (ics_fileTransfer) |
| B4 | IM Chat |

OPTIONS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request URI | B5 B6 | User-B’s IMS identity (SIP-URI or TEL-URI/MSISDN) User-A’s IMS contact-URI |  |  |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | B1 | +g.3gpp.cs-voice |  |  |
|  | B2 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-”s" |  |  |
|  | B3 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.”t";+g.oma.sip-im (optional) |  |  |
|  | B4 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.”m";+g.oma.sip-im (optional) |  |  |
| Contact |  |  |  |  |
| feature-param | B1 | +g.3gpp.cs-voice+ |  |  |
|  | B2 | g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-”s" |  |  |
|  | B3 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.”t";+g.oma.sip-im (optional) |  |  |
|  | B4 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.”m";+g.oma.sip-im (optional) |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type | B1 | application/sdp |  |  |
| Message-body | B1 | supported video codecs. m=video 0 RTP/AVP (rtp payload types) a=rtpmap: payload type H264/90000 a=rtpmap: payload type H263-2000/90000 |  | RFC 4119 [99] [RCS 1.2.2] 2.7.3 |
|  | Other cases | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | Video Sharing (ics_videoShare) AND in CS Call |
| B2 | Image Sharing (ics_imageShare) AND in CS Call |
| B3 | File Sharing (ics_fileTransfer) |
| B4 | IM Chat |
| B5 | Mobile originated |
| B6 | Mobile terminated |
| Note: If more than one of the above conditions evaluate to true and therefore multiple IARI tags are included, these must be combined in a comma separated list, as defined in [RCS 1.2.2] section 2.3.1.1. For example: +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.im,urn%3Aurn-7%3A3gppapplication.ims.iari.rcse.”t" |  |

200 OK for OPTIONS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Contact |  |  |  |  |
| feature-param | B1 | +g.3gpp.cs-voice |  |  |
|  | B2 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-”s" |  |  |
|  | B3 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.”t";+g.oma.sip-im (optional) |  |  |
|  | B4 | +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.”m";+g.oma.sip-im (optional) |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type | B1 | application/sdp |  |  |
| Message-body | B1 | supported video codecs. m=video 0 RTP/AVP (rtp payload types) a=rtpmap: payload type H264/90000 a=rtpmap: payload type H263-2000/90000 |  | RFC 4119 [99] |
|  | Other cases | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | Video Sharing (ics_videoShare) AND in CS Call |
| B2 | Image Sharing (ics_imageShare) AND in CS Call |
| B3 | File Sharing (ics_fileTransfer) |
| B4 | IM Chat |
| Note: If more than one of the above conditions evaluate to true and therefore multiple IARI tags are included, these must be combined in a comma separated list, as defined in [RCS 1.2.2] section 2.3.1.1. For example: +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.im,urn%3Aurn-7%3A3gppapplication.ims.iari.rcse.”t" |  |

404 Not found

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 404 |  |  |
| Reason-Phrase | Not Found |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

408 Request Timeout

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 408 |  |  |
| Reason-Phrase | Request Timeout |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

480 Temporarily Unavailable

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 480 |  |  |
| Reason-Phrase | TEMPORARILY UNAVAILABLE |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

403 Forbidden

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 403 |  |  |
| Reason-Phrase | FORBIDDEN |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

486 BUSY HERE

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 486 |  |  |
| Reason-Phrase | BUSY HERE |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

SUBSCRIBE (conference)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request URI |  | Conference URI |  |  |
| Event |  | conference |  |  |

NOTIFY (conference)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request URI |  |  |  |  |
| Contact |  |  |  | RFC 3261[15] |
| addr-spec |  | <px_FinalConferenceUri>;isfocus |  |  |
| Content-Type |  | application/conference-info+xml |  |  |
| Event |  |  |  | RFC 3265[34] RFC 4575[86] |
| event-type |  | conference |  |  |
| Subscription-State |  | active ;expires=7200 |  |  |
|  |  | <?xml version="1.0" encoding="UTF”8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info" entity="conference1@mrfc2.home1.net" state="full" version=”0" > <conference-description> <display-text>OMA Group Chat</display-text> <maximum-user-count>11</maximum-user-count> </conference-description> <conference-state> <user-count>1</user-count> <active>true</active> <locked>false</locked> </conference-state> <users> <user entity="<User SIP URI>" state="full" yourown="true"> <display- text>User 1</display-text> <roles> <entry>chairman</entry> </roles> <endpoint entity=" sip:[5555::eee:fff:aaa:bb”]"> <display-text>User 1</display-text> <status>connected</status> </endpoint> <etype>gpmanage</etype> </user> (NOTE: Add further users as necessary) <user entity="<User SIP URI>" state="full" yourown="true"> <display- text>User 2</display-text> <roles> <entry>participant</entry> </roles> <endpoint entity=" sip:[5555::eee:fff:aaa:bb”]"> <display-text>User 2</display-text> <status>connected</status> </endpoint> <etype>gpmanage</etype> </user> </users> </conference-info> |  |  |

BYE

See 3GPP 34.229-1 A.2.8

MESSAGE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request URI |  |  |  |  |
| Accept-Contact |  | +g.oma.sip-im |  |  |
| Content-type |  | message/cpim |  |  |
|  |  | From: Bob <im:bob@example.com> To: Alice <im:alice@example.com> NS: imdn <urn:ietf:params:imdn> imdn.Message-ID: d834jied93rf Content-type: message/imdn+xml Content-Disposition: notification Content-length: ... <?xml versio”="1”0" encodin”="UTF”8"?> <imdn xmln”="urn:ietf:params:xml:ns:im”n"> <message-id>34jk324j</message-id> <datetime>2008-04-04T12:16:49-05:00</datetime> <recipient-uri>im:bob@example.com</recipient-uri> <original-recipient-uri >im:bob@example.com</original-recipient-uri> <delivery-notification> <status> <delivered/> </status> </delivery-notification> </imdn> |  |  |

200 OK (Generic)

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 200 |  |  |
| Reason-Phrase | OK |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

REFER

See 3GPP 34.229-1 A.2.10

202 ACCEPTED

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Status-Line |  |  | RFC 3261 [15] |
| SIP-Version | SIP/2.0 |  |  |
| Status-Code | 202 |  |  |
| Reason-Phrase | ACCEPTED |  |  |
| Via |  |  | RFC 3261 [15] |
| via-parm | same value as received in request message |  |  |
| From |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as received in request message |  |  |
| To |  |  | RFC 3261 [15] |
| addr-spec | same value as received in request message |  |  |
| tag | same value as in an earlier responses sent to the UE for this dialog |  |  |
| Call-ID |  |  | RFC 3261 [15] |
| callid | same value as received in request message |  |  |
| Session-ID |  |  | draft-kaplan-dispatch-session-id [115] |
| sess-id | same value as received in request message, if Session-ID header field exists in received request message, otherwise, not present. |  |  |
| CSeq |  |  | RFC 3261 [15] |
| value | same value as received in request message |  |  |
| Content-Length |  |  | RFC 3261 [15] |
| value | 0 |  |  |

PUBLISH

| Header/param | Cond | Value/remark |  | Rel | Reference |
| --- | --- | --- | --- | --- | --- |
| Request URI |  |  |  |  |  |
| Contact |  |  |  |  |  |
| feature-param | B1 |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.session” |  |  |
|  | B2 |  | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.oma.cpm.filetransfer" |  |  |
|  | B3 |  | +g.3gpp.iari-ref="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.dp " |  |  |
|  | B4 |  | +g.3gpp.iari-ref="urn%3Aurn-7%3A3gpp-application.ims.iari.rcs.ftthumb" |  |  |
| Content-Type |  | application/pidf+xml |  |  |  |
| Event |  | presence |  |  |  |
| Expires |  | Any value |  |  |  |
|  |  | <?xml version=”1.0” encoding=”UTF-8”?> <presence xmlns=”urn:ietf:params:xml:ns:pidf” xmlns:op=”urn:oma:xml:prs:pidf:oma-pres” xmlns:opd=”urn:oma:xml:pde:pidf:ext” xmlns:opd11=”urn:oma:xml:pde:pidf:ext:1.1” xmlns:pdm=”urn:ietf:params:xml:ns:pidf:data-model” xmlns:rpid=”urn:ietf:params:xml:ns:pidf:rpid” xmlns:gp=”urn:ietf:params:xml:ns:pidf:geopriv10” xmlns:caps="urn:ietf:params:xml:ns:pidf:caps" xmlns:gml="http://www. opengis.net/gml"xmlns:gs="http://www.opengis.net/pidflo/1.0" entity=”<IMPU>”> <tuple id=”any”><!—multiple elements --> <status><basic>open</basic></status> <op:service-description> <op:service-id><SERVICEID></op:service-id> <op:version>1.0</op:version> </op :service-description> <contact><IMPU></contact> </tuple> </presence> |  |  | RRC.07 2.6.1.2.5.1 |

| Condition | SERVICEID | Explanation |
| --- | --- | --- |
| B1 |  | CPM |
| B2 | org.openmobilealliance:File-Transfer | File Sharing (ics_fileTransfer) |
| B3 | org.3gpp.urn:urn-7:3gpp-application.ims.iari.rcse.dp | Presence for capability discovery |
| B4 | org.openmobilealliance:File-Transfer-thumb | Filetransfer Thumbnail |
|  | org.openmobilealliance:IM-session |  |
|  | org.openmobilealliance:ChatSession |  |
|  | org.openmobilealliance:StandaloneMsg |  |
|  | org.openmobilealliance:File-Transfer-HTTP | HTTP Filetransfer |
|  | org.3gpp.urn:urn-7:3gpp-application.ims.iari.rcs.geopush | Geolocation PUSH |
|  | org.3gpp.urn:urn-7:3gpp-application.ims.iari.rcs.geopullft | Geolocation PULL |
| Note: If more than one of the above conditions evaluate to true and therefore multiple IARI tags are included, these must be combined in a comma separated list, as defined in [RCS 1.2.2] section 2.3.1.1. For example: +g.3gpp.iari-re”="urn%3Aurn-7%3A3gpp-application.ims.iari.rcse.im,urn%3Aurn-7%3A3gppapplication.ims.iari.rcse.”t" |  |  |

SUBSCRIBE (presence)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request URI |  | Target URI |  |  |
| Event |  | presence |  |  |

NOTIFY (presence)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request URI |  |  |  |  |
| Event |  | presence |  |  |
| Content-Type |  | application/pidf+xml |  |  |
|  |  | tbd |  |  |

HTTP autoconfiguration message content

Initial HTTP request

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request Line |  |  |  |  |
| Method URI |  | GET / and no parameters |  |  |
| Message-Body |  | none |  |  |

200 OK for the initial HTTP request

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status Line |  |  |  |  |
| Status |  | 200 OK |  |  |
| Set-Cookie |  | Random value |  |  |
| Message-Body |  | none |  |  |

Initial HTTPS request

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request Line |  |  |  | [RCS 1.2.2] sect. 2.2.2.1.2 |
| Method URI Parameter vers IMSI client_vendor client_version terminal_vendor terminal_model terminal_sw_version IMEI | B1 B1 | GET / Int (-1, 0 or a positive integer) 15 digit string 4 letter string Max 10 letter string 4 letter string Max 10 letter string Max 10 letter string Max 15 letter string |  |  |
| Cookie |  | Same as in 200 OK for HTTP |  |  |
| Message-Body |  | None |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | OS provides access to IMSI and IMEI (ics_os_supports_imsi_imei) |

200 OK for the initial HTTPS request

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status Line |  |  |  |  |
| Status |  | 200 OK |  |  |
| Message-Body |  | <?xml version="1.0"?> <wap-provisioningdoc version="1.1"> <characteristic type="VERS"> <parm name=”version” value=”<vers from request incremented by 1>”/> <parm name=”validity” value=”1728000”/> </characteristic> <characteristic type="APPLICATION"> See D.2.5 </characteristic> </wap-provisioningdoc> |  |  |

Default provisioning XML [RCS 1.2.2]

<characteristic type="ConRefs">

<parm name="ConRef" value="X"/>

</characteristic>

<characteristic type="Public_User_Identity_List">

<parm name="Public_User_Identity" value="px_IMS_PublicUserIdentity1"/>

</characteristic>

<characteristic type="Ext">

<characteristic type="SecondaryDevicePar">

<parm name="VoiceCall" value="0"/>

<parm name="Chat" value="0"/>

<parm name="SendSms" value="0"/>

<parm name="FileTranfer" value="0"/>

<parm name="VideoShare" value="0"/>

<parm name="ImageShare" value="0"/>

</characteristic>

<parm name="NatUrlFmt" value="1"/>

<parm name="IntUrlFmt" value="1"/>

<parm name="Q-Value" value="0.5"/>

<parm name="MaxSizeImageShare" value="0"/>

<parm name="MaxTimeVideoShare" value="0"/>

</characteristic>

<characteristic type="ICSI_List"/>

<characteristic type="LBO_P-CSCF_Address">

<parm name="Address" value="derived from px_P_CSCF_IPAddr"/>

<parm name="AddressType" value="IPvx derived from px_P_CSCF_IPAddr "/>

</characteristic>

<characteristic type="PhoneContext_List"/>

<characteristic type="APPAUTH">

<parm name="AuthType" value="derived from pc_IMS_Sec, pc_IMS_GIBA_Sec and pc_IMS_Digest_sec "/>

</characteristic>

<parm name="AppID" value="ap2001"/>

<parm name="Name" value="IMS Settings"/>

<parm name="AppRef" value="IMS-Settings"/>

<parm name="PDP_ContextOperPref" value="X"/>

<parm name="Timer_T1" value="2000"/>

<parm name="Timer_T2" value="16000"/>

<parm name="Timer_T4" value="17000"/>

<parm name="Private_User_Identity" value="px_IMS_Private_UserId"/>

<parm name="Home_network_domain_name" value="px_IMS_HomeDomainName"/>

<parm name="Voice_Domain_Preference_E_UTRAN" value="4"/>

<parm name="SMS_Over_IP_Networks_Indication" value="1"/>

<parm name="Keep_Alive_Enabled" value="0"/>

<parm name="Voice_Domain_Preference_UTRAN" value="2"/>

<parm name="Mobility_Management_IMS_Voice_Termination" value="0"/>

<parm name="RegRetryBaseTime" value="0"/>

<parm name="RegRetryMaxTime" value="0"/>

</characteristic>

<characteristic type="APPLICATION">

<characteristic type="IMS">

<parm name="To-AppRef" value="IMS-Settings"/>

</characteristic>

<characteristic type="PRESENCE">

<characteristic type="FAVLINK">

<parm name="AutMa" value="X"/>

</characteristic>

<characteristic type="SERVCAPWATCH">

<parm name="FetchAuth" value="X"/>

<parm name="ContactCapPresAut" value="X"/>

</characteristic>

<characteristic type="ServCapPresentity">

<parm name="WATCHERFETCHAUTH" value="X"/>

</characteristic>

<parm name="usePresence" value="0"/>

<parm name="presencePrfl" value="X"/>

<parm name="AvailabilityAuth" value="X"/>

<parm name="IconMaxSize" value="X"/>

<parm name="NoteMaxSize" value="X"/>

<parm name="PublishTimer" value="X"/>

<parm name="client-obj-datalimit" value="X"/>

<parm name="content-serveruri" value="X"/>

<parm name="source-throttlepublish" value="X"/>

<parm name="max-number-ofsubscriptions-inpresence-list" value="X"/>

<parm name="service-uritemplate" value="X"/>

</characteristic>

<characteristic type="XDMS">

<parm name="RevokeTimer" value="X"/>

<parm name="XCAPRootURI" value="X"/>

<parm name="XCAPAuthenticationUserName" value="X"/>

<parm name="XCAPAuthenticationSecret" value="X"/>

<parm name="XCAPAuthenticationType" value="X"/>

</characteristic>

<characteristic type="IM">

<parm name="imCapAlwaysON" value="1"/>

<parm name="imWarnSF" value="1"/>

<parm name="ftWarnSize" value="ixit_FTWARNSIZE"/>

<parm name="ftAutAccept" value="1"/>

<parm name="ChatAuth" value="1"/>

<parm name="SmsFallBackAuth" value="0"/>

<parm name="AutAccept" value="1"/>

<parm name="AutAcceptGroupChat" value="1"/>

<parm name="MaxSize1to1" value="20000"/>

<parm name="MaxSize1toM" value="20000"/>

<parm name="TimerIdle" value="10"/>

<parm name="MaxSizeFileTr" value="ixit_FTMAXSIZE"/>

<parm name="pres-srv-cap" value="0"/>

<parm name="deferred-msg-func-uri" value="sip:foo@bar"/>

<parm name="max_adhoc_group_size" value="4"/>

<parm name="conf-fcty-uri" value="sip:foo@bar"/>

<parm name="exploder-uri" value="sip:foo@bar"/>

</characteristic>

<characteristic type="CAPDISCOVERY">

<parm name="pollingPeriod" value="0"/>

<parm name="capInfoExpiry" value="0"/>

<parm name="presenceDisc" value="0"/>

</characteristic>

<characteristic type="APN">

<parm name="rcseOnlyAPN" value="ims"/>

<parm name="enableRcseSwitch" value="1"/>

</characteristic>

<characteristic type="OTHER">

<characteristic type="transportProto">

<parm name="psSignalling" value="SIPoUDP"/>

<parm name="psMedia" value="MSRP"/>

<parm name="psRTMedia" value="RTP"/>

<parm name="wifiSignalling" value="SIPoUDP"/>

<parm name="wifiMedia" value="MSRP"/>

<parm name="wifiRTMedia" value="RTP"/>

</characteristic>

<parm name="endUserConfReqId" value="0"/>

<parm name="deviceID" value="0"/>

</characteristic>

<parm name="AppID" value="ap2002"/>

<parm name="Name" value="RCS-e settings"/>

<parm name="AppRef" value="RCSe-Settings"/>

MSRP message content

MSRP SEND

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request Line |  | MSRP <transactionID> SEND |  |  |
| To-Path |  | msrp://<IP:port>/<ID>;tcp |  |  |
| From-Path |  | msrp://<IP:port>/<ID>;tcp |  |  |
| Message-ID |  | present |  |  |
| Success-Report |  | No |  |  |
| Failure-Report |  | Yes |  |  |
| Byte-Range | B1, B2, B3 | 1-*/<length> |  |  |
| Content-Type | B1, B2, B3 | message/cpim |  |  |
| Message Body | B1 | From: <USERID> To: <sip:anonymous@anonymous.invalid> DateTime: 2016-08-23T07:55:41.208Z NS: imdn <urn:ietf:params:imdn> imdn.Message-ID: <imdn-messageID> imdn.Disposition-Notification: positive-delivery, display Content-type: text/plain Content-Length: <length> <some text> |  |  |
| Message Body | B2 | To: <sip:anonymous@anonymous.invalid> From: <sip:anonymous@anonymous.invalid> DateTime: 2016-08-23T09:55:44Z NS: imdn <urn:ietf:params:imdn> imdn.Message-ID: <new imdn-messageID> Content-type: message/imdn+xml Content-Disposition: notification <?xml version="1.0" encoding="UTF-8"?> <imdn xmlns="urn:ietf:params:xml:ns:imdn"> <message-id><imdn-messageID as in text message></message-id> <datetime>2016-08-23T07:55:41.208Z</datetime> <recipient-uri><USERID></recipient-uri> <delivery-notification> <status> <delivered/> </status> </delivery-notification> </imdn> |  |  |
| Message Body | B3 | To: <sip:anonymous@anonymous.invalid> From: <sip:anonymous@anonymous.invalid> DateTime: 2016-08-23T09:55:45Z NS: imdn <urn:ietf:params:imdn> imdn.Message-ID: ><new imdn-messageID > Content-type: message/imdn+xml Content-Disposition: notification <?xml version="1.0" encoding="UTF-8"?> <imdn xmlns="urn:ietf:params:xml:ns:imdn"> <message-id><imdn-messageID as in text message></message-id> <datetime>2016-08-23T07:55:41.208Z</datetime> <recipient-uri><USERID></recipient-uri> <display-notification> <status> <displayed/> </status> </display-notification> </imdn> |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | MSRP SEND transmitting text |
| B2 | MSRP SEND transmitting delivery notification |
| B3 | MSRP SEND transmitting display notification |

MSRP 200 OK for MSRP SEND message

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Response Line |  | MSRP <transactionID> 200 OK |  |  |
| To-Path |  | msrp://<IP:port>/<ID>;tcp (same as in Request) |  |  |
| From-Path |  | msrp://<IP:port>/<ID>;tcp (same as in Request) |  |  |

Mapping to RCS profiles

The following table specifies the applicability of each conformance test case for previous RCS profiles.

|  | Test Case Title | RCS 1.2.2 | RCS 5.1 | RCS 5.2 | RCS 5.3 |
| --- | --- | --- | --- | --- | --- |
|  | RCS-con-001 First-time unsuccessful configuration – Subscriber unauthorized (Auto-Configuration Server) | Applies | Applies | Applies | Applies |
|  | RCS-con-004 Configuration – RCS re-configuration successful | Applies | Applies | Applies | Applies |
|  | RCS-con-020 Keep-Alive for SIPoUDP (STUN) | Applies | Applies | Applies | Applies |
|  | RCS-con-030 Mobile Originated Capability Discovery – Successful | Applies | Applies | Applies | Applies |
|  | RCS-con-031 Mobile Originated Capability Discovery – Unsuccessful – 480 Temporarily Unavailable | Applies | Applies | Applies | Applies |
|  | RCS-con-032 Mobile Originated Capability Discovery – Unsuccessful –408 Request Timeout | Applies | Applies | Applies | Applies |
|  | RCS-con-033 Mobile Originated Capability Discovery – Unsuccessful – 404 Not Found | Applies | Applies | Applies | Applies |
|  | RCS-con-034 Mobile Originated Capability Discovery – Successful – Multiple Identities | Applies | Applies | Applies | Applies |
|  | RCS-con-035 Mobile Originated Capability Discovery – Successful (IMS registered, does not support RCS) | Applies | Applies | Applies | Applies |
|  | RCS-con-040 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities | Applies | Applies | Applies | Applies |
|  | RCS-con-041 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS Capabilities | Applies | Applies | Applies | Applies |
|  | RCS-con-050 Capability update during a MO call for image sharing | Applies | Applies | Applies | Applies |
|  | RCS-con-051 Capability update during a MT call for image sharing | Applies | Applies | Applies | Applies |
|  | RCS-con-052 Capability update during initiation of an MO voice call – fails as other end has no capability | Applies | Applies | Applies | Applies |
|  | RCS-con-053 Capability Exchange Optimization during a Call | Applies | Applies | Applies | Applies |
|  | RCS-con-100 Mobile Originated 1-to-1 Chat – session establishment – Mobile Originated session termination | Applies | Applies | Applies | Applies |
|  | RCS-con-101 Mobile Originated 1-to-1 Chat – session establishment – Mobile Terminated session termination | Applies | Applies | Applies | Applies |
|  | RCS-con-102 Mobile Originated 1-to-1 Chat – display notification | Applies | Applies | Applies | Applies |
|  | RCS-con-103 Mobile Originated 1-to-1 Chat – duplicate session | Applies | Applies | Applies | Applies |
|  | RCS-con-104 Mobile Originated 1-to-1 Chat – race condition | Applies | Applies | Applies | Applies |
|  | RCS-con-105 Mobile Originated 1-to-1 Chat – inactivity timeout | Applies | Applies | Applies | Applies |
|  | RCS-con-106 Mobile Originated 1-to-1 Chat – store and forward – sender still in active IM session | Applies | Applies | Applies | Applies |
|  | RCS-con-107 Mobile Originated 1-to-1 Chat – store and forward – sender still online | Applies | Applies | Applies | Applies |
|  | RCS-con-108 Mobile Originated 1-to-1 Chat – Switch up to group Chat |  | Applies | Applies | Applies |
|  | RCS-con-109 Mobile Originated 1-to-1 Chat – Maximum Concurrent Sessions |  | Applies | Applies | Applies |
|  | RCS-con-120 Mobile Terminated 1-to-1 Chat – session establishment – Mobile Terminated session | Applies | Applies | Applies | Applies |
|  | RCS-con-121 Mobile Terminated 1-to-1 Chat – session establishment – Mobile Originated session | Applies | Applies | Applies | Applies |
|  | RCS-con-122 Mobile Terminated 1-to-1 Chat – spam/blacklist | Applies | Applies | Applies | Applies |
|  | RCS-con-123 Mobile Terminated 1-to-1 Chat – display notification | Applies | Applies | Applies | Applies |
|  | RCS-con-124 Mobile Terminated 1-to-1 Chat– duplicate session | Applies | Applies | Applies | Applies |
|  | RCS-con-125 Mobile Terminated 1-to-1 Chat – race condition | Applies | Applies | Applies | Applies |
|  | RCS-con-126 Mobile Terminated 1-to-1 Chat – store and forward | Applies | Applies | Applies | Applies |
|  | RCS-con-127 Mobile Terminated 1-to-1 Chat – Switch up to group Chat |  | Applies | Applies | Applies |
|  | RCS-con-130 Mobile Originated Group Chat – establishment – Originating User leaves the session | Applies | Applies | Applies | Applies |
|  | RCS-con-131 Mobile Originated Group Chat – non-originating Users leave the session | Applies | Applies | Applies | Applies |
|  | RCS-con-132 Mobile Originated Group Chat – adding new User to the session | Applies | Applies | Applies | Applies |
|  | RCS-con-135 Mobile Orignated Group Chat – closed Group Chat |  | Applies | Applies | Applies |
|  | RCS-con-140 Mobile Terminated Group Chat – establishment – Terminating User leaves the session | Applies | Applies | Applies | Applies |
|  | RCS-con-145 Group Chat Store and Forward – notification for sent messages |  | Applies | Applies | Applies |
|  | RCS-con-146 Group Chat Store and Forward – delivery of stored messages |  | Applies | Applies | Applies |
|  | RCS-con-147 Group Chat Store and Forward – race condition |  | Applies | Applies | Applies |
|  | RCS-con-165 Group Chat automatic re-join | Applies | Applies | Applies | Applies |
|  | RCS-con-166 Group Chat re-start: 404 (Not Found) | Applies | Applies | Applies | Applies |
|  | RCS-con-167 Group Chat re-start: 403 (Forbidden) | Applies | Applies | Applies | Applies |
|  | RCS-con-168 Group Chat: Concurrent sessions | Applies | Applies | Applies | Applies |
|  | RCS-con-169 Group Chat: Maximum Concurrent Sessions |  | Applies | Applies | Applies |
|  | RCS-con-200 Mobile Originated File Transfer | Applies | Applies | Applies | Applies |
|  | RCS-con-201 Mobile Originated File Transfer – Rejected | Applies | Applies | Applies | Applies |
|  | RCS-con-202 Mobile Originated File Transfer – Store and Forward Sending 1 |  | Applies | Applies | Applies |
|  | RCS-con-205 Mobile Originated File Transfer – File Transfer within a Group Chat |  | Applies | Applies | Applies |
|  | RCS-con-206 Mobile Originated File Transfer – Resume a Send File Transfer |  | Applies | Applies | Applies |
|  | RCS-con-209 Mobile Originated File Transfer – Send Audio Message |  |  | Applies | Applies |
|  | RCS-con-210 Mobile Terminated File Transfer | Applies | Applies | Applies | Applies |
|  | RCS-con-211 Mobile Terminated File Transfer – Rejected | Applies | Applies | Applies | Applies |
|  | RCS-con-212 Mobile Terminated File Transfer – File Size Limit | Applies | Applies | Applies | Applies |
|  | RCS-con-213 Mobile Terminated File Transfer – File Size Warn Limit | Applies | Applies | Applies | Applies |
|  | RCS-con-214 Mobile Terminated File Transfer – Store and Forward Receiving 1 |  | Applies | Applies | Applies |
|  | RCS-con-218 Mobile Terminated File Transfer in a Group Chat – Receive a file |  | Applies | Applies | Applies |
|  | RCS-con-219 Mobile Terminated File Transfer in a Group Chat – Ignore to receive a file |  | Applies | Applies | Applies |
|  | RCS-con-220 Mobile Originated File Transfer in a Group Chat – Blocked file transfer |  | Applies | Applies | Applies |
|  | RCS-con-223 Mobile Terminated File Transfer – Resume receiving a file |  | Applies | Applies | Applies |
|  | RCS-con-226 Mobile Terminated File Transfer – Receive Audio Message |  |  | Applies | Applies |
|  | RCS-con-250 Mobile Originated Image Sharing Session Establishment – Mobile Originated Session Termination | Applies | Applies | Applies | Applies |
|  | RCS-con-251 Mobile Originated Image Sharing – Session Establishment – rejected | Applies | Applies | Applies | Applies |
|  | RCS-con-270 Mobile Terminated Image Sharing – Session Establishment – Mobile Terminated Session | Applies | Applies | Applies | Applies |
|  | RCS-con-271 Mobile Terminated Image Sharing – Session Establishment – rejected | Applies | Applies | Applies | Applies |
|  | RCS-con-400 Mobile Originated Pager Mode – Send Text |  |  | Applies | Applies |
|  | RCS-con-401 Mobile Originated Pager Mode – Send Multimedia |  |  | Applies | Applies |
|  | RCS-con-402 Mobile Originated Pager Mode – Send Long Text |  |  | Applies | Applies |
|  | RCS-con-420 Mobile Terminated Pager Mode – Receive Text |  |  | Applies | Applies |
|  | RCS-con-421 Mobile Terminated Pager Mode – Receive Multimedia |  |  | Applies | Applies |
|  | RCS-con-422 Mobile Terminated Pager Mode – Receive Long Text |  |  | Applies | Applies |
|  | RCS-con-425 Mobile Terminated Pager Mode – No Displayed Notification Sent |  |  | Applies | Applies |
|  | RCS-con-600 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination |  | Applies | Applies | Applies |