Change Request

| Title: | Modify the Default Value of Timer ST2 | Public OMA Confidential |
| --- | --- | --- |
| To: | LOC WG |  |
| Doc to Change: | OMA-AD-SUPL-V1_0-20050628-D |  |
| Submission Date: | <15 Aug 2005> |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Clerical |  |
| Source: | Shanliang, ZTE Corporation, shan.liang@zte.com.cn |  |
| Replaces: | n/a |  |

- Reason for Change

In SUPL AD, We can see that the SLP using timer ST2 to control the delay from sending of SUPL INIT message to receipt of SUPL POS INIT. If there is notification and verification in the SUPL INIT message, the SET user MAY not return the response in a short time. The value of the timer ST2 in this case may be large than that for non-verification case. This CR suggest to distinguish between those two default values in Table-8-2.

- Impact on Backward Compatibility

None as this is a new specification.

- Impact on Other Specifications

None.

- Intellectual Property Rights

None.

- Recommendation

Discuss and agree this CR. Then apply changes to TS as detailed in section 6.

- Detailed Change Proposal

- Table 8-2- SLP timer values

| Timer | Default value (sec.) | Description | Actions on expiration |
| --- | --- | --- | --- |
| ST1 | Proxy: 10 Non-proxy: 50+ (optionally) response time in QoP | For proxy mode: from sending of SUPL RESPONSE to receipt of SUPL POS INIT. For non-proxy mode: from sending of SUPL RESPONSE to receipt of the notification (internal communication between SPC and SLC) that SUPL END has been sent to the SET. | - For proxy: - Send SUPL END to SET - Clear session resources at SLP - For non-proxy: - Internal communication is used to send SUPL END to SET - Clear session resources at SLC/SLP |
| ST2 | Proxy (SUPL INIT without verification): 10 (optionally) Proxy(SUPL INIT with verification): 50 Non-proxy: 50+ (optionally) response time in QoP | For proxy mode: from sending of SUPL INIT to receipt of SUPL POS INIT. For non-proxy mode: from sending SUPL INIT to (a) receipt of notification (internal communitation between SPC and SLC) that SUPL POS INIT has been received or (b) receipt of RLP-SSRP(SUPL END) from V-SLP. | For non-roaming scenario: - Send MLP-SLIA to SUPL agent For roaming scenario: - Send RLP-SRLIA to R-SLP For proxy: Clear session resources at SLP For non-proxy: Clear session resources at SLC |
| ST3 | 10 | From sending of RLP-SSRLIR(SUPL START) to receipt of RLP-SSRLIA(SUPL RESPONSE) | For network initiated scenario: - Send RLP-SRLIA to R-SLP - Clear session resources at SLP For SET initiated scenario: - Send SUPL END to SET - Clear session resources at SLP |
| ST4 | 10 | From sending of RLP-SSRLIR(msid, lid) to receipt of RLP-SSRLIA(msid, posresult) | For network initiated scenario: - Send SUPL END to SET - Send RLP-SRLIA to R-SLP - Clear session resources at SLP For SET initiated scenario: - Send SUPL END to SET - Clear session resources at SLP |